END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 14AE2030 / 18AE2045 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # BASICS OF AEROSPACE ENGINEERING | # Max. Marks | # 100 |

| Q. No. | Questions | CO | BL | M |
| --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |
| 1. | ______________ broke the thought of flapping wing and separated the concept of lift generation and propulsion. | CO1 | R | 1 |
| 2. | The concept of Aerial steam engine was proposed by __________________. | CO1 | R | 1 |
| 3. | The force that causes the aircraft to move forward against the drag forces is known as _______________. | CO2 | U | 1 |
| 4. | Name the high lift devices used in aircrafts. | CO2 | U | 1 |
| 5. | Major aircraft component which holds the passengers along with cargo is called as __________. | CO3 | R | 1 |
| 6. | Flaps are located at the leading edge of the airplane wing. True / False | CO3 | U | 1 |
| 7. | High pressure air can escape around the tip and flow up around to the top surface leading to _____________. | CO4 | R | 1 |
| 8. | Name 4 common non-metals used in aircraft. | CO4 | R | 1 |
| 9. | Gas turbine engines work on ___________ cycle. | CO5 | R | 1 |
| 10. | Blade angle is the angle between the __________ of the blade and the plane of rotation. | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |
| 11. | Paraphrase the working principle of Ornithopters. | CO1 | U | 3 |
| 12. | Explain the advantages of monoplanes over biplanes. | CO2 | R | 3 |
| 13. | Explain the 5 major stresses acting on an aircraft. | CO3 | R | 3 |
| 14. | Write the use of Magnesium in aircraft structures. | CO4 | R | 3 |
| 15. | Explain the role of matrix and reinforcements in a composite. | CO5 | R | 3 |
| 16. | Why are high earth orbits also known as geosynchronous orbits? | CO6 | R | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |
| 17. | # Enumerate the salient contributions of Otto Lilienthal to the early developments of aircraft. | CO1 | R | 12 |
| 18. | Explain the working principle of basic T instruments in aircraft. | CO2 | U | 12 |
| 19. | Explain the 3 predominant structures of fuselage types and its construction with a schematic representation. | CO3 | R | 12 |
| 20. | Explain the different metallic materials used for the construction of aircraft structures. | CO4 | R | 12 |
| 21. | Explain the working of a turboprop engine with a neat sketch. | CO5 | R | 12 |
| 22. | Explain the principle and working of a pressure-fed liquid propellant rocket engine. | CO5 | R | 12 |
| 23. | Write a brief note on the characteristics and uses of low, medium and high earth orbits. | CO6 | U | 12 |
| COMPULSORY QUESTION |  |  |  |  |
| 24. | Explain the 5 major parts and functions of a typical aircraft with a neat sketch. | CO2 | R | 12 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL                  M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the evolution of aircrafts and flying vehicles. |
| CO2 | Understand the parts and function of aircrafts. |
| CO3 | Obtain knowledge on principles of flight. |
| CO4 | Understand the fundamentals of structures and materials used. |
| CO5 | Understand the principles of aircraft and rocket propulsion. |
| CO6 | Obtain knowledge on the engines used in aircraft propulsion. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | R | U | A | An | E | C | Total |
| CO1 | 14 | 3 | - | - | - | - | 17 |
| CO2 | 15 | 14 | - | - | - | - | 29 |
| CO3 | 16 | 1 | - | - | - | - | 17 |
| CO4 | 17 | 0 | - | - | - | - | 17 |
| CO5 | 28 | 0 | - | - | - | - | 28 |
| CO6 | 3 | 13 | - | - | - | - | 16 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 17AE2030 / 18AE2030 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # WIND TUNNEL TECHNIQUES | # Max. Marks | # 100 |

| Q. No | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | Classify the types of Wind Tunnels. |  | CO1 | U | 1 |
| 2. | Define “Mach Number”. |  | CO3 | R | 1 |
| 3. | Indicate the use of convergent and divergent section in the wind tunnel. |  | CO2 | U | 1 |
| 4. | Locate the use of honeycomb section in the wind tunnel. |  | CO1 | R | 1 |
| 5. | State the various measurements systems to be installed in a wind tunnels. |  | CO5 | R | 1 |
| 6. | Mention about the history & innovation of the wind tunnel. |  | CO1 | U | 1 |
| 7. | Differentiate between the terms “Yawing” and “Rolling”. |  | CO1 | U | 1 |
| 8. | Mention the application of a “wind tunnel balance”. |  | CO4 | U | 1 |
| 9. | Show the use of “hot wire anemometer” system used in wind tunnel. |  | CO6 | U | 1 |
| 10. | Show the use of heat flux measurements in wind tunnel. |  | CO5 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | Write the reasons for energy losses in wind tunnel. |  | CO5 | A | 3 |
| 12. | List out the parameters which can be measured by use of wind tunnel. |  | CO5 | R | 3 |
| 13. | Explain the use of “Shock Tube” in wind tunnel measurement and write its applications. |  | CO5 | A | 3 |
| 14. | Show an “Aircraft Aerofoil” and indicates the various forces and moments acting on it. |  | CO2 | U | 3 |
| 15. | Differentiate between the “internal balance” and “outer balance “in wind tunnel measurement system. |  | CO4 | U | 3 |
| 16. | Explain the application of “Interferometer” concept in the wind tunnel system. |  | CO6 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Illustrate the constructional detail and operation of smoke tunnel. | CO6 | A | 6 |
|  | b. | Illustrate the operation of a water tunnel in detail with help of a schematic diagram. | CO2 | A | 6 |
| 18. | a. | Write the various parts of a Supersonic wind tunnel with help of a diagram and explain its working principle. | CO2 | A | 6 |
|  | b. | Distinguish between the “Hypersonic” and “Supersonic” condition in the wind tunnel. | CO2 | An | 6 |
| 19. | a. | Appraise about the “starting and stopping Loads” in supersonic wind tunnel in detail. | CO2 | An | 6 |
|  | b. | Analyze the “Runtime mass flow rate” in the supersonic wind tunnel. | CO4 | An | 6 |
| 20. |  | With help of a schematic diagram illustrate the operation of a laser Doppler Anemometry | CO5 | An | 12 |
| 21. | a. | Sketch the Hypersonic wind tunnel, explain its nomenclature and working principle. | CO2 | A | 6 |
|  | b. | Establish the various calibration tests to be conducted on wind tunnels with suitable sketches. | CO2 | A | 6 |
| 22. | a. | Explain the various parts of a “Strain gauge balance” and indicate how the lift, drag and other aerodynamic moments are measured. | CO4 | A | 6 |
|  | b. | Relate the Strut type balance with other types of balances with suitable sketches and description. | CO3 | A | 6 |
| 23. |  | Explain how the “Tufts” can be fixed to the model and testing is being conducted. | CO6 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Draw a layout diagram of a Shadowgraph technique and explain its construction and working principle in detail. | CO6 | A | 6 |
|  | b. | Draw a layout diagram of “Schlieren imaging system” and explain its construction and working principle in detail. | CO6 | A | 6 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the various types of wind tunnels and test techniques. |
| CO2 | Choose proper high speed wind tunnel for required test |
| CO3 | Choose correct model for wind tunnel testing. |
| CO4 | Estimate the forces and moments for given model. |
| CO5 | Estimate pressure, velocity and temperature using measurement techniques. |
| CO6 | Choose the proper flow visualization techniques |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | R | U | A | An | E | C | Total |
| CO1 | 1 | 3 | - | - | - | - | 4 |
| CO2 | - | 4 | 24 | 12 | - | - | 40 |
| CO3 | 1 | - | 6 | - | - | - | 7 |
| CO4 | - | 4 | 6 | 6 | - | - | 16 |
| CO5 | 4 | 1 | 6 | 12 | - | - | 23 |
| CO6 | - | 1 | 33 | - | - | - | 34 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 17AE3022 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # ELEMENTS OF AEROSPACE ENGINEERING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |  |
| 1. | a. | Explain how interference and pressure drag can be minimized. | CO1 | R | 8 |
|  | b. | Show how pressure and density varies with altitude in graph. | CO1 | U | 8 |
| 2. | a. | Explain with neat sketch the components of airplane and their functions. | CO2 | U | 8 |
|  | b. | Define Mach number. Explain the different regimes using Mach number with near diagrams. | CO2 | U | 8 |
| 3. | a. | Recite NACA. Classify merits, demerits and application NACA airfoils. | CO3 | U | 8 |
|  | b. | Recall the Aerodynamic forces acting on aircraft. | CO3 | R | 8 |
| 4. | a. | List the types, characteristics, uses of titanium in aircraft. | CO4 | R | 8 |
|  | b. | Explain monocoque and semi-monocoque construction of fuselage. | CO4 | U | 8 |
| 5. | a. | Construct the working of Fly-By-Wire system. | CO5 | A | 8 |
|  | b. | Classify the basic instrumentation in aircraft and explain altimeter. | CO5 | U | 8 |
| 6. | a. | Explain the types of wing structure in aircraft with necessary diagrams. | CO4 | U | 8 |
|  | b. | With neat illustrations, explain in detail the main group of materials used in aircraft construction. | CO4 | U | 8 |
| 7. | a. | State the advantage and disadvantage of solid propellant rocket. | CO6 | R | 8 |
|  | b. | Illustrate the application of rocket engine. | CO6 | U | 8 |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |  |
| 8. | a. | Explain in detail how the space exploration benefit science, human understanding. | CO6 | U | 10 |
|  | b. | With neat illustrations, explain in detail the working of a turboprop engine. | CO6 | U | 10 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL        M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand standard atmosphere and properties. |
| CO2 | Understand Principles of flight. |
| CO3 | Get knowledge in aerodynamic shapes. |
| CO4 | Understand Aerospace materials and aircraft structural component. |
| CO5 | Classify the Aircraft instrumentation systems. |
| CO6 | Categorize the Power plants used in various aircraft. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 8 | 8 |  |  |  |  | 16 |
| CO2 |  | 16 |  |  |  |  | 16 |
| CO3 | 8 | 8 |  |  |  |  | 16 |
| CO4 | 8 | 24 |  |  |  |  | 32 |
| CO5 |  | 8 | 8 |  |  |  | 16 |
| CO6 | 8 | 28 |  |  |  |  | 36 |
|  |  |  |  |  |  |  | 132 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 18AE2014 / 14AE2021 / 17AE2016 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # GAS DYNAMICS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Write down the expression of Crocco number. |  | CO1 | A | 1 |
| 2. | Identify the relationship between pressure and Mach number for M>1. |  | CO1 | R | 1 |
| 3. | Define overexpanding nozzle. |  | CO2 | R | 1 |
| 4. | Compare the nozzle and diffuser. |  | CO2 | U | 1 |
| 5. | Identify the condition of choked flow in a C-D nozzle. |  | CO3 | R | 1 |
| 6. | State the significance of shock polar. |  | CO3 | R | 1 |
| 7. | Define drag divergence Mach number. |  | CO4 | R | 1 |
| 8. | Identify the limitation of small perturbation theory. |  | CO4 | R | 1 |
| 9. | Define lower critical Mach number. |  | CO5 | R | 1 |
| 10. | Name the wind tunnel that has a knife edge. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Air at stagnation condition has a temperature of 800 K. Calculate the stagnation velocity of sound and the maximum possible fluid velocity. |  | CO1 | A | 3 |
| 12. | Write the expression of Rayleigh pitot tube formula. |  | CO2 | A | 3 |
| 13. | Define Mach cone. |  | CO3 | R | 3 |
| 14. | Explain the similarity rule. |  | CO4 | U | 3 |
| 15. | Sketch Fanno curve. |  | CO5 | A | 3 |
| 16. | Distinguish between open and closed-circuit wind tunnels. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Produce the expression of speed of sound for a calorically perfect gas with suitable assumptions. | CO1 | A | 12 |
| 18. |  | The pressure, temperature and Mach number at the entry of a flow passage are 2.45 bar, 26.5ºC and 1.4 respectively. If the exit Mach number is 2.5, determine for adiabatic flow of a perfect gas (γ=1.3 and R=0.469 kJ/kg K): - Stagnation temperature - Temperature and velocity of gas at exit - Flow rate per square meter of the inlet cross-section | CO2 | A | 4 4 4 |
| 19. | a. | A normal shock wave forms in an air stream at a static temperature of 22K. If the total temperature is 400 K, determine the Mach number and static temperature behind the shock. | CO3 | A | 6 |
|  | b. | Explain the regular reflection of shock from a solid boundary. | CO3 | U | 6 |
| 20. |  | Produce the linearized perturbation velocity potential equation. | CO4 | A | 12 |
| 21. |  | Produce an equation representing the Rayleigh line and sketch Rayleigh lines on h-s and p-v planes. | CO5 | A | 12 |
| 22. |  | Illustrate the changes across a normal shock in terms of thermodynamic variables. | CO3 | U | 12 |
| 23. | a. | Discuss the advantages of a swept back wing. | CO4 | U | 6 |
|  | b. | Explain the tip effects on aircraft performance. | CO4 | U | 6 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Explain in detail the working principle of shadowgraph with a neat diagram. | CO6 | U | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the influence of compressibility to distinguish between the flow regime |
| CO2 | Apply compressibility corrections for flow in C-D passages and instrument like Pitot static tube |
| CO3 | Estimate the sudden changes in the flow field |
| CO4 | Analyse the compressible flow field over an airfoil and finite wings |
| CO5 | Estimate the influence of friction and heat transfer in the flow field |
| CO6 | Choose proper flow visualisation techniques for the given situation |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | R | U | A | An | E | C | Total |
| CO1 | 1 | - | 16 | - | - | - | 17 |
| CO2 | 1 | 1 | 15 | - | - | - | 17 |
| CO3 | 5 | 18 | 6 | - | - | - | 29 |
| CO4 | 2 | 15 | 12 | - | - | - | 29 |
| CO5 | 1 | - | 15 | - | - | - | 16 |
| CO6 | 1 | 15 | - | - | - | - | 16 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 18AE2018 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # PROPULSION-II | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Determine the thrust produced, if the mass flow rate is 32 kg/s and specific impulse is 288 s. |  | CO1 | A | 1 |
| 2. | Define velocity correction factor. |  | CO1 | R | 1 |
| 3. | State the purpose of multi-staging in rocket. |  | CO2 | R | 1 |
| 4. | State the methods of thrust reversal used in aircraft. |  | CO2 | R | 1 |
| 5. | State the significance of binder in solid rocket motor |  | CO3 | R | 1 |
| 6. | List the reason for change in ratio of fuel to oxidizer in hybrid rocket motor having a constant oxidizer flow-rate. |  | CO4 | R | 1 |
| 7. | List the various types of cooling systems used in liquid rocket engine. |  | CO4 | R | 1 |
| 8. | Illustrate the various application of integral ram rocket. |  | CO5 | U | 1 |
| 9. | List the disadvantages of electro thermal systems. |  | CO5 | R | 1 |
| 10. | Discuss the purpose of electrostatic accelerator in ion propulsion system. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | The following data are given for a certain rocket unit, thrust 7000 N, Propellant consumption 3.5 kg/s determine the specific impulse in s. |  | CO1 | A | 3 |
| 12. | Describe jetavators with neat sketch. |  | CO2 | U | 3 |
| 13. | Illustrate the performance of the following oxidizer with respect to Specific impulse for variation in concentration AP, AN and KP. |  | CO3 | A | 3 |
| 14. | Discuss the requirement of good atomizer. |  | CO4 | U | 3 |
| 15. | State the various advantages of liquid rocket engine. |  | CO5 | R | 3 |
| 16. | Explain frozen flow losses in pulse plasma thrusters. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Discuss the challenges and limitations of using subsonic intakes for achieving high-speed with low mass flow conditions. | CO2 | U | 6 |
|  | b. | Describe the key considerations for designing an air inlet that efficiently captures and compresses air at supersonic flight speeds. | CO2 | U | 6 |
| 18. | a. | Define specific impulse and explain its significance in the context of rocket engine performance. | CO1 | A | 4 |
|  | b. | Explain how ambient temperature, pressure, and atmospheric composition can affect the performance of chemical rocket propellants. | CO1 | U | 8 |
| 19. | a. | Differentiate any two chemical propellant combinations, compare and contrast their specific impulse, thrust-to-weight ratio, and suitability for a specific mission profile. | CO3 | U | 6 |
|  | b. | Analyze the internal ballistics of a solid rocket motor, including the relationship between burning rate, pressure, and thrust, and explain how these factors influence motor performance. | CO3 | An | 6 |
| 20. | a. | Differentiate between a gas generator and a pre-burner, highlighting their specific functions in a rocket engine. | CO5 | U | 8 |
|  | b. | Draw the schematic diagram of single turbine pump assemble of a pre-burner. | CO5 | R | 4 |
| 21. | a. | List and elaborate on six key criteria considered when selecting liquid propellants for a rocket engine. | CO4 | R | 6 |
|  | b. | Draw the schematic diagram of pressure feed system for liquid rocket and explain the significance of each system. | CO4 | R | 6 |
| 22. |  | Describe the fundamental principle behind the operation of a semi-cryogenic rocket engine. | CO5 | U | 12 |
| 23. |  | Explain the need and potential benefits of employing electrical propulsion systems for spacecraft. | CO6 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Compare and contrast different types of electric propulsion systems and discuss their suitability for various spacecraft missions based on factors like thrust level and specific impulse. | CO6 | U | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand and evaluate the performance of chemical propellant. |
| CO2 | Select and design a suitable air inlets and nozzles. |
| CO3 | Select and design a suitable solid rocket motor. |
| CO4 | Select and design a suitable liquid rocket engine. |
| CO5 | Understand the working of sub-systems of the propulsion system. |
| CO6 | Assess the performance of electric propulsion systems. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | R | U | A | An | E | C | Total |
| CO1 | 1 | 8 | 8 |  |  |  | 17 |
| CO2 | 2 | 15 |  |  |  |  | 17 |
| CO3 | 1 | 6 | 3 | 6 |  |  | 16 |
| CO4 | 14 | 3 |  |  |  |  | 17 |
| CO5 | 8 | 21 |  |  |  |  | 29 |
| CO6 |  | 16 | 12 |  |  |  | 28 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 18AE2019 / 17AE2020 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # AIRCRAFT STABILITY AND CONTROL | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | Write the basic equations of motion. |  | CO1 | U | 1 |
| 2. | Discuss the degree of freedom. |  | CO1 | U | 1 |
| 3. | Define-static margin. |  | CO2 | R | 1 |
| 4. | Describe a neutral point in aircraft stability. |  | CO2 | R | 1 |
| 5. | Write the conditions for static directional stability. |  | CO3 | R | 1 |
| 6. | Discuss dorsal fins in aircraft. |  | CO3 | U | 1 |
| 7. | Define-anhedral angle. |  | CO4 | R | 1 |
| 8. | Describe the aileron effectiveness. |  | CO4 | R | 1 |
| 9. | Define neutral stability. |  | CO5 | R | 1 |
| 10. | Describe the spin phenomenon of an aircraft. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | Describe the elevator requirements for static longitudinal stability. |  | CO1 | R | 3 |
| 12. | State the hinge moment with necessary equations. |  | CO2 | R | 3 |
| 13. | Describe the dorsal fin and its control. |  | CO3 | R | 3 |
| 14. | Explain the aerodynamic balancing of an aircraft. |  | CO4 | U | 3 |
| 15. | Discuss the Phugoid and short period motion of an aircraft. |  | CO5 | U | 3 |
| 16. | Explain the spiral instability of an aircraft. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No. 17 to 23, Q.No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Infer the equation of fuselage contribution in static longitudinal stability. | CO1 | U | 12 |
| 18. | a. | Assume that the pitching moment curve for the landing configuration at its forwardmost center of gravity position as follows: Cm,cg=-0.20-0.035α, where α is in degrees. Calculate the size of the elevator to trim the aircraft at the landing angle of attach 10°. Assume that the elevator angle is constrained to +20° and -25°. Tail area is given 43 feet2. (VH=0.66, η=1.0, CL,αt=3.91rad-1 | CO2 | A | 8 |
|  | b. | Infer the equation of elevator hinge moment. | CO2 | U | 4 |
| 19. | a. | Discuss the adverse yaw and crosswind landing with necessary equations & diagrams. | CO3 | U | 12 |
| 20. | a. | Infer the equation for aileron control force requirements for lateral stability. | CO4 | U | 12 |
| 21. | a. | Discuss the application of small perturbation theory in dynamic stability equation of motion. | CO5 | U | 12 |
| 22. | a. | Infer the equation of motion for lateral dynamic stability. | CO6 | U | 12 |
| 23. | a. | Infer the equation of tail contribution in static longitudinal stability. | CO1 | U | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | The wind tunnel tests on an airplane model indicate that full aileron deflection to right introduces an adverse yaw causing Cn = -0.008. How many degrees of rudder must be applied to keep the sideslip zero during the roll? Given that Sw = 16.4 m2 ,Sv = 2.1 m2 , lv = 5.5 m, b = 9.8 m, ηv = 0.95 , CLαv = 0.045 deg-1, τrudder = 0.5. | CO3 | A | 9 |
|  | b. | Describe the stick free directional stability. | CO3 | R | 3 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the degree of freedom of aircraft system. |
| CO2 | Analyse the static stability behaviour of the aircraft. |
| CO3 | Understand the dynamic longitudinal stability of aircraft. |
| CO4 | Perform the dynamic analysis to determine stability of aircraft. |
| CO5 | Estimate the requirement of control force and power plant. |
| CO6 | Assess the motion of unstable aircraft and related modes of instability. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | R | U | A | An | E | C | Total |
| CO1 | 3 | 26 | - | - | - | - | 29 |
| CO2 | 5 | 4 | 8 | - | - | - | 17 |
| CO3 | 7 | 13 | 9 | - | - | - | 29 |
| CO4 | 2 | 15 | - | - | - | - | 17 |
| CO5 | 1 | 15 | - | - | - | - | 16 |
| CO6 | 1 | 15 | - | - | - | - | 16 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 18AE2025 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # NAVIGATION, GUIDANCE AND CONTROL OF AEROSPACE VEHICLES | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | List out the primary function of radio detection and ranging (RADAR) system. |  | CO1 | R | 1 |
| 2. | State the unit in which signal to noise power (SNR) is expressed. |  | CO1 | R | 1 |
| 3. | Give an example of a navigation system. |  | CO2 | U | 1 |
| 4. | Identify the navigation that is based on the phase difference between radio signals from two or more synchronized transmitter. |  | CO2 | U | 1 |
| 5. | Indicate the navigation system that comprises of an inertial measurement unit and computational unit. |  | CO3 | U | 1 |
| 6. | Identify the commonly used sensor in an inertial navigation system. |  | CO3 | U | 1 |
| 7. | State the function of an autopilot system. |  | CO4 | R | 1 |
| 8. | Indicate the importance of guidance laws. |  | CO4 | U | 1 |
| 9. | Write any one application of a signal flow graph. |  | CO5 | A | 1 |
| 10. | Choose the plot employed for determining the stability of a system. |  | CO6 | A | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Compare the two types of polarization. |  | CO1 | U | 3 |
| 12. | Differentiate between instrument landing system (ILS) and microwave landing system (MLS). |  | CO2 | U | 3 |
| 13. | List out the reference frames used in navigation system. |  | CO3 | R | 3 |
| 14. | State the importance of missile guidance laws. |  | CO4 | R | 3 |
| 15. | Indicate the properties of a transfer function. |  | CO5 | U | 3 |
| 16. | Enumerate the applications of Routh’s stability criteria. |  | CO6 | R | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Describe the frequency and operational range of radio detection and ranging (RADAR) system | CO1 | R | 6 |
|  | b. | Examine the effects of noise in radio detection and ranging (RADAR) system. | CO1 | R | 6 |
| 18. | a. | Enumerate the salient features of a global positioning system (GPS). | CO2 | R | 6 |
|  | b. | Describe the construction of instrument flight rules (IFR) navigation system. | CO2 | R | 6 |
| 19. | a. | Discuss the transformation methods employed in inertial navigation. | CO3 | U | 12 |
| 20. | a. | Explain the mechanization of inertial navigation system. | CO3 | U | 6 |
|  | b. | Illustrate the salient features of an integrated navigation system. | CO3 | U | 6 |
| 21. | a. | Articulate on the classical and modern guidance laws. | CO4 | A | 12 |
| 22. | a. | Compute the transfer function for the following block diagram. | CO5 | A | 12 |
| 23. | a. | Write the steps to compute the transfer function from a signal flow graph using Mason’s rule. | CO5 | A | 6 |
|  | b. | Articulate on the applications of open and closed loop transfer functions in a control system. | CO5 | A | 6 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Summarize the applications of Bode and Nyquist plot. | CO6 | U | 6 |
|  | b. | Explain the concept of Routh’s stability criterion. | CO6 | A | 6 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Recall the radar concepts and their operation. |
| CO2 | Identify fundamental navigation concepts and their working. |
| CO3 | Exemplify various inertial sensors and their applications in IMU. |
| CO4 | Compute guidance commands with the knowledge of the guidance laws. |
| CO5 | Illustrate control system concepts. |
| CO6 | Integrate and validate control systems in aerospace applications. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | R | U | A | An | E | C | Total |
| CO1 | 14 | 3 | - | - | - | - | 17 |
| CO2 | 12 | 5 | - | - | - | - | 17 |
| CO3 | 3 | 26 | - | - | - | - | 29 |
| CO4 | 4 | 1 | 12 | - | - | - | 17 |
| CO5 | - | 3 | 25 | - | - | - | 28 |
| CO6 | 3 | 6 | 7 | - | - | - | 16 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 18AE2027 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # HEAT AND MASS TRANSFER | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | State Fourier’s law of heat conduction. |  | CO1 | R | 1 |
| 2. | Identify the unit of thermal diffusivity. |  | CO1 | R | 1 |
| 3. | Name the modes of condensation. |  | CO2 | R | 1 |
| 4. | State Fick’s law of diffusion. |  | CO2 | R | 1 |
| 5. | Define Biot number. |  | CO3 | R | 1 |
| 6. | Give examples of transient heat conduction. |  | CO3 | U | 1 |
| 7. | Define Grashof number. |  | CO4 | R | 1 |
| 8. | Define the momentum thickness. |  | CO4 | R | 1 |
| 9. | Define the effectiveness of the heat exchanger. |  | CO5 | R | 1 |
| 10. | Define ablative heat transfer. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Explain the critical thickness of insulation. |  | CO1 | U | 3 |
| 12. | Define film boiling. |  | CO2 | R | 3 |
| 13. | Explain lumped heat analysis. |  | CO3 | U | 3 |
| 14. | Sketch the boundary layer inside a pipe. |  | CO4 | A | 3 |
| 15. | Classify the heat exchanger. |  | CO5 | U | 3 |
| 16. | Explain aerodynamic heating. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Hot air at 40ºC is flowing through a steel pipe of 10 cm diameter. The pipe is covered with two layers of insulating materials of thickness 4 cm and 3 cm and their corresponding thermal conductivities are 0.1 and 0.32 W/mK. The inside and outside convective heat transfer coefficients are 50 W/m²K and 15 W/m²K respectively. The outer temperature is 10ºC. Determine the heat loss per meter length of the steam pipe. | CO1 | A | 12 |
| 18. |  | A nickel wire carrying an electric current of 1.5 mm diameter and 50 cm long, is submerged in a water bath that is open to atmospheric pressure. Calculate the voltage at the burn out point, if at this point the wire carries a current of 200 A. | CO2 | A | 12 |
| 19. | a. | Explain the types of fins and indicate their applications. | CO3 | U | 6 |
|  | b. | An aluminum sphere weighing 7 kg and initially at a temperature of 320ºC is suddenly immersed in a liquid at 25ºC. The convective heat transfer coefficient is 50 W/m²K. Determine the time required for the sphere to reach 100ºC. | CO3 | U | 6 |
| 20. |  | Air at 0ºC flows over a flat plate at a speed of 90 m/s and is heated to 100ºC. The plate is 60 cm long and 75 cm wide. Assuming the transition of boundary layer take place at Re= 5x105. Calculate the following: - Average friction coefficient - Average heat transfer coefficient - Rate of energy dissipation | CO4 | A | 4 4 4 |
| 21. |  | A parallel flow heat exchanger is used to cool 4.2 kg/min of hot liquid of specific heat 3.5 kJ/kg K at 130˚C. A cooling water of specific heat 4.18 kJ/kg K is used for cooling purposes at a temperature of 15 ˚C. The mass flow rate of cooling water is 17 kg/min. Calculate the following. - Outlet temperature of liquid - Outlet temperature of water - Effectiveness of heat exchanger Take overall heat transfer coefficient is 1100 W/m²K. The heat exchanger area is 0.3 m². | CO5 | A | 4 4 4 |
| 22. |  | The molecular weights of the two components A and B of a gas mixture are 24 and 48 respectively. The molecular weight of a gas mixture is found to be 30. If the mass concentration of the mixture is 1.2 kg/m³, determine the following: - Density of components A and B - Molar fractions - Mass fractions | CO2 | A | 4 4 4 |
| 23. |  | Explain in detail the heat transfer in rocket thrust chambers with a neat sketch. | CO6 | U | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | A horizontal plate of 800 mm long and 70 mm wide is maintained at a temperature of 140ºC in a large tank that is full of water at 60ºC. Determine the total heat loss from the plate. | CO4 | A | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the fundamental modes of heat transfer. |
| CO2 | Understand the phase change heat transfer. |
| CO3 | Use the heat transfer correlation for different heat transfer applications. |
| CO4 | Understand the concept of hydrodynamic and thermal boundary layers. |
| CO5 | Analyse and design the different types of heat exchangers. |
| CO6 | Apply heat transfer principles of different applications. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | R | U | A | An | E | C | Total |
| CO1 | 2 | 3 | 12 | - | - | - | 17 |
| CO2 | 5 | - | 24 | - | - | - | 29 |
| CO3 | 1 | 16 | - | - | - | - | 17 |
| CO4 | 2 | - | 27 | - | - | - | 29 |
| CO5 | 1 | 3 | 12 | - | - | - | 16 |
| CO6 | 1 | 15 | - | - | - | - | 16 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 18AE2032 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # FINITE ELEMENT ANALYSIS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Define discrete element. |  | CO1 | R | 1 |
| 2. | Define boundary value problem. |  | CO1 | R | 1 |
| 3. | Define shape function. |  | CO2 | R | 1 |
| 4. | List the stages of FEA. |  | CO2 | R | 1 |
| 5. | Write the equation of the stiffness matrix for a 2D truss element. |  | CO3 | A | 1 |
| 6. | Define degrees of freedom. |  | CO3 | R | 1 |
| 7. | Name any two FEA software. |  | CO4 | R | 1 |
| 8. | List the methods to analyse fluid flow. |  | CO4 | R | 1 |
| 9. | Explain the Linear Strain Triangular (LST) element. |  | CO5 | U | 1 |
| 10. | List the stresses of a typical axisymmetric element. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | State the principle of the Rayleigh-Ritz method. |  | CO1 | R | 3 |
| 12. | Compare and contrast local and natural coordinate systems. |  | CO2 | U | 3 |
| 13. | Identify the properties of the global stiffness matrix. |  | CO3 | R | 3 |
| 14. | Explain plane strain condition. |  | CO4 | U | 3 |
| 15. | State the significance of the subparametric element. |  | CO5 | R | 3 |
| 16. | Write the expression of the strain-displacement matrix for an axisymmetric element. |  | CO6 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | The differential equation of a physical phenomenon is given by d2yd2x+500x2=0, 0≤x≤1 , Trial function, y= a1(x-x4) and boundary conditions are y(0) = 0 and y(1)=0. Calculate the value of the parameter a1 by the following methods: - Point collocation method - Subdomain collocation method - Least squares method - Galerkin method | CO1 | A | 3 3 3 3 |
| 18. |  | A thin steel plate of uniform thickness 25 mm is subjected to a point load of 420 N at mid-depth as shown in the figure below. The plate is also subjected to self-weight. If young`s modulus E= 2 x 105 N/mm2 and unit weight density, ρ = 0.8 x 10-4 N/mm3, calculate the following: - Displacement at each nodal point - Stresses in each element | CO2 | A | 6 6 |
| 19. |  | For the two bar truss shown in Fig. below, determine the stiffness matrix of elements 1 and 2. Take Young’s modulus E= 70 GPa and Area A= 200 mm². | CO3 | A | 12 |
| 20. |  | A pump pumping fluid at Q=6500 m³/hr is located at coordinates (5,2) in the element as shown in Fig. below. Determine the amount of Q allotted to each node. All nodal coordinates are in m. Assume unit thickness of t= 1mm. | CO4 | A | 12 |
| 21. |  | Determine the stiffness matrix for the constant strain triangular (CST) element shown in Fig. below. The coordinates are given units of millimeters. Assume plane stress conditions. Take E= 210 GPa, ʋ= 0.25 and t= 10 mm. | CO5 | A | 12 |
| 22. |  | A beam, fixed at one end and supported by a roller at the other end, has a 20kN concentrated load applied at the center of the span, as shown in Fig. below. Calculate the deflection under the load and construct the shear force and bending moment diagrams for the beam. Take E= 20X106 N/cm² and I = 2500 cm4 | CO2 | A | 12 |
| 23. |  | A wall of 0.6 m thickness has a thermal conductivity of 1.2 W/mK. The wall is to be insulated with a material of thickness 0.06 m having an average thermal conductivity of 0.3 W/mK. The inner surface is 1000ºC and the outside of the insulation is exposed to atmospheric air at 30ºC with a heat transfer coefficient of 35 W/m²K. Calculate the nodal temperatures. | CO4 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | The nodal coordinates for an axisymmetric triangular element are given below: r1= 20 mm; z1= 40 mm, r2= 40 mm, z2= 40 mm, r3= 30 mm and z3=60 mm Determine the strain-displacement matrix [B] for that element. | CO6 | A | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Analyze the discrete and continuum problem using finite element method. |
| CO2 | Understand the different Numerical solution to the FEA Problems. |
| CO3 | Identify mathematical model for solution of common engineering problems. |
| CO4 | Describe the usage of professional-level finite element software to solve engineering problems in Solid mechanics, fluid mechanics and heat transfer. |
| CO5 | Analyze the functions of different elements and Stiffness Matrix. |
| CO6 | Perform the Axisymmetric problems. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | R | U | A | An | E | C | Total |
| CO1 | 5 | - | 12 | - | - | - | 17 |
| CO2 | 2 | 3 | 24 | - | - | - | 29 |
| CO3 | 4 | - | 13 | - | - | - | 17 |
| CO4 | 2 | 3 | 24 | - | - | - | 29 |
| CO5 | 3 | 1 | 12 | - | - | - | 16 |
| CO6 | 1 | - | 15 | - | - | - | 16 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 18AE2036 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # INTRODUCTION TO NON DESTRUCTIVE TESTING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | List the various scopes of non-destructive testing. |  | CO1 | R | 1 |
| 2. | Name any two NDT techniques used to detect internal defects. |  | CO1 | R | 1 |
| 3. | Distinguish between Liquid penetrant testing & Visual inspection. |  | CO2 | U | 1 |
| 4. | List out any two properties of a good developer. |  | CO2 | A | 1 |
| 5. | Write the merits of Eddy current testing. |  | CO3 | A | 1 |
| 6. | Define the principle behind Magnetic particle testing. |  | CO3 | U | 1 |
| 7. | Indicate the limitations of Radiography Test. |  | CO4 | A | 1 |
| 8. | State the normal beam location technique used in Acoustic emission testing. |  | CO5 | R | 1 |
| 9. | Differentiate between Acoustic Emission & Ultrasonic testing. |  | CO5 | U | 1 |
| 10. | Define the basic principle of Thermography. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Outline the history of Non Destructive Testing. |  | CO1 | A | 3 |
| 12. | Discuss the reason why liquid penetrant testing cannot be used to detect internal defects. |  | CO2 | U | 3 |
| 13. | List out the essential properties required to increase sensitivity of the MPT test. |  | CO3 | A | 3 |
| 14. | Evaluate the influencing factors affecting radiographic testing. |  | CO4 | E | 3 |
| 15. | Differentiate between Ultrasonic testing and other NDT methods. |  | CO5 | U | 3 |
| 16. | Describe the applications of thermography test in electronics industry. |  | CO6 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Describe the differences between the NDT and destructive test. | CO1 | U | 6 |
|  | b. | Discuss the various features of NDT techniques. | CO1 | U | 6 |
| 18. | a. | Explain the physical principle of Liquid penetrant test procedure with neat sketches. | CO2 | U | 12 |
| 19. | a. | In detail explain the working of Magnetic particle testing with neat sketch and state its applications, merits and limitations. | CO3 | R | 12 |
| 20. | a. | Describe with neat sketch the Radiographic techniques and its limitations. | CO4 | R | 12 |
| 21. | a. | Enumerate the working principle of Ultrasonic technique with a block diagram and state its limitations. | CO5 | R | 12 |
| 22. | a. | Explain the principle of Eddy current testing and its limitations with neat sketch. | CO3 | U | 12 |
| 23. | a. | Explain the principle of Acoustic Emission technique and the various parameters involved in AET. | CO5 | U | 9 |
|  | b. | Write short note about modes of displays used in Radiography test. | CO5 | A | 3 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Write the history and development of Thermography test with its advantages and limitations. | CO6 | R | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understanding various types of discontinuities. |
| CO2 | Knowledge in non – destructive testing, its scope and purpose. |
| CO3 | Understand the different NDT processes. |
| CO4 | Evaluate the properties of material without causing damage. |
| CO5 | Learn dynamic behavior of defect with NDT tools. |
| CO6 | Choose the best NDT method for different application. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | R | U | A | An | E | C | Total |
| CO1 | 2 | 12 | 3 |  |  |  | 17 |
| CO2 |  | 16 | 1 |  |  |  | 17 |
| CO3 | 12 | 13 | 4 |  |  |  | 29 |
| CO4 | 12 |  | 1 |  | 3 |  | 16 |
| CO5 | 13 | 13 | 3 |  |  |  | 29 |
| CO6 | 12 | 1 | 3 |  |  |  | 16 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 18AE2039 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # CRYOGENIC PROPULSION | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | State the application of nitrogen as a cryogen. |  | CO1 | R | 1 |
| 2. | Explain why stainless steel is considered for handling cryogenic fluids. |  | CO1 | U | 1 |
| 3. | Discuss the need for expansion valve in simple Linde Hampson system. |  | CO2 | U | 1 |
| 4. | List the type of refrigeration system used below 4 K. |  | CO2 | R | 1 |
| 5. | Define C.O.P. |  | CO3 | R | 1 |
| 6. | State the influence of shape on the storage tank design. |  | CO4 | R | 1 |
| 7. | Define apparent thermal conductivity. |  | CO5 | R | 1 |
| 8. | State the temperature range for which gas filled power insulation in best suited. |  | CO5 | R | 1 |
| 9. | State the specific impulse range obtained from liquid hydrogen liquid oxygen propellant system. |  | CO6 | R | 1 |
| 10. | Illustrate the properties of liquid hydrogen. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Discuss how the specific heat capacity of a cryogenic fluid influences its storage capability. |  | CO1 | U | 3 |
| 12. | Explain1 Joule Thompson effects. |  | CO2 | U | 3 |
| 13. | Differentiate between Gifford Mc-Mohan (G-M) refrigerator and Solvay’s refrigerator. |  | CO3 | An | 3 |
| 14. | Explain the various Safety devices used to handle cryogenic cryogens. |  | CO4 | A | 3 |
| 15. | State the need for insulating cryogenic containers. |  | CO5 | R | 3 |
| 16. | Discuss the challenges associated with cryogenic fuel management in a spacecraft propulsion system. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Describe the properties of hydrogen, helium-3, and helium-4, and explain their applications in cryogenic engineering. | CO1 | U | 12 |
| 18. | a. | Differentiate between simple Linde Hampson system and dual pressure Linde Hampson system. | CO2 | A | 6 |
|  | b. | Describe the difference between open-loop and closed-loop cryogenic refrigeration systems. | CO2 | U | 6 |
| 19. | a. | Explain the role of Joule-Thomson expansion in achieving cryogenic temperatures. | CO2 | A | 4 |
|  | b. | Explain the concept of a thermodynamic ideal system, accompanied by a clear diagram. Additionally, derive the equation for the work required per unit mass of gas liquefied. | CO2 | A | 8 |
| 20. | a. | Discuss the challenges associated with heat transfer in cryogenic fluid storage and transfer lines. | CO5 | U | 6 |
|  | b. | Discuss the different materials and design considerations used to minimize heat transfer in these lines. | CO5 | U | 6 |
| 21. | a. | Describe the design procedure for inner and outer cryogenic storage vessel. | CO4 | R | 12 |
| 22. | a. | Explain the concept of multi-layer insulation (MLI) and its effectiveness in cryogenic propellant tanks. | CO5 | A | 12 |
| 23. | a. | Compare and contrast the working principles of two main types of liquefaction systems used for cryogenic fluids. | CO3 | U | 12 |
| COMPULSORY= QUESTION |  |  |  |  |  |
| 24. | a. | Discuss how the use of cryogenic propellants improves the performance of a rocket engine compared to conventional fuels. | CO6 | U | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the thermal, physical and flow properties of cryogenic fluids. |
| CO2 | Understand the liquefaction systems to produce cryogenic fluids |
| CO3 | Know the various method of cryogenic refrigeration systems |
| CO4 | Explain the various cryogenic fluid storage and transfer lines |
| CO5 | Design of various insulations for cryogenic propellant tanks |
| CO6 | Know the various applications of cryogenics in propulsion systems |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | R | U | A | An | E | C | Total |
| CO1 | 1 | 16 | - | - | - | - | 17 |
| CO2 | 1 | 10 | 18 | - | - | - | 29 |
| CO3 | 1 | 12 | - | 3 | - | - | 16 |
| CO4 | 13 | - | 3 | - | - | - | 16 |
| CO5 | 5 | 12 | 12 | - | - | - | 29 |
| CO6 | 2 | 15 | - | - | - | - | 17 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 18AE2041 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # ADVANCED SPACE DYNAMICS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | Define reference frames. |  | CO1 | R | 1 |
| 2. | Define perturbed motion. |  | CO1 | R | 1 |
| 3. | Define Lambert's Theorem. |  | CO2 | R | 1 |
| 4. | List the significance of Lambert’s theorem in orbital mechanics. |  | CO2 | R | 1 |
| 5. | Define planar restricted three body problem. |  | CO3 | R | 1 |
| 6. | Define Sidereal Day. |  | CO3 | R | 1 |
| 7. | Define critical mass. |  | CO4 | R | 1 |
| 8. | List the significance of Lagrangian point in celestial mechanics. |  | CO4 | R | 1 |
| 9. | Define comets. |  | CO5 | R | 1 |
| 10. | Define Lissajous orbits. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | Explain the fundamental principles and definitions that describe two body problem. |  | CO1 | U | 3 |
| 12. | Explain the applications of Lambert’s theorem. |  | CO2 | A | 3 |
| 13. | Explain Lagrangian frame in celestial mechanics |  | CO3 | A | 3 |
| 14. | Explain the computation of location of collinear and equilateral points. |  | CO4 | U | 3 |
| 15. | Explain Tisserand's criterion used for identifying comets |  | CO5 | A | 3 |
| 16. | Explain Earth oblateness. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No. 17 to 23, Q.No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Explain by deriving the differential equation governing the relative motion of a spacecraft or satellite with respect to a planet in a two-body system. | CO1 | U | 12 |
| 18. |  | Derive Lambert’s theorem and explain the mathematical steps involved. | CO2 | A | 12 |
| 19. |  | Develop the fifth-degree algebraic equations used to determine the positions of the collinear Lagrangian points L1, L2, and L3 in celestial mechanics. | CO3 | A | 12 |
| 20. |  | Derive Jacobi Integral and explain its significance in describing the motion of a particle in a rotating coordinate system. | CO3 | A | 12 |
| 21. |  | Determine the second-order derivatives at the collinear points with respect to position at the collinear Lagrangian points. | CO4 | A | 12 |
| 22. |  | Describe the complexities and dynamics involved in the three-dimensional restricted three-body problem. | CO5 | U | 12 |
| 23. |  | Explain Halo orbits and its complex dynamics. | CO6 | U | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Explain the perturbed restricted three-dimensional three-body problem with earth oblateness and solar radiation pressure. | CO6 | U | 12 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Ability to understand two-body orbital motion and regularization. |
| CO2 | Gain knowledge of orbital transfer technique |
| CO3 | Understand planar restricted three-body problem. |
| CO4 | Understand orbital motion in planar restricted three-body problem. |
| CO5 | Attain knowledge of 3-dimensional restricted three-body problem and identification of comets. |
| CO6 | Gain knowledge of halo orbits and perturbed 3 body problem. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | R | U | A | An | E | C | Total |
| CO1 | 2 | 15 | - | - | - | - | 17 |
| CO2 | 2 | - | 15 | - | - | - | 17 |
| CO3 | 2 | - | 27 | - | - | - | 29 |
| CO4 | 2 | 3 | 12 | - | - | - | 17 |
| CO5 | 1 | 12 | 3 | - | - | - | 16 |
| CO6 | 1 | 27 | - | - | - | - | 28 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 18AE2043 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # AIRCRAFT SYSTEMS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | Name the component that uses the hydraulic power. |  | CO1 | R | 1 |
| 2. | State the significance of fine coating over the surface of cellulose cartridge. |  | CO1 | R | 1 |
| 3. | Name the landing gear on helicopter. |  | CO2 | R | 1 |
| 4. | Name the aircraft that uses power boost system. |  | CO2 | R | 1 |
| 5. | State the application of single ignition system. |  | CO3 | R | 1 |
| 6. | Identify the composition of grade 91 fuel. |  | CO3 | R | 1 |
| 7. | Identify the lubrication system in the most turbojet engines. |  | CO4 | R | 1 |
| 8. | Identify the role of face mask in the combat aircraft. |  | CO5 | R | 1 |
| 9. | Identify the source of gas of MSOC. |  | CO5 | R | 1 |
| 10. | Name the ice protection system that prevents ice on aircraft. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | Name any three fluid contamination tests. |  | CO1 | R | 3 |
| 12. | Classify shimmy dampers. |  | CO2 | U | 3 |
| 13. | Explain the characteristics of gaseous fuels. |  | CO3 | U | 3 |
| 14. | Summarize the cooling methods of reciprocating engine. |  | CO4 | U | 3 |
| 15. | State the main purpose of cabin distribution system. |  | CO5 | R | 3 |
| 16. | Name the locations of fire protection system in the aircraft. |  | CO6 | R | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Describe the function of typical pneumatic system with a neat sketch. | CO1 | U | 12 |
| 18. | a. | Explain the wheel arrangements of landing gear. | CO2 | U | 6 |
|  | b. | Illustrate the working principle of hydraulic landing gear retraction system. | CO2 | U | 6 |
| 19. |  | Explain the components and working of magneto ignition system. | CO3 | U | 12 |
| 20. |  | Discuss the characteristics of aviation fuel and the methods of quality testing. | CO3 | U | 12 |
| 21. | a. | Explain dry-sump lubrication system with a neat sketch. | CO4 | U | 6 |
|  | b. | Describe the cooling methods of turbine engine. | CO4 | U | 6 |
| 22. |  | Discuss the features of aircraft cabin pressurization system with a neat diagram. | CO5 | U | 12 |
| 23. |  | Explain the salient features of rain removal system with a neat sketch. | CO6 | U | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Explain aircraft position and warning system. | CO6 | U | 12 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the principles Aircraft Hydraulic systems |
| CO2 | Obtain knowledge on the Landing Gear systems |
| CO3 | Obtain knowledge on Fuel systems and Engine starting systems |
| CO4 | Diagnose aircraft engine starting systems performance |
| CO5 | Obtain knowledge on Cabin atmosphere control systems |
| CO6 | Understand the basics of auxilliary systems in aircraft |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | R | U | A | An | E | C | Total |
| CO1 | 5 | 12 | - | - | - | - | 17 |
| CO2 | 2 | 15 | - | - | - | - | 17 |
| CO3 | 2 | 27 | - | - | - | - | 29 |
| CO4 | 2 | 15 | - | - | - | - | 17 |
| CO5 | 4 | 12 | - | - | - | - | 16 |
| CO6 | 4 | 24 | - | - | - | - | 28 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 18AE3027 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # UNMANNED AIRCRAFT SYSTEMS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |  |
| 1. | a. | Compare and contrast HALE and MALE UAVs. | CO1 | U | 8 |
|  | b. | Explain the salient features of a remotely piloted helicopter (RPH). | CO1 | U | 8 |
| 2. |  | Explain the various types of horizontal take-off and Landing (HTOL) configurations of UAVs based on lift/mass balance, stability and control. | CO2 | U | 16 |
| 3. | a. | Write in detail the selection of powerplant for the Unmanned Aerial Vehicle (UAV). | CO3 | A | 8 |
|  | b. | Compare the characteristics of a piston engine with a gas turbine engine. | CO3 | U | 8 |
| 4. |  | Examine the unique UAV launching mechanisms designed for particular aircraft types and purposes. | CO4 | An | 16 |
| 5. |  | Explain the methods employed in the testing of VTOL aircraft and documentation of results. | CO5 | U | 16 |
| 6. |  | Explain the design concepts of stealth aircraft to prevent it from enemy forces. | CO3 | U | 16 |
| 7. |  | Describe the non-dispensable payloads that are available for UAVs and the mounting techniques used. | CO4 | U | 16 |
| PART – B (1 X 20 = 20 MARKS) [Compulsory Question] |  |  |  |  |  |
| 8. |  | Discuss the commercial, industrial and military applications of UAVs. | CO6 | U | 20 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL        M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the basic terminologies and classification of UAS. |
| CO2 | Relate the design parameters of UAV systems. |
| CO3 | Obtain knowledge on the application of UAV standards to design UAS. |
| CO4 | Obtain knowledge on payloads and launch systems for UAS. |
| CO5 | Understand the basic principles of UAV Testings. |
| CO6 | Apply the principles to design UAS for specific applications. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | R | U | A | An | E | C | Total |
| CO1 | - | 16 | - | - | - | - | 16 |
| CO2 | - | 16 | - | - | - | - | 16 |
| CO3 | - | 24 | 8 |  |  |  | 32 |
| CO4 | - | 16 | - | 16 | - | - | 32 |
| CO5 | - | 16 | - | - | - | - | 16 |
| CO6 | - | 20 | - | - | - | - | 20 |
|  |  |  |  |  |  |  | 132 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 19AE2004 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # ENGINEERING DESIGN AND COST ENGINEERING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Define the “maturity stage” in PLC. |  | CO1 | R | 1 |
| 2. | List out the various types of design. |  | CO3 | R | 1 |
| 3. | Explain the “theory of inventive problem solving”. |  | CO2 | U | 1 |
| 4. | Illustrate any two ideas which can be used to develop a technical product. |  | CO3 | U | 1 |
| 5. | Show the relation between value and cost by drawing a related sketch. |  | CO3 | U | 1 |
| 6. | State the reasons for developing “Alternates”. |  | CO4 | R | 1 |
| 7. | Define the term “Lean Design”. |  | CO5 | R | 1 |
| 8. | List out the various causes of a project failure. |  | CO5 | R | 1 |
| 9. | Write about the various members in “value engineering team”. |  | CO6 | U | 1 |
| 10. | Indicate the applications of Rapid Prototyping. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Write the use of Quality function deployment (QFD). |  | CO4 | A | 3 |
| 12. | Show the expected sales and profit pattern corresponding to various stages of product life cycle process with help of a diagram. |  | CO3 | U | 3 |
| 13. | Show the difference between “Make or Buy” decision with an example. |  | CO6 | U | 3 |
| 14. | Write the process of measuring the profit with a suitable example. |  | CO1 | A | 3 |
| 15. | List out the types of “construction contracts”. |  | CO2 | R | 3 |
| 16. | Explain briefly the Fused Deposition Modeling (FDM). |  | CO5 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No 17 to 23, Q.No 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Sketch a block diagram which indicates the various idea / design process and explain in detail. | CO3 | A | 12 |
| 18. |  | With an example illustrate the various steps involved in value engineering process. | CO6 | An | 12 |
| 19. |  | Appraise the parameters to be considered while assigning a rupee equivalent to a product. | CO2 | An | 12 |
| 20. |  | Analyze the steps of construction of a FAST diagram. Draw the block diagram. | CO4 | An | 12 |
| 21. |  | Explain the weighted decision matrix method for decision making process in detail. | CO5 | A | 12 |
| 22. |  | Write the various phases of “construction management contract” in detail. | CO5 | A | 12 |
| 23. |  | Write the five principles that are examined during the DFM with suitable examples. | CO6 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Illustrate the Selective Laser Sintering (SLS) method with help of a diagram and write its applications. | CO5 | A | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Appreciate of the concept of Product Life Cycle |
| CO2 | Conduct requirement analysis |
| CO3 | Generate ideas, evaluate and select engineering techniques |
| CO4 | Carryout FMEA, Fault Tree Analysis etc. |
| CO5 | Carry out functional analysis |
| CO6 | Apply the basics of Value Engineering |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | 1 | - | 3 | - | - | - | 3 |
| CO2 | 3 | 1 | - | 12 | - | - | 16 |
| CO3 | 1 | 8 | 12 | - | - | - | 21 |
| CO4 | 1 | - | 3 | 12 | - | - | 16 |
| CO5 | 2 | - | 36 | - | - | - | 36 |
| CO6 | - | 5 | 12 | 12 | - | - | 29 |
| Total | 8 | 14 | 66 | 36 | - | - | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 20AE2001 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # INTRODUCTION TO AEROSPACE ENGINEERING | # Max. Marks | # 100 |

| Q. No. | Questions |  |  | CO |  | BL |  | M |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |  |  |  |
| 1. | Describe the center of pressure. |  |  | CO1 |  | R |  | 1 |
| 2. | Explain the function of spoilers. |  |  | CO1 |  | U |  | 1 |
| 3. | Describe chord line of an airfoil. |  |  | CO2 |  | R |  | 1 |
| 4. | State the purpose of winglets in the wing. |  |  | CO2 |  | R |  | 1 |
| 5. | Explain the function of a rudder in an airplane. |  |  | CO3 |  | U |  | 1 |
| 6. | Name the material which is commonly used in the construction of aircraft wings. |  |  | CO3 |  | R |  | 1 |
| 7. | List the disadvantage of a turboprop engine. |  |  | CO4 |  | R |  | 1 |
| 8. | State advantage of a turboprop engine over a turbojet engine |  |  | CO4 |  | R |  | 1 |
| 9. | State gravitational law. |  |  | CO5 |  | R |  | 1 |
| 10. | Define Newton’s second law. |  |  | CO6 |  | R |  | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |  |  |  |
| 11. | Explain the contributions of the Wright brothers. |  |  | CO1 |  | U |  | 3 |
| 12. | Write a brief note on airfoil nomenclature with a neat sketch. |  |  | CO2 |  | A |  | 3 |
| 13. | Distinguish between monocoque and semi-monocoque structure. |  |  | CO3 |  | U |  | 3 |
| 14. | Explain principle of power generation in a four stroke reciprocating engine used in airplane. |  |  | CO4 |  | U |  | 3 |
| 15. | List the classification of chemical rocket propulsion. |  |  | CO5 |  | R |  | 3 |
| 16. | Explain Kepler’s first law of motion. |  |  | CO6 |  | U |  | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No. 17 to 23, Q.No. 24 is Compulsory) |  |  |  |  |  |  |  |  |
| 17. |  | Define standard atmosphere. Derive an expression for pressure and density ratio in the isothermal region of the standard atmosphere. | CO1 |  | R |  | 12 |  |
| 18. |  | Explain the function of primary and secondary flight control surfaces. | CO2 |  | U |  | 12 |  |
| 19. | a. | With help of a neat sketch explain wing construction. | CO3 |  | U |  | 6 |  |
|  | b. | With help of a neat sketch explain fuselage construction. | CO3 |  | U |  | 6 |  |
| 20. |  | Describe the working of a turbo jet engine with schematic diagram. Depict the various thermodynamic processes occurring in it on P-V diagram. | CO4 |  | R |  | 12 |  |
| 21. | a. | Discuss the working principle of rocket with a neat sketch. | CO5 |  | R |  | 9 |  |
|  | b. | Write short notes on specific impulse. | CO5 |  | A |  | 3 |  |
| 22. |  | Deduce the Vis-viva equation for orbiting bodies. | CO6 |  | An |  | 12 |  |
| 23. |  | Explain the classifications of aircraft according to the wing, landing gear, and engine with a neat sketch. | CO3 |  | A |  | 12 |  |
| COMPULSORY QUESTION |  |  |  |  |  |  |  |  |
| 24. | a. | Explain the theory of lift generation. | CO2 |  | A |  | 8 |  |
|  | b. | Write short notes on wing tip vortices. | CO2 |  | A |  | 4 |  |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL              M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the nature of aerospace technologies. |
| CO2 | Identify the different types of Aircraft components and their functions. |
| CO3 | Assess the forces and moments due to flow over the aircraft components. |
| CO4 | Apply the principles of aerodynamics to different parts of an aeroplane. |
| CO5 | Evaluate the performance of propulsion system. |
| CO6 | Apply the knowledge of gravitational law, Kepler’s law and Newton’s law to the space vehicle |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | 13 | 4 |  |  |  |  | 17 |
| CO2 | 2 | 12 | 15 |  |  |  | 29 |
| CO3 | 1 | 16 | 12 |  |  |  | 29 |
| CO4 | 14 | 3 |  |  |  |  | 17 |
| CO5 | 13 |  | 3 |  |  |  | 16 |
| CO6 | 1 | 3 |  | 12 |  |  | 16 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 20AE2002 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # BASICS OF FLUID MECHANICS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | Specific weight of water in SI units is equal to _________. |  | CO1 | U | 1 |
| 2. | Manometer is used to measure ___________. |  | CO1 | R | 1 |
| 3. | Continuity equation deals with the law of conservation of _________. |  | CO2 | R | 1 |
| 4. | Number of stagnation points present in a source and sink pair in a uniform flow is ____________. |  | CO2 | U | 1 |
| 5. | A submerged body will be in stable equilibrium if the centre of buoyancy is ______ the centre of gravity. |  | CO3 | U | 1 |
| 6. | The working of orifice meter is based on _________ theorem. |  | CO4 | U | 1 |
| 7. | The friction head loss due to the flow of a viscous fluid through a circular pipe of length L and diameter d with a velocity v, and Fanning pipe friction ‘f’ is given by ____________________. |  | CO5 | R | 1 |
| 8. | For the laminar flow through a circular pipe the maximum velocity is __________ times the average velocity. |  | CO5 | R | 1 |
| 9. | The ratio of inertia force to viscous force of fluid is ___________ number. |  | CO6 | R | 1 |
| 10. | The force exerted by a jet of water on a stationary inclined plate in the direction of jet is given by ______________. |  | CO2 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | A hydraulic press has a ram of 30 cm diameter and plunger of 5 cm diameter. Find the weight lifted by the hydraulic press. When the force applied at the plunger is 400 N. |  | CO1 | A | 3 |
| 12. | Determine the velocity of flow at radii of 0.2 m, 0.4 m and 0.8 m, when the water is flowing radially outward in a horizontal plane from a source at a strength of 12m2 /s |  | CO2 | A | 3 |
| 13. | Explain the terms ‘meta-centre’ and ‘meta-centric height’ |  | CO3 | R | 3 |
| 14. | Draw a neat sketch for venturimeter. |  | CO4 | R | 3 |
| 15. | Estimate the loss of head when a pipe of diameter 200mm is suddenly enlarged to a diameter of 400 mm. The rate of flow of water through the pipe is 250 litres/s. |  | CO5 | A | 3 |
| 16. | State Buckingham’s π Theorem. |  | CO6 | R | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No. 17 to 23, Q.No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | If the velocity distribution over a plate is given by u=23y-y2 in which u is the velocity in metre per second at a distance y metre above the plate, determine the shear stress at y=0 and y=0.15m. Take dynamic viscosity of fluid as 8.63 Poises. | CO1 | A | 6 |
|  | b. | Determine the total pressure on a circular plate of diameter 1.5m which is placed vertically in water in such a way that the centre of the plate is 3m below the free surface of water. Find the position of centre of pressure also. | CO1 | A | 6 |
| 18. | a. | The velocity vector in a fluid flow is given by V=4x3i-10x2yj+2tk. Determine the velocity and acceleration of a fluid particle at (2,1,3) at time t=1. | CO2 | A | 6 |
|  | b. | The velocity potential function is given by =5(x2-y2). Calculate the velocity components at the point (4,5) | CO2 | A | 6 |
| 19. |  | A uniform flow with a velocity of 3 m/s is flowing over a plane source of strength 30 m2/s. The uniform flow and source flow are in the same plane. A point P is situated in the flow field. The distance of the point P from the source is 0.5 m and it is at an angle of 300 to the uniform flow. Determine: i) stream function at point P ii) Resultant velocity of flow at P and iii) location of stagnation point from the source. | CO2 | A | 12 |
| 20. |  | A 30 cm x 15 cm venturimeter is provided in a vertical pipe line carrying oil of specific gravity 0.9, the flow being upwards. The difference in elevation of the throat section and entrance section of the venturimeter is 30cm. The differential manometer shows a gauge deflection of 25cm. Calculate: i) the discharge of oil, and ii) the pressure difference between the entrance section and the throat section. Take the co-efficient of discharge as 0.98 and specific gravity of mercury as 13.6. | CO4 | A | 12 |
| 21. |  | A fluid of viscosity 0.7 Ns/m2 and specific gravity 1.3 is flowing through a circular pipe of diameter 100 mm. The maximum shear stress at the pipe wall is given as 196.2 N/m2. Determine i) the pressure gradient, ii) the average velocity and iii) Reynolds number of the flow. | CO5 | A | 12 |
| 22. |  | Determine the rate of flow of water through a pipe of diameter 20cm and length 50m when one end of the pipe is connected to a tank and other end of the pipe is open to the atmosphere. The pipe is horizontal and the height of water in the tank is 4m above the centre of the pipe. Consider all minor losses and take f = 0.009 in the formula hf= 4fLV2/2gd. | CO5 | A | 12 |
| 23. |  | The variables controlling the motion of a floating vessel through water are the drag force F, the speed V, the length L, the density and dynamic viscosity of water and acceleration due to gravity g. Derive an expression for F by dimensional analysis. | CO6 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | A jet of water of diameter 150mm strikes a flat plate normally with a velocity of 20 m/s. The plate is moving with a velocity of 6 m/s in the direction of the jet and away from the jet. Find: (i) the force exerted by the jet on the plate, (ii) work done by the jet on the plate per second. (iii) Power of the jet and iv) efficiency of the jet. | CO2 | A | 12 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Know the properties of different fluids and pressure measurements. |
| CO2 | Apply mathematical knowledge to predict the properties and characteristics of a fluid. |
| CO3 | Understand the nature of buoyancy of submerged and floating bodies. |
| CO4 | Attain the Knowledge of flow measurement systems. |
| CO5 | Estimate the friction factor of pipe flow and losses associated with it. |
| CO6 | Understand the non-dimensional parameters used in airflow. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | 1 | 1 | 15 | - | - | - | 17 |
| CO2 | 1 | 2 | 39 | - | - | - | 42 |
| CO3 | 3 | 1 | - | - | - | - | 4 |
| CO4 | 3 | 1 | 12 | - | - | - | 16 |
| CO5 | 2 | - | 27 | - | - | - | 29 |
| CO6 | 4 | - | 12 | - | - | - | 16 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 20AE2005 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # STRENGTH OF MATERIALS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | Identify the unit of strain. |  | CO1 | R | 1 |
| 2. | Define the modulus of elasticity. |  | CO1 | R | 1 |
| 3. | Define the sagging moment. |  | CO2 | R | 1 |
| 4. | Define the overhanging beam. |  | CO2 | R | 1 |
| 5. | Define the neutral axis of a beam. |  | CO3 | R | 1 |
| 6. | Write the equation of the theory of simple bending. |  | CO3 | A | 1 |
| 7. | List the methods of finding slope and deflection at a section in a loaded beam. |  | CO4 | R | 1 |
| 8. | Write the torsion equation of the shaft. |  | CO5 | A | 1 |
| 9. | Define thin cylinder. |  | CO6 | R | 1 |
| 10. | Define principal plane. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | Sketch the stress-strain curve for a ductile material. |  | CO1 | A | 3 |
| 12. | A cantilever 9 m long has a uniformly distributed load over the entire length. The maximum bending moment is 8100 Nm. Estimate the rate of loading. |  | CO2 | U | 3 |
| 13. | Explain the assumptions in pure bending. |  | CO3 | U | 3 |
| 14. | A beam 4 m long, simply supported at its ends, carries a point load W at its center. If the slope at the ends of the beam is not to exceed 1˚, estimate the deflection at the center of the beam. |  | CO4 | U | 3 |
| 15. | Explain the strength of the shaft. |  | CO5 | U | 3 |
| 16. | A cylinder of internal diameter of 0.5 m contains air at a pressure of 7 N/mm2 (gauge). If the maximum permissible stress induced in the material is 80 N/mm2, estimate the thickness of the cylinder. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No. 17 to 23, Q.No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | A member ABCD is subjected to point loads P1, P2, P3 and P4 as shown in fig. below. Calculate the force P2 necessary for equilibrium, if P1= 45 kN, P3= 450 kN and P4=130 kN. Calculate the total elongation of the member, assuming the modulus of elasticity to be 2.1 X 105 N/mm2. | CO1 | A | 6 |
|  | b. | A steel rod of 3 cm is enclosed centrally in a hollow copper tube of external diameter 5 cm and internal diameter of 4 cm. The composite bar is then subjected to an axial pull of 45000 N. If the length of each bar is equal to 15 cm, calculate the following: - The stresses in the rod and tube - Load carried by each bar Take E for steel = 2.1 X 105 N/mm2 and for copper = 1.1 X 105 N/mm2 | CO1 | A | 6 |
| 18. | a. | A cantilever of length 2 m carries a uniformly distributed load of 1.5 kN/m run over the whole length and a point load of 2kN at a distance of 0.5 m from the free end. Show the shear force and bending moment diagrams for the cantilever. | CO2 | U | 6 |
|  | b. | Show the shear force and bending moment diagrams for a simply supported beam of length 8m and carrying a uniformly distributed load of 10 kN/m for a distance of 4 m as shown in fig. below. | CO2 | U | 6 |
| 19. | a. | A cast iron bracket subject to bending has the cross-section of an I-form with unequal flanges. The dimensions of the section are shown in fig. below. Predict the position of the neutral axis and moment of inertia of the section about the neutral axis. If the maximum bending moment on the section is 40 MN mm, determine the maximum bending stress. | CO3 | A | 12 |
| 20. | a. | A cantilever of length 2 m carries a point load of 20 kN at the free end and another load of 20 kN at its center. If E 105 N/mm2 and I = 108 mm4 for the cantilever then determine by moment area method, the slope and deflection of the cantilever at the free end. | CO4 | A | 12 |
| 21. | a. | A hollow shaft, having an inside diameter of 60% of its outer diameter, is to replace a solid shaft transmitting the same power at the same speed. Calculate the percentage saving in material, if the material to be used is also the same. | CO5 | A | 12 |
| 22. | a. | A close coiled helical spring of 10 cm mean diameter is made up of 1 cm diameter rod and has 20 turns. The spring carries an axial load of 200 N. Determine the shearing stress. Taking the value of modulus of rigidity = 8.4 X 104 N/mm2, determine the deflection when carrying this load. Also calculate the stiffness of the spring and the frequency of free vibration for a mass hanging from it. | CO5 | A | 12 |
| 23. | a. | The tensile stresses at a point across two mutually perpendicular planes are 120 N/mm2 and 60 N/mm2. Determine the normal, tangential and resultant stresses on a plane inclined at 30˚ to the axis of the minor stress using Mohr’s circle method. | CO6 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Show the shear force and bending moment diagrams for the overhanging beam carrying a uniformly distributed load of 2 kN/m over the entire length and a point load of 2 kN as shown in fig. below. Predict the point of contraflexure. | CO2 | U | 12 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Describe the characteristics of conventional metals. |
| CO2 | Understand the effect loads acting at different sections of the beam. |
| CO3 | Calculate the stresses developed in beams. |
| CO4 | Compare different methods of beam deflection. |
| CO5 | Analyze the stresses developed in the shaft and spring. |
| CO6 | Analyze the states of stress in a 2D oblique plane. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | 2 | - | 15 | - | - | - | 17 |
| CO2 | 2 | 27 | - | - | - | - | 29 |
| CO3 | 1 | 3 | 13 | - | - | - | 17 |
| CO4 | 1 | 3 | 12 | - | - | - | 16 |
| CO5 | - | 3 | 25 | - | - | - | 28 |
| CO6 | 2 | 3 | 12 | - | - | - | 17 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 20AE2007 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # ENGINEERING THERMODYNAMICS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | A tank is filled with oil whose density is 850 kg/m3. If the volume of the tank is V = 2 m3, determine the amount of mass m of oil in the tank. |  | CO1 | R | 1 |
| 2. | State intensive and extensive properties. |  | CO1 | R | 1 |
| 3. | During a heating process, the temperature of a system rises by 23°C. Express this rise in temperature in K, °F, and R. |  | CO1 | R | 1 |
| 4. | In 1 kg mixture of gas at 1.013 bar and 300 k the various constituent’s gases are as follows 75% nitrogen, 18 % oxygen and remaining carbon dioxide. Determine gas constants for each gas constituents. Take universal gas constant as 8314 J/kg K. |  | CO2 | R | 1 |
| 5. | Explain PVT surface. |  | CO2 | U | 1 |
| 6. | State the limitation of first law of thermodynamics. |  | CO3 | R | 1 |
| 7. | Define thermal reservoir. |  | CO4 | R | 1 |
| 8. | A fluid at 200 kPa and 300 C has a volume of 0.8 m3. In a frictionless process at constant volume the pressure is 100 kPa, find the final temperature of the air. |  | CO5 | U | 1 |
| 9. | Draw the PV and TS diagram of an otto cycle. |  | CO6 | R | 1 |
| 10. | State the various types of gas power cycles. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | Distinguish between the terms change of state, path, and process· |  | CO1 | U | 3 |
| 12. | A rigid tank contains a hot fluid that is cooled while being stirred by a paddle wheel. Initially, the internal energy of the fluid is 800 kJ. During the cooling process, the fluid loses 500 kJ of heat, and the paddle wheel does 100 kJ of work on the fluid. Determine the final internal energy of the fluid. Neglect the energy stored in the paddle wheel. |  | CO2 | U | 3 |
| 13. | The table given below shows the values of heat transfer and work done during aa cyclic process. Determine work interaction for the process 4-1and state whether it is done upon the system or by the system. Process; Q (kJ); W (kJ). 1-2; 850; 75. 2-3; -80; -65. 3-4; -700; 70. 4-1; 210; ?. | CO3 | A | 3 |
| 14. | Explain Dalton’s law of partial pressure. |  | CO4 | U | 3 |
| 15. | State Avogadro law. |  | CO5 | R | 3 |
| 16. | Differentiate between vapor compression refrigeration and vapor absorption refrigeration cycle. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No. 17 to 23, Q.No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | A manometer is used to measure the pressure in a tank. The fluid used has a specific gravity of 0.85, and the manometer column height is 55 cm, as shown in Fig. If the local atmospheric pressure is 96 kPa, determine the absolute pressure within the tank. | CO1 | R | 6 |
|  | b. | A rigid tank contains a hot fluid that is cooled while being stirred by a paddle wheel. Initially, the internal energy of the fluid is 800 kJ. During the cooling process, the fluid loses 500 kJ of heat, and the paddle wheel does 100 kJ of work on the fluid. Determine the final internal energy of the fluid. Neglect the energy stored in the paddle wheel. | CO1 | R | 6 |
| 18. | a. | In a steady flow apparatus, 135 kJ of work is done by each kg of fluid. The specific volume of the fluid, pressure and velocity at the inlet are 0.37 m3 /kg, 600 kPa, and 16 m/s. The inlet is 32 m above the floor, and the discharge pipe is at floor level. The discharge conditions are 0.62 m3/kg, 100 kPa, and 270 m/s. The total heat loss between the inlet and discharge is 9 kJ/kg of fluid. In flowing through this apparatus, docs the specific internal energy increase or decrease and by how much? | CO2 | A | 8 |
|  | b. | State Kelvin-plank and Clausius statements of second law. | CO2 | R | 4 |
| 19. | a. | Explain the T-V diagram for the heating process of water at 1 atm pressure. | CO2 | U | 6 |
|  | b. | Explain the difference between the ideal gas law and the compression equation. | CO2 | U | 6 |
| 20. | a. | A cyclic heat engine operates between a source temperature of 800 ֯C and a sink temperature of 30 ֯C. What is the least rate of heat rejection per kW net output of the engine? | CO3 | A | 6 |
|  | b. | Derive transfer of heat through a finite temperature difference is ΔS univ =Q(T1-T2)T1T2 | CO4 | U | 6 |
| 21. |  | (a) One kg of water at 273 K is brought into contact with a heat reservoir at 373 K. When the water has reached 373 K. find the entropy change of the water, of the heat reservoir, and of the universe. (b) If water is heated from 273 K to 373 K by first bringing it in contact with a reservoir at 323 K and then with a reservoir at 373 K. what will the entropy change of the universe be? Take Cv = 4.187 kJ/Kg K. | CO4 | An | 12 |
| 22. | a. | Derive Maxwell's equations. | CO5 | U | 6 |
|  | b. | Derive Clausius-Clapeyron equation. | CO5 | U | 6 |
| 23. | a. | An engine working on the Otto cycle is supplied with air at 0.1 MPa, 35°C. The compression ratio is 8. Heat supplied is 2100 kI/kg. Calculate the maximum pressure and temperature of the cycle, the cycle efficiency, and the mean effective pressure. (For air, cP = l.005, Cv = 0.718, and R = 0.287 kJ/kg K). | CO6 | An | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Write short note on four stroke diesel cycle with neat sketch. | CO6 | A | 8 |
|  | b. | State the difference between otto cycle and diesel cycle. | CO6 | R | 4 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the basic concepts of thermodynamics, laws of thermodynamics and types of work and heat interactions. |
| CO2 | Evaluate the properties of pure substances, ideal gases and real gases from property tables or state equations. |
| CO3 | Apply the first law of thermodynamics for closed and open systems undergoing different thermodynamic processes and cycles. |
| CO4 | Understand the concept of entropy and properties of pure substances and real gases |
| CO5 | Perform energy calculations of engineering systems and analyze the feasibility of the processes undergone by the systems. |
| CO6 | Evaluate the efficiency and co-efficient of performance of thermal systems and vapor power cycles. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | R | U | A | An | E | C | Total |
| CO1 | 15 | 3 | - | - | - | - | 18 |
| CO2 | 5 | 16 | 8 | - | - | - | 29 |
| CO3 | 1 | - | 9 | - | - | - | 10 |
| CO4 | 1 | 9 | - | 12 | - | - | 22 |
| CO5 | 3 | 13 | - | - | - | - | 16 |
| CO6 | 6 | 3 | 8 | 12 | - | - | 29 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 20AE2009 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # AERODYNAMICS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | Illustrate, how the location of the center of pressure changes with varying angles of attack on an airfoil. |  | CO1 | U | 1 |
| 2. | Explain the basic principle underlying Bernoulli's equation and how it relates to fluid flow. |  | CO2 | U | 1 |
| 3. | Describe the term “chord” in the context of airfoil characteristics. |  | CO3 | R | 1 |
| 4. | State the fundamental concept behind a vortex sheet in fluid dynamics. |  | CO4 | R | 1 |
| 5. | Illustrate source sheet with a neat sketch. |  | CO5 | U | 1 |
| 6. | Define the term boundary layer thickness in terms of free stream velocity. |  | CO6 | R | 1 |
| 7. | Describe the significance of “aerodynamic centre” in perspective of an aircraft wing. |  | CO1 | R | 1 |
| 8. | Express the relationship between the velocity components (u and v) and the velocity potential function (ϕ) in an irrotational flow. |  | CO2 | U | 1 |
| 9. | Explain the concept of conformal mapping in complex analysis and its significance in mathematical modeling and engineering applications. |  | CO3 | U | 1 |
| 10. | Establish the two basic principles of vortex behavior put forward by Hermann Von Helmholtz. |  | CO4 | A | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | Compare and contrast vorticity and circulation in terms of their physical interpretations and mathematical representations. |  | CO1 | U | 3 |
| 12. | Evaluate the conditions under which the Kutta–Joukowski theorem is valid in describing lift generation. What are the limitations of this theorem in practical aerodynamics? |  | CO2 | An | 3 |
| 13. | Explain the significance of the term "angle of attack" in airfoil nomenclature and how it affects lift. Draw neat sketch. |  | CO3 | U | 3 |
| 14. | Describe the significance and importance of term “downwash” in the context of aerodynamics with neat sketch. |  | CO4 | R | 3 |
| 15. | Define boundary point and control point. |  | CO5 | R | 3 |
| 16. | Distinguish between laminar region and turbulent region of boundary layer in the context of flow over a flat plate. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No. 17 to 23, Q.No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Given a control volume and fluid flow scenario, describe and derive the momentum equation in the integral form using the principles of momentum conservation, divergence theorem and Gradient theorem. Provide a step-by-step explanation of the derivation. | CO1 | U | 12 |
| 18. | a. | Write the mathematical expression of Euler equation. | CO2 | A | 2 |
|  | b. | Describe and derive the expression for stream function (ψ) and velocity potential function (Φ) in polar coordinates for doublet with neat sketch. | CO2 | U | 10 |
| 19. | a. | State the concept of thin airfoil in terms of vortex sheet with neat sketch. | CO3 | R | 3 |
|  | b. | Prove that using Joukowski transformation, circle in the complex plane Z can be transformed to an ellipse in complex plane W, except at radius = 1. | CO3 | A | 9 |
| 20. |  | Derive and assess the expression of the following parameters for an elliptical lift distribution with. Where = Circulation and b = blade span. |  |  |  |
|  | a. | downwash (w(θ0)) | CO4 | E | 5 |
|  | b. | induced angle of attack (α(i)) | CO4 | E | 2 |
|  | c. | lift (L) | CO4 | E | 5 |
| 21. |  | Deduce the expression for pressure distribution over the non lifting body of arbitrary shape using source panel method. | CO5 | An | 12 |
| 22. |  | Explain briefly and derive the expressions for the following parameters from the context of boundary layer theory. |  |  |  |
|  | a. | Displacement thickness | CO6 | U | 4 |
|  | b. | Momentum thickness | CO6 | U | 4 |
|  | c. | Energy thickness | CO6 | U | 4 |
| 23. |  | Write short notes on the following terms with neat sketch and appropriate expressions. |  |  |  |
|  | a. | Kutta condition | CO3 | A | 4 |
|  | b. | Kelvin circulation theorem | CO3 | A | 4 |
|  | c. | Starting vortex | CO3 | A | 4 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Consider an elliptical wing with 11m span and 3.5m root chord. If the wing is made up of NACA23012 wing sections and is flying 60m/s at a geometric angle of attack of 8 degree. (Given: density = 1.225 kg/m3, Circulation (Γ0) = 45 m3rads , Span area = 20 m2). Compute - aspect ratio (AR), b) coefficient of lift (CL), c) coefficient of induced drag (CDi), d) total lift (L), e) total induced drag (Di), f) induced angle of attack (αi), effective angle of attack (αeff). | CO4 | A | 12 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the aerodynamic variables connected with airflow.. |
| CO2 | Understand the concept of basic flows and its characteristics.. |
| CO3 | Develop the knowledge of incompressible flow over airfoil.. |
| CO4 | Assess the flow field over a finite wing span.. |
| CO5 | Estimate the flow parameters over aircraft wings and fuselages.. |
| CO6 | Understand the concept of the boundary layer and its characteristics.. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | R | U | A | An | E | C | Total |
| CO1 | 1 | 16 |  |  |  |  | 17 |
| CO2 |  | 12 | 2 | 3 |  |  | 17 |
| CO3 | 4 | 4 | 21 |  |  |  | 29 |
| CO4 | 4 | - | 13 |  | 12 |  | 29 |
| CO5 | 3 | 1 |  | 12 |  |  | 16 |
| CO6 | 1 | 15 |  |  |  |  | 16 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 20AE2011 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # AEROSPACE STRUCTURES-I | # Max. Marks | # 100 |

| Q. No. | Questions |  |  | CO |  | BL |  | M |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |  |  |  |
| 1. | State the conditions of equilibrium. |  |  | CO1 |  | R |  | 1 |
| 2. | Identify the condition of a statically determinate truss. |  |  | CO1 |  | R |  | 1 |
| 3. | State carry-over theorem. |  |  | CO2 |  | R |  | 1 |
| 4. | List the assumptions of Clapeyron’s three-moment theorem. |  |  | CO2 |  | R |  | 1 |
| 5. | Identify the expression of maximum deflection for a cantilever beam with a point load at the free end. |  |  | CO3 |  | R |  | 1 |
| 6. | Write down the formula to calculate the strain energy if the torsion value is given. |  |  | CO3 |  | A |  | 1 |
| 7. | Define unit load. |  |  | CO4 |  | R |  | 1 |
| 8. | Write the expression of strain energy for the unit load method. |  |  | CO4 |  | A |  | 1 |
| 9. | Define the ideal column. |  |  | CO5 |  | R |  | 1 |
| 10. | Identify the design condition according to the maximum principal strain theory. |  |  | CO6 |  | R |  | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |  |  |  |
| 11. | List the components of a truss. |  |  | CO1 |  | R |  | 3 |
| 12. | Sketch bending moment diagram for a simply supported beam with uniformly distributed load over the entire span. |  |  | CO2 |  | A |  | 3 |
| 13. | State Castigliano’s first theorem. |  |  | CO3 |  | R |  | 3 |
| 14. | Write the equation of simple bending. |  |  | CO4 |  | A |  | 3 |
| 15. | Define crippling load |  |  | CO5 |  | R |  | 3 |
| 16. | State the strain energy theory. |  |  | CO6 |  | R |  | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |  |  |  |
| 17. |  | Determine the forces in the members of a truss of 15 m span loaded as shown in Fig. below. | CO1 |  | A |  | 12 |  |
| 18. |  | Analyze the continuous beam as shown in Fig. by using Clapeyron’s three moment theorem and sketch shear force and bending moment diagrams. | CO2 |  | An |  | 12 |  |
| 19. |  | Determine the maximum deflection and slope for a cantilever beam with a uniformly distributed load over the entire span. | CO3 |  | A |  | 12 |  |
| 20. |  | The cross-sectional area of each member of the truss shown is A= 400 mm² and E= 200 GPa. Determine the horizontal and vertical displacements of joint C if a 4 kN force is applied to the truss at C. | CO4 |  | A |  | 12 |  |
| 21. |  | Produce the expression of crippling load when a column is fixed at its base and free at its upper end. | CO5 |  | A |  | 12 |  |
| 22. |  | Determine the moment and draw shear force and bending moment diagrams for the beam shown in Fig. below using the moment distribution method. | CO2 |  | A |  | 12 |  |
| 23. |  | A hollow cylindrical cast iron column is 4 m long with both ends fixed. Determine the minimum diameter of the column if it has to carry a safe load of 250 kN with a factor of safety of 5. Take the internal diameter as 0.8 times the external diameter. Take the internal diameter as 0.8 times the external diameter. Take σc= 550 N/m² and a= 1/1600 in Rankine’s formula. | CO5 |  | A |  | 12 |  |
| COMPULSORY QUESTION |  |  |  |  |  |  |  |  |
| 24. |  | At a section of a mild steel shaft, the maximum torque is 8437.5 Nm and the maximum bending moment is 5062.5 Nm. The diameter of the shaft is 90 mm and the stress at the elastic limit in simple tension for the material of the shaft is 220 N/mm². Determine whether the failure of the material will occur or not according to the maximum shear stress theory. If not, then find the factor of safety. | CO6 |  | A |  | 12 |  |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Determine the forces of each member in a truss. |
| CO2 | Analyze statically indeterminate beam under different support/ loading conditions. |
| CO3 | Find the deflection of an elastic structure based on strain energy of the structure. |
| CO4 | Analyze the indeterminate trusses using energy method. |
| CO5 | Compare the buckling of columns with different support conditions. |
| CO6 | Predict failure of the structures made of conventional metals. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | R | U | A | An | E | C | Total |
| CO1 | 5 | - | 12 | - | - | - | 17 |
| CO2 | 2 | - | 15 | 12 | - | - | 29 |
| CO3 | 4 | - | 13 | - | - | - | 17 |
| CO4 | 1 | - | 16 | - | - | - | 17 |
| CO5 | 4 | - | 24 | - | - | - | 28 |
| CO6 | 4 | - | 12 | - | - | - | 16 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 20AE2012 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # PROPULSION-I | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Define – Thrust specific fuel consumption. |  | CO1 | R | 1 |
| 2. | Illustrate the P-V diagram for Brayton cycle with reheat. |  | CO1 | R | 1 |
| 3. | Define-Thermal efficiency of a turbojet engine. |  | CO2 | R | 1 |
| 4. | State the significant advantages of turboprop engine aircraft. |  | CO2 | U | 1 |
| 5. | Define-Isolated system. |  | CO3 | R | 1 |
| 6. | Illustrate the exit velocity triangle of centrifugal compressor, if β2<90° |  | CO3 | U | 1 |
| 7. | Define – choking in axial flow compressor. |  | CO4 | R | 1 |
| 8. | Define the stages of axial flow compressor. |  | CO4 | U | 1 |
| 9. | What is meant by dilution zone in combustion chamber? |  | CO5 | U | 1 |
| 10. | Explain the needs of turbine blade cooling. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Differentiate – open cycle & closed cycle gas turbine engines. |  | CO1 | U | 3 |
| 12. | Recall the difference between pulse jet and ram jet engine. |  | CO2 | R | 3 |
| 13. | Discuss about pre-whirl in centrifugal compressor. |  | CO3 | U | 3 |
| 14. | Differentiate – Positive stalling and Negative stalling. |  | CO4 | U | 3 |
| 15. | Describe the purpose of flame holder with necessary diagram. |  | CO5 | R | 3 |
| 16. | Differentiate – radial flow & axial flow turbine. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Discuss about the factors affecting thrust. | CO1 | U | 6 |
|  | b. | Explain about Brayton cycle with necessary sketches. | CO1 | U | 6 |
| 18. |  | A flight speed of a turbojet is 900 kmph at 11000 m altitude. The density of air at that altitude is 00.17kg/m3. The Thrust for a plane is 6.8kN. The propulsive efficiency of the jet is 65%. Calculate SFC, ratio of mass flow rate of air and fuel, jet velocity. Assume the calorific value of fuel is 50 MJ/kg and overall efficiency is 22%. | CO2 | A | 12 |
| 19. | a. | Explain about velocity triangles of centrifugal compressor with necessary diagrams. | CO3 | U | 9 |
|  | b. | Compare the advantages and disadvantages of centrifugal compressor. | CO3 | U | 3 |
| 20. |  | An axial compressor stage has mean diameter 600mm and runs at 250rps. The actual temperature rise is 30C and the pressure ratio developed is 1.35. Initial temperature is 35C and temperature rise in the rotor is 20C. Mass flow rate is 50kg/s and the mechanical efficiency is 85%. Find out, (i) Power required to drive the compressor, (ii) Loading coefficient, (iii) Degree of reaction, (iv) Stage efficiency. | CO4 | A | 12 |
| 21. | a. | Explain the factors affecting combustion chamber design. | CO5 | U | 8 |
|  | b. | Discuss about flame stabilizers. | CO5 | U | 4 |
| 22. | a. | Illustrate the velocity triangles for a turbine machines with necessary equations. | CO6 | U | 8 |
|  | b. | Predict the limiting factors in gas turbine design. | CO6 | U | 4 |
| 23. | a. | Differentiate – Low by-pass & high by-pass turbo fan engine with necessary diagram. | CO2 | U | 8 |
|  | b. | Explain the thrust Vs altitude graph for turbo jet engine. | CO2 | U | 4 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Describe the types of combustion chambers with necessary diagrams. | CO5 | U | 7 |
|  | b. | Explain the flame tube cooling methods with sketches. | CO5 | U | 5 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Estimate the performance of Brayton cycle. |
| CO2 | Analyze the performance of various air breathing engines. |
| CO3 | Understand the working of sub-systems of jet engines. |
| CO4 | Assess the performance of compressor and turbine. |
| CO5 | Evaluate combustion chamber, cooling and afterburner performance. |
| CO6 | Understand the procedure for matching compressor and turbine. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | R | U | A | An | E | C | Total |
| CO1 | 2 | 15 |  |  |  |  | 17 |
| CO2 | 4 | 13 | 12 |  |  |  | 29 |
| CO3 | 1 | 16 |  |  |  |  | 17 |
| CO4 | 1 | 4 | 12 |  |  |  | 17 |
| CO5 | 3 | 25 |  |  |  |  | 28 |
| CO6 |  | 16 |  |  |  |  | 16 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 20AE2014 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # AIRPLANE PERFORMANCE | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | List the sources of aerodynamics force. |  | CO1 | U | 1 |
| 2. | Define Aerodynamic center. |  | CO1 | R | 1 |
| 3. | Define Specific fuel consumption. |  | CO2 | R | 1 |
| 4. | What are propeller coefficients? |  | CO2 | R | 1 |
| 5. | Write the conditions for unaccelerated, steady level flight. |  | CO3 | U | 1 |
| 6. | Define Drag divergence. |  | CO3 | R | 1 |
| 7. | Define Rate of climb. |  | CO4 | U | 1 |
| 8. | Define Hodograph. |  | CO4 | R | 1 |
| 9. | Define Load factor. |  | CO5 | U | 1 |
| 10. | List the uses of spoilers. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Explain Drag polar. |  | CO1 | U | 3 |
| 12. | Differentiate between fixed and variable pitch propeller. |  | CO2 | U | 3 |
| 13. | An airplane has a wing loading of 2400 N/m2, and its drag equation is CD = 0.016 + 0.055 CL2. Calculate its maximum lift / drag ratio and the minimum drag speed. |  | CO3 | A | 3 |
| 14. | Define absolute ceiling and service ceiling. |  | CO4 | U | 3 |
| 15. | Differentiate between steady turn and coordinated turn. |  | CO5 | U | 3 |
| 16. | Compare approach distance and flare distance. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Discuss in detail the various types of drag in an airplane and methods of minimizing the drag. | CO1 | U | 12 |
| 18. | a. | Derive the Froude’s efficiency of propulsive system from momentum theory of propulsion. | CO2 | U | 8 |
|  | b. | Explain the characteristics of different types of power plants. | CO2 | U | 4 |
| 19. | a. | Derive the equation of motion of a rigid flight vehicle with six degrees of freedom and simplify it for the steady level flight conditions. | CO3 | An | 8 |
|  | b. | Briefly discuss on power available and power required curves for both jet and propeller driven airplanes. | CO3 | An | 4 |
| 20. | a. | An aeroplane weighs 160 kN and has a wing area of 42 m2. At a flight speed of 100 m/s the engines give a thrust of 27 kN. If the aircraft drag equation is CD = 0.014 + 0.05 CL2. Find the angle and rate of climb at 100 m/s flight speed. Assuming the angle of climb to be small. | CO4 | A | 8 |
|  | b. | Show that for a propeller driven airplane the maximum rate of climb is given by Where =Propeller efficiency and P = Brake Horse power. | CO4 | A | 4 |
| 21. |  | An airplane is flying straight and level at a speed of 75 m/s at sea level. It weighs 250 kN and requires a thrust of 18 kN. At full throttle at an altitude of 4500 m (σ = 0.634) the engine produces a total of 5 MW thrust power. Calculate the min radius of the horizontal banked turn that the airplane can make at that altitude. Assume that the angle of incidence is same for both cases. Also estimate the time taken to turn through 180º. | CO5 | A | 12 |
| 22. | a. | Derive the Breguet Range equations for a jet propelled and propeller driven aircraft | CO4 | An | 8 |
|  | b. | For steep angles of glide, show that the rate of descent (sinking speed) of an airplane in powerless glide is given by . | CO4 | An | 4 |
| 23. | a. | An airplane weighing 250 kN has a wing area of 80 m2 and its drag equation is CD =0.016+0.04 CL2. Calculate the minimum thrust required for straight and level flight, and the corresponding true air speeds at sea level and at 10,000 m (√σ= 0.58). Calculate the minimum power required, and the corresponding true air speeds at the above altitudes. | CO3 | A | 8 |
|  | b. | Show that the velocity corresponding to minimum power condition is 76% of the velocity corresponding to minimum thrust required condition in steady and level flight. | CO3 | A | 4 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Estimate the total take-off and landing distances for an aircraft and describe various methods to reduce the take-off and landing distances. | CO6 | An | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the preliminary design of aircraft based on the performance. |
| CO2 | Differentiate performance characteristics of jet engine from propeller engine. |
| CO3 | Estimate the performance characteristics in level flight. |
| CO4 | Assess the climbing performance characteristics of aircraft. |
| CO5 | Estimate the turning performance characteristics of aircraft. |
| CO6 | Realize the ground effects on performance. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | R | U | A | An | E | C | Total |
| CO1 | 1 | 16 |  |  |  |  | 17 |
| CO2 | 2 | 15 |  |  |  |  | 17 |
| CO3 | 1 | 1 | 15 | 12 |  |  | 29 |
| CO4 | 1 | 4 | 12 | 12 |  |  | 29 |
| CO5 |  | 4 | 12 |  |  |  | 16 |
| CO6 |  | 4 |  | 12 |  |  | 16 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 20AE2016 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # INTRODUCTION TO AEROSPACE MATERIALS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | Mention the composition of “Gunmetal” and its applications. |  | CO5 | U | 1 |
| 2. | Define the term “toughness”. |  | CO2 | R | 1 |
| 3. | List out the various material properties which are required for the materials which are being used for aerospace applications. |  | CO5 | R | 1 |
| 4. | Express the importance of “Chrome-vanadium Steels” and mention its composition. |  | CO2 | U | 1 |
| 5. | State the temperature range of ultra-high-temperature materials. |  | CO3 | R | 1 |
| 6. | State the various testing to be done on materials. |  | CO4 | R | 1 |
| 7. | Differentiate between the “tension testing” and “fatigue testing”. |  | CO4 | U | 1 |
| 8. | Show a related graph of “Glass transition temperature” and infer it. |  | CO1 | U | 1 |
| 9. | Name few materials used in solid propellant and liquid propellant rocket motors. |  | CO6 | R | 1 |
| 10. | Differentiate between Graphene and Carbon. |  | CO3 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | Write about the Hydrostatic test for materials. |  | CO4 | A | 3 |
| 12. | Explain the applications of Nickel-chromium Steels. |  | CO1 | U | 3 |
| 13. | Show the classification of composite material. |  | CO3 | U | 3 |
| 14. | Indicate the various materials and its percentage of compositions used in fabrication of Boeing787. |  | CO2 | U | 3 |
| 15. | List out the reason for developing composite material. |  | CO5 | R | 3 |
| 16. | Give examples of materials used for storage of cryogenic propellants and pumps. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Illustrate the hardness test procedure for materials with suitable sketch. | CO2 | A | 6 |
|  | b. | Write the “structure of alloys” in detail. | CO1 | A | 6 |
| 18. |  | With suitable diagram explain the various phases in the Iron Carbon Equilibrium diagram. | CO5 | An | 12 |
| 19. |  | With help of a schematic diagram illustrate the arrangement and test procedure for conducting fatigue test on materials. | CO4 | A | 12 |
| 20. | a. | Draw the testing arrangement used for “Charpy” Impact test and explain the testing method in detail. | CO4 | An | 6 |
|  | b. | Differentiate between the torsion test and hydrostatic test on materials. | CO4 | An | 6 |
| 21. | a. | Appraise in detail, the effect of environmental resistance, erosion and wear on high temperature materials. | CO3 | An | 6 |
|  | b. | Explain the aerospace applications of high temperature materials with suitable examples. | CO2 | A | 6 |
| 22. | a. | Sketch the “Stir-casting” furnace which is used for development of aluminum casting and explain the process. | CO3 | A | 6 |
|  | b. | Explain the process of “Pre-preg” method used for composite preparation with help of a schematic diagram. | CO5 | A | 6 |
| 23. | a. | Explain extrusion process used for processing of materials. | CO6 | A | 6 |
|  | b. | Discuss the methods of “Heat Treatment” carried out on steel. | CO5 | A | 6 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Analyze the materials used for nose cone of launch vehicles and reentry vehicles. | CO6 | An | 6 |
|  | b. | Explain the features, structure, properties and applications of graphene material. | CO5 | A | 6 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL              M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Explain the influenced of microstructure on mechanical properties of metals and alloys |
| CO2 | Understand the material properties |
| CO3 | Classify different materials |
| CO4 | Identify the testing method of materials |
| CO5 | - Select the right material for particular applications. |
| CO6 | Develop new material combinations based on requirement. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | R | U | A | An | E | C | Total |
| CO1 | - | 4 | 6 | - | - | - | 10 |
| CO2 | 1 | 4 | 12 | - | - | - | 17 |
| CO3 | 1 | 4 | 6 | 6 | - | - | 17 |
| CO4 | 1 | 1 | 15 | 12 | - | - | 29 |
| CO5 | 4 | 1 | 18 | 12 | - | - | 35 |
| CO6 | 1 | 3 | 6 | 6 | - | - | 16 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 20AE2017 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # GAS DYNAMICS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Define Mach Number. |  | CO1 | R | 1 |
| 2. | Illustrate the importance of conical geometry in supersonic flight. |  | CO1 | U | 1 |
| 3. | Sketch a under expanded nozzle. |  | CO2 | A | 1 |
| 4. | State the working principle of Pitot static tube. |  | CO2 | R | 1 |
| 5. | State the use of Shock tube. |  | CO3 | R | 1 |
| 6. | Write the equations of motion of a flow across a moving normal shock. |  | CO3 | A | 1 |
| 7. | Sketch the shock diamond produced by a wedge in a supersonic wind tunnel test section. |  | CO4 | A | 1 |
| 8. | Predict the change in parameters for a flow with friction. |  | CO4 | U | 1 |
| 9. | List the types of flow visualization techniques. |  | CO5 | R | 1 |
| 10. | Classify wind tunnels based on run time. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Describe the effect of speed of sound on compressibility using Mach cone. |  | CO1 | R | 3 |
| 12. | Compare and contrast under expanded and over expanded nozzles. |  | CO2 | U | 3 |
| 13. | Write the expression for Rankine Hugoniot equation. |  | CO3 | A | 3 |
| 14. | State the expression for θ-β-M relation. |  | CO4 | A | 3 |
| 15. | Interpret the flow through constant area duct with heat addition. |  | CO5 | U | 3 |
| 16. | Categorize wind tunnels based on various factors. |  | CO6 | An | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No. 17 to 23, Q.No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | A pressure vessel has a volume of 12 m3 is used to store high-pressure air for operating a supersonic wind tunnel. If the air pressure and temperature inside the vessel are 20 atm and 300 K, respectively. Determine the mass and internal energy of air stored in the vessel. Also calculate the change in entropy if the temperature of air inside the vessel is increased to 550 K. | CO1 | An | 6 |
|  | b. | Articulate the expressions for entropy and isentropic relations. | CO1 | A | 6 |
| 18. | a. | Consider a rocket engine having combustion chamber pressure and temperature of 25 atm and 3571 K respectively. The molecular weight of chemically reacting gas is 16 and γ=1.22. Exit pressure is 117.4 atm. Area of the throat is 0.4 m2. Assume calorically perfect gas, calculate i) the exit mach number, ii) the exit velocity iii) area of the exit and iv) mass through the nozzle. | CO2 | An | 6 |
|  | b. | Evaluate the expression for exit to throat area ratio. | CO2 | An | 6 |
| 19. | a. | Consider a Normal shock moving at 520 m/s into still air (300 K, 1 atm). Compute the pressure, temperature and velocity of gas behind the shock, stagnation temperature before and after the shock. | CO3 | A | 7 |
|  | b. | Devise the equations of motion for a flow across a normal shock. | CO3 | An | 5 |
| 20. | a. | Consider a double wedge airfoil placed in a supersonic flow field at 00 AOA. The half angle of the wedge is 200, Free stream Mach number is 3 and static pressure and temperature are 1 atm and 283 K respectively. Take γ=1.4. Compute Mach number, static pressure and temperature across the entire flow field. | CO5 | A | 8 |
|  | b. | Summarize the expressions for flow across a centered expansion wave | CO5 | U | 4 |
| 21. | a. | The Mach number at the exit of a combustion chamber is 0.9. The ratio of stagnation temperatures at exit and entry is 3.74. If the pressure and temperature of the gas at exit are 2.5 bar and 1273 K respectively estimate the Mach number, pressure and temperature of gas at entry, the heat supplied per kg of the gas and the maximum heat that can be supplied. Take γ=1.3. | CO4 | U | 12 |
| 22. | a. | A gas (γ=1.3) at inlet pressure of 1 bar, inlet temperature of 400 K enters a 30cm diameter duct at a Mach number of 2. A normal shock occurs at a Mach number of 1.5 and the exit Mach number is 0.003 examine: the lengths of the ducts upstream and downstream of the shock, mass flow rate of the gas and change of entropy upstream of the shock, across the shock and downstream of the shock. | CO4 | R | 12 |
| 23. | a. | Explain in detail the different types of flow visualization techniques used in wind tunnels with neat sketches. | CO6 | U | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Interpret the expression relating flow parameters with length, diameter of the pipe and coefficient of friction of the pipe. Also determine the expression for calculating the limiting Mach number and limiting length of the pipe. | CO4 | U | 12 |

CO – COURSE OUTCOME	BL – BLOOM’S LEVEL                  M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the influence of compressibility to distinguish between the flow regimes. |
| CO2 | Apply compressibility corrections for flow in converging-diverging passages and instruments like Pitot static tube. |
| CO3 | Estimate the sudden changes in the flow field due to normal shocks. |
| CO4 | Estimate the influence of friction and heat transfer in the flow field. |
| CO5 | Understand oblique shocks and its effect on supersonic flow fields. |
| CO6 | Choose proper flow visualization techniques for any given situation. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | R | U | A | An | E | C | Total |
| CO1 | 4 | 1 | 6 | 6 |  |  | 17 |
| CO2 | 1 | 3 | 1 | 12 |  |  | 17 |
| CO3 | 1 |  | 11 | 5 |  |  | 17 |
| CO4 | 12 | 25 | 4 |  |  |  | 41 |
| CO5 | 1 | 7 | 8 |  |  |  | 16 |
| CO6 |  | 13 |  | 3 |  |  | 16 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 20AE2018 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # AIRCRAFT INSTRUMENTATION AND AVIONICS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Name the system used to measure the quantity or parameter in avionics. |  | CO1 | R | 1 |
| 2. | Indicate the term, which refers to the degree of closeness between a measured, and its true value. |  | CO1 | R | 1 |
| 3. | Convert 20.6 meter of water column into bar. |  | CO2 | U | 1 |
| 4. | Give an example of a gyroscopic instrument. |  | CO2 | U | 1 |
| 5. | Select a temperature indicating system employed in the aircraft. |  | CO3 | R | 1 |
| 6. | Indicate the system that is also referred to as boost gauge. |  | CO3 | U | 1 |
| 7. | Choose a system that depends on electronics for its operation although it contains electromechanical elements. |  | CO4 | U | 1 |
| 8. | Cite the parameter on which the maximum altitude an aircraft can fly depends on. |  | CO4 | U | 1 |
| 9. | Name the data bus that was developed at Wright Patterson Air Force base in 1970. |  | CO5 | R | 1 |
| 10. | Give an example of a cockpit system. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | List out the stages of a generalized measurement system. |  | CO1 | R | 3 |
| 12. | Classify the instruments employed in aircraft. |  | CO2 | U | 3 |
| 13. | State the functions of an accelerometer. |  | CO3 | R | 3 |
| 14. | Summarize the role of avionics in civil and military aircraft system. |  | CO4 | U | 3 |
| 15. | Group the commercial and military data buses in avionics. |  | CO5 | U | 3 |
| 16. | Enumerate the features of an alphanumeric display used in avionics. |  | CO6 | R | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | Describe the functions of an indicator, recorder and integrator with the help of a neat schematic. |  | CO1 | R | 12 |
| 18. | Discuss the construction and operation of a Pitot static system. Summarize its merits and demerits. |  | CO2 | U | 12 |
| 19. | Explain the salient features of an accelerometer highlighting its advantages and disadvantages. |  | CO3 | U | 12 |
| 20. | Describe any two pressure measuring instruments employed in avionics system. |  | CO3 | A | 12 |
| 21. | Sketch the cabin pressure control system and explain its construction. |  | CO4 | A | 12 |
| 22. | Construct the architecture of military standard (MIL STD) 1553B and summarize its salient features. |  | CO5 | A | 12 |
| 23. | Examine the trends in display technology in avionics. |  | CO6 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Write about synthetic and enhanced vision system with neat sketches. | CO6 | A | 6 |
|  | b. | Explain the working principle of multi function keyboard system. | CO6 | A | 6 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the basics of measurements and different parameters. |
| CO2 | Identify the fundamental cockpit instruments and their working principles. |
| CO3 | Differentiate various sensors and transducers used in aerospace vehicles. |
| CO4 | Comprehend the principles behind temperature, pressure, fuel flow and engine measurements. |
| CO5 | Analyze the functioning of military/civil aircraft data buses and communication process between them. |
| CO6 | Identify display technologies and their working principles. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | R | U | A | An | E | C | Total |
| CO1 | 17 | - | - | - | - | - | 17 |
| CO2 | - | 17 | - | - | - | - | 17 |
| CO3 | 4 | 13 | 12 | - | - | - | 29 |
| CO4 | - | 5 | 12 | - | - | - | 17 |
| CO5 | 1 | 3 | 12 | - | - | - | 16 |
| CO6 | 3 | 1 | 24 | - | - | - | 28 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 20AE2019 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # SPACE DYNAMICS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | State the Thrust equation of Rocket. |  | CO1 | R | 1 |
| 2. | Define Multi staging of rockets. |  | CO1 | R | 1 |
| 3. | State Kepler’s first law of planetary motion. |  | CO2 | R | 1 |
| 4. | Define Solar day. |  | CO2 | R | 1 |
| 5. | Sate the Kepler’s equation representing eccentric and mean anomaly. |  | CO3 | R | 1 |
| 6. | Define Newtons Law of gravitation. |  | CO3 | R | 1 |
| 7. | State the eccentricity values for circular, elliptical and parabolic orbits. |  | CO4 | R | 1 |
| 8. | Define Orbital Perturbation. |  | CO4 | R | 1 |
| 9. | Define single impulse maneuver. |  | CO5 | R | 1 |
| 10. | Define Interplanetary Trajectories. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | Explain the stability of Rockets. |  | CO1 | A | 3 |
| 12. | It takes Venus 225 days to orbit the sun. If the Earth –Sun distance is 1.5 x 1011 m, calculate the mean distance between Venus and Sun. |  | CO2 | An | 3 |
| 13. | Name the six orbital elements. |  | CO3 | R | 3 |
| 14. | Explain the effects on a satellite due to Earth shape. |  | CO4 | A | 3 |
| 15. | Describe different scenarios in which orbital maneuvers are executed in space missions. |  | CO5 | U | 3 |
| 16. | Explain the concept of planetary departure in space mechanics. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No. 17 to 23, Q.No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Describe and derive the classical Rocket equation to find the incremental velocity of a Rocket. | CO1 | U | 12 |
| 18. |  | A four stage rocket is used to put up a satellite of 40kg mass in Low Earth Orbit (LEO). The approximate values of mass of the propellant, structure mass and jet velocity for each stage are given below: Stage; I; II; III; IV. Propellant Mass(kg); 9000; 3500; 1700; 260. Structure Mass(kg); 1500; 550; 250; 40. VJ (m/s); 2200; 2400; 2500; 2750. Determine: i. Payload mass fraction of the total rocket. ii. Structural mass fraction of each stage iii. The ideal incremental velocity provided by each stage and total ΔV. iv. If the first stage fires for a period of 50sec and the rate of mass depletion can be assumed to be constant, calculate the acceleration of the rocket at takeoff. | CO1 | A | 12 |
| 19. |  | Describe the characteristics and distinctions among the planets, asteroids, comets, and meteoroids within our solar system and represent them with neat sketches. | CO2 | U | 12 |
| 20. |  | Calculate the orbital elements of a geocentric satellite whose inertial position and velocity vectors in a geocentric equatorial frame are: r = -6045I^- 3490J^ + 2500K^ (km) v = -3.457I^ + 6.618J^ + 2.533K^ (km/s) | CO3 | An | 12 |
| 21. | a. | A geocentric elliptical orbit has a perigee radius of 9600 km and an apogee radius of 21000 km. Calculate the time to fly from perigee to a true anomaly of 120o. | CO3 | An | 6 |
|  | b. | Explain Critical Inclination and Sun Synchronous orbit. | CO4 | A | 6 |
| 22. |  | Explain the different types of forces that cause orbital perturbations to satellites. | CO4 | An | 12 |
| 23. |  | A 2000kg spacecraft is in a 480 km by 800 km earth orbit. Calculate i. The ∆V required at perigee A to place the spacecraft in a 480km by 1600km transfer ellipse (Orbit 2) ii. The ∆V required for apogee kick at B, in the transfer orbit to establish a circular orbit of 16000 km altitude (orbit 3). iii. The total propellant required if specific impulse is 300s. | CO5 | An | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Explain the concept of an interplanetary Hohmann transfer for moving between different celestial bodies in the solar system. | CO6 | A | 8 |
|  | b. | Explain the rendezvous opportunities in interplanetary trajectories. | CO6 | An | 4 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL              M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Estimate performance and stability of rockets. |
| CO2 | Attain a general knowledge of laws governing orbital motion. |
| CO3 | Compute orbits of satellites. |
| CO4 | Study the effects of perturbations on orbital motion. |
| CO5 | Study orbital maneuvers useful for the study of inter-planetary trajectories. |
| CO6 | Generate preliminary design of inter-planetary trajectories |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | R | U | A | An | E | C | Total |
| CO1 | 2 | 12 | 15 | - | - | - | 29 |
| CO2 | 2 | 12 | - | 3 | - | - | 17 |
| CO3 | 5 | - | - | 18 | - | - | 23 |
| CO4 | 2 | - | 8 | 12 | - | - | 22 |
| CO5 | 1 | 3 | - | 12 | - | - | 16 |
| CO6 | 1 | 3 | 9 | 4 | - | - | 17 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 20AE2020 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # AIRCRAFT STRUCTURES-II | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | State whether the beam of square cross section undergo unsymmetrical bending. |  | CO1 | R | 1 |
| 2. | Write the angle between principal axes of a given cross section. |  | CO1 | A | 1 |
| 3. | Write the relationship between shear flow “q” and shear stress “τ” |  | CO2 | A | 1 |
| 4. | Locate the shear center for a thin walled beam of cross section as shown in Figure. |  | CO2 | R | 1 |
| 5. | State whether the shear center will lie outside of cross section of a thin walled closed section beam. |  | CO3 | R | 1 |
| 6. | To carry a given load, a monocoque structure is heavier than semi monocoque structure. Whether the above statement is true or false and justify your answer. |  | CO3 | R | 1 |
| 7. | Buckling coefficient for a plate with all the four edges simply supported is lesser than that of a plate with all the four edges clamped. Justify the statement. |  | CO4 | U | 1 |
| 8. | Write lowest value of buckling coefficient for a plate with all the four edges simply supported. |  | CO4 | A | 1 |
| 9. | State the reason for studying shear flow (q) in thin walled section beam subjected to torque instead of shear stress (τ). |  | CO5 | R | 1 |
| 10. | Write one advantage of countersunk rivet. |  | CO6 | A | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | List three cross sections of the beam which will never undergo unsymmetrical bending. |  | CO1 | R | 3 |
| 12. | Determine Ixx for a thin walled symmetrical I section with thickness = t and flange width = web height = h. |  | CO2 | A | 3 |
| 13. | Determine the shear flow for a thin walled beam of square cross section with middle side length 10 cm subjected to a torque of 100 Nm. |  | CO3 | A | 3 |
| 14. | List the two empirical methods of finding crippling stress of plates with multi-corner section subjected to compressive loads. |  | CO4 | R | 3 |
| 15. | Classify the following structure into determinate / Indeterminate with reason. - Single cell structure subjected to shear force. - Single cell structure subjected to torque. |  | CO5 | U | 3 |
| 16. | List the various types of joints used in connecting different structural components. |  | CO6 | R | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No. 17 to 23, Q.No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | # A 75 mm x 75 mm x 12 mm angle is used as a cantilever with the face AB horizontal, as shown in Fig. A vertical load of 3 kN is applied at the tip of the cantilever which is 1 m long. Determine the stress at A, B and C using the Eq, σz=MyIxy+MxIyyIxxIyy-Ixy2y-MxIxy+MyIxxIxxIyy-Ixy2x. | CO1 | A | 12 |
| 18. |  | Determine the shear flow and locate the shear center for the “C” section shear web beam as shown in the figure using the equation q= VxIxy-VyIyyIxxIyy-Ixy2Aiyi+VyIxy-VxIxxIxxIyy-Ixy2Aixi. Assume area of each boom = 4 cm2. | CO2 | A | 12 |
| 19. |  | Determine the shear flow, shear stress and twist per unit length of the two cell wing structure shown in Fig. Assume G = 25x105 N/cm2 and thickness = 0.1 cm | CO3 | A | 12 |
| 20. |  | # Determine the crippling stress for the formed section shown in Fig using angle method, if material is aluminium alloy 2024-T3. Fcy = 2.75x108 N/m2. Ec = 70x109 N/m2. FCSFCYE12=Ceb't0.75 Ce=0.316 two edge free, Ce=0.342 one edge free, Ce=0.366 (no edge free) | CO4 | A | 12 |
| 21. |  | A cantilever beam as shown in Fig. carries concentrated loads as shown. Calculate the distribution of stiffener loads and the shear flow distribution in the web panels assuming that the latter are effective only in shear. | CO5 | A | 12 |
| 22. |  | Two plates 10 mm thick are joined by a double riveted lap joint. The pitch of each row of rivets is 50 mm. the rivets are 20 mm diameter and the permissible stresses are: Shear stress in rivets = 70 MPa, Crushing stress of plates = 160 MPa and Tearing stress of plates = 100 MPa. Determine the maximum tensile force on the joint and efficiency of the joint. | CO6 | A | 12 |
| 23. |  | For the section, x and y axes are symmetrical axes and hence, these are also the principal axes. The force can be resolved into two components 800 cos 30° along the vertical axis and 800 sin 30° along the x axis. Determine the normal stress due to bending at points A, B, C and D using the equation σz=MyIxy+MxIyyIxxIyy-Ixy2y-MxIxy+MyIxxIxxIyy-Ixy2x | CO1 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | The Fig. Shows a single cell beam with four flanges, Determine the internal shear flow and the shear center using the equation, when the beam carries an external load of 2000 N as shown. q= VxIxy-VyIyyIxxIyy-Ixy2Aiyi+VyIxy-VxIxxIxxIyy-Ixy2Aixi Consider the thickness of the cell is uniform and is equal to 0.1 cm. | CO3 | A | 12 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Describe the stresses due to unsymmetrical bending of beams. |
| CO2 | Predict the shear flow and shear center in thin walled open section beams. |
| CO3 | Calculate the shear stress in thin walled closed section beams. |
| CO4 | Analyze the buckling characteristics of plates. |
| CO5 | Assess the load and stress distribution of wing and fuselage sections. |
| CO6 | Analyze the stresses in structural joints of aircraft components. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | R | U | A | An | E | C | Total |
| CO1 | 4 |  | 25 |  |  |  | 29 |
| CO2 | 1 |  | 16 |  |  |  | 17 |
| CO3 | 2 |  | 27 |  |  |  | 29 |
| CO4 | 3 | 1 | 13 |  |  |  | 17 |
| CO5 | 1 | 3 | 12 |  |  |  | 16 |
| CO6 | 3 |  | 13 |  |  |  | 16 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 20AE2022 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # PROPULSION-II | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | Define exhaust velocity. |  | CO1 | R | 1 |
| 2. | The mixture ratio of a hypergolic engine is 2.8 for a total propellant flow of 17 kg/s find the fuel flow rate? |  | CO1 | U | 1 |
| 3. | List the application of flush intake. |  | CO2 | R | 1 |
| 4. | Define expansion ratio. |  | CO2 | R | 1 |
| 5. | List any one chemical that can increase the burn rate in solid propellant rocket motor. |  | CO3 | R | 1 |
| 6. | Draw the schematic diagram of case bonded grain. |  | CO3 | R | 1 |
| 7. | Write down any two earth storable propellant. |  | CO4 | R | 1 |
| 8. | State the purpose of preburner in liquid rocket engine. |  | CO4 | R | 1 |
| 9. | State the disadvantage of integral ram rocket engine. |  | CO5 | R | 1 |
| 10. | State the purpose of electrostatic accelerator in ion thrusters. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | State any two advantages and two disadvantages of the electric propulsion. |  | CO1 | R | 3 |
| 12. | State the purpose of thrust reversal and list the various types of thrust reversal used in modern aircraft. |  | CO2 | R | 3 |
| 13. | Describe jet-avators with neat sketch. |  | CO3 | U | 3 |
| 14. | Differentiate between dump cooling and film cooling. |  | CO4 | U | 3 |
| 15. | State the advantages of hybrid rocket motor over solid rocket motor. |  | CO5 | R | 3 |
| 16. | Give the broad classification of electric propulsion. |  | CO6 | R | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No. 17 to 23, Q.No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Explain the various types of staging used in a rocket engine with neat sketch. | CO1 | A | 6 |
|  | b. | Design a nozzle for an ideal rocket that has to operate at 25 km altitude and give 5000 N thrust at a chamber pressure of 2.068 MPa and a chamber temperature of 2800 K. Assuming that k = 1.30 and R = 355.4 J/kg-K, determine the throat area, exit area, throat velocity, and exit temperature. (Take pressure at 25 km altitude as 2.5 kpa). | CO1 | An | 6 |
| 18. | a. | Explain under expanded, over expanded and optimum flow in C-D nozzle. | CO2 | A | 8 |
|  | b. | State any four differences between ideal nozzle and real nozzle. | CO2 | R | 4 |
| 19. | a. | Write short note on the solid propellant fuel composition and processing. - Binders. - Burning-Rate Modifiers. - Plasticizers. - Curing Agents or Crosslinkers. | CO3 | A | 3 3 3 3 |
| 20. | a. | Explain the factors influencing the burn rate in solid rocket motor with appropriate sketch. (any three). | CO3 | A | 12 |
| 21. | a. | Explain how thrust can be varied in liquid rocket engine with neat sketch. | CO4 | A | 12 |
| 22. | a. | State the various types of thrust vectoring in rocket engine. | CO5 | R | 6 |
|  |  | Explain any four thrust vectoring devices in rocket engine. | CO5 | A | 6 |
| 23. | a. | Compare the advantages of Gel propellant to Solid and Liquid propellant based on the following points - Safety aspects - Performance aspects - Storage aspects - Burning process | CO6 | U | 3 3 3 3 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Explain the working of solar sail with neat sketch. | CO6 | A | 12 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand and evaluate the performance of chemical propellant. |
| CO2 | Select and design suitable air inlets and nozzles. |
| CO3 | Select and design a suitable solid rocket motor. |
| CO4 | Select and design a suitable liquid rocket engine. |
| CO5 | Understand the working of sub-systems of the propulsion system. |
| CO6 | Assess the performance of electric propulsion systems. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | 4 | 1 | 6 | 6 | - | - | 17 |
| CO2 | 9 | - | 8 | - | - | - | 17 |
| CO3 | 2 | 3 | 24 | - | - | - | 29 |
| CO4 | 2 | 3 | 12 | - | - | - | 17 |
| CO5 | 10 |  | 6 | - | - | - | 16 |
| CO6 | 4 | 12 | 12 | - | - | - | 28 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 20AE2023 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # COMPUTATIONAL FLUID DYNAMICS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | Write the continuity equation for an inviscid incompressible flow through a control volume fixed in space. |  | CO1 | A | 1 |
| 2. | Interpret the approach mentioned in Qn. 1. |  | CO1 | U | 1 |
| 3. | Name and sketch some of the 2D elements. |  | CO4 | A | 1 |
| 4. | How can we identify the maximum permissible time step size for solving using explicit method? |  | CO4 | R | 1 |
| 5. | Examine the order of accuracy of the upwind differencing scheme |  | CO2 | A | 1 |
| 6. | Define peclet number. |  | CO2 | R | 1 |
| 7. | Classify the inlets based on input parameters. |  | CO5 | U | 1 |
| 8. | Illustrate staggered grid with a neat sketch. |  | CO3 | U | 1 |
| 9. | Label the parameter that determines the turbulance of a flow. |  | CO6 | R | 1 |
| 10. | Categorize the turbulence models based on the number of equations. |  | CO6 | An | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | Represent the applications of Elliptic equation. |  | CO1 | U | 3 |
| 12. | Sketch the Voronoi polygon and state its significance. |  | CO4 | A | 3 |
| 13. | Justify the conservative and bounded nature of central differencing scheme. |  | CO2 | E | 3 |
| 14. | Articulate the importance of relaxation factor in achieving convergence. |  | CO3 | A | 3 |
| 15. | Distinguish between the translational and rotational periodic boundary condition. |  | CO5 | U | 3 |
| 16. | Define Prandtls mixing length. |  | CO6 | R | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No. 17 to 23, Q.No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Derive the momentum equation for a infinitesimal fluid element fixed in space inside steady compressible flow field. | CO1 | U | 10 |
|  | b | List out the types of PDEs. | CO1 | R | 2 |
| 18. | a. | Write short notes on Delaunay Triangulation. | CO4 | U | 6 |
|  | b. | Derive the FVM equation for diffusion problems. | CO4 | U | 6 |
| 19. | a. | A thin plate is initially at a uniform temperature of 200°C. At a certain time t = 0 the temperature of the east side of the plate is suddenly reduced to 0°C. The other surface is insulated. Use the explicit finite volume method in con-junction with a suitable time step size to calculate the transient temperature distribution of the slab and compare it with the analytical solution at time, t = 4 s, The data are: plate thickness, L = 2 cm, thermal conductivity k = 10 W/m.K and ρc = 10 × 106 J/m3.K | CO2 | An | 12 |
| 20. | a. | Explain briefly about TDMA. | CO3 | U | 6 |
|  | b. | Express the steps involved in employing the Gauss seidal method in detail. | CO3 | U | 6 |
| 21. | a. | Illustrate in detail about the different types of physical boundary conditions. | CO5 | U | 8 |
|  | b. | Draw the flowchart for SIMPLER algorithm and explain how it differs from SIMPLE algorithm. | CO5 | U | 4 |
| 22. | a. | Describe in detail about the k-Ꜫ turbulence model and its limitations. | CO6 | R | 10 |
|  | b. | Express in short about the laminar-turbulent transition. | CO6 | U | 2 |
| 23. | a. | Compare the properties of discretized equations. | CO2 | U | 7 |
|  | b. | Assess the properties of central differencing scheme. | CO2 | E | 5 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | A property φ is transported by means of convection and diffusion through the one-dimensional domain sketched in Figure below. The governing equation is given below; the boundary conditions are φ0 = 1 at x = 0 and φL = 0 at x = L. Using five equally spaced cells and the central differencing scheme for convection and diffusion, calculate the distribution of φ as a function of x for u = 0.1 m/s. | CO3 | An | 12 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the governing equations for fluid flow and its classification. |
| CO2 | Choose proper turbulent models for given flow situations. |
| CO3 | Apply proper solution methodologies for PDE. |
| CO4 | Arrive at proper domain for the numerical simulation for given flow situations. |
| CO5 | Define the boundary conditions and generate grids. |
| CO6 | Solve real life fluid dynamic problems. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | 2 | 14 | 1 |  |  |  | 17 |
| CO2 | 1 | 7 | 1 | 12 | 8 |  | 29 |
| CO3 |  | 13 | 3 | 12 |  |  | 28 |
| CO4 | 1 | 12 | 4 |  |  |  | 17 |
| CO5 |  | 16 |  |  |  |  | 16 |
| CO6 | 14 | 3 |  |  |  |  | 17 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 20AE2025 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # AIRCRAFT STABILITY AND CONTROL | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Define degrees of freedom. |  | CO1 | R | 1 |
| 2. | Illustrate the importance of neutral point in static longitudinal stability. |  | CO1 | U | 1 |
| 3. | Write the expression for the contribution of elevator hinge moment towards stability. |  | CO2 | A | 1 |
| 4. | Sketch trim tabs. |  | CO2 | A | 1 |
| 5. | Interpret the expression for contribution of wing towards static directional stability. |  | CO3 | U | 1 |
| 6. | Illustrate dorsal fins with a neat sketch. |  | CO3 | U | 1 |
| 7. | State the significance of aerodynamic balancing. |  | CO4 | R | 1 |
| 8. | Describe the effects of coupling of static lateral and directional stability. |  | CO4 | R | 1 |
| 9. | Compare and contrast the static and dynamic stability. |  | CO5 | U | 1 |
| 10. | Give some examples of dynamic instabilities. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Name the 6 degrees of freedom of an airplane. |  | CO1 | R | 3 |
| 12. | Represent stick force gradients with a neat sketch. |  | CO2 | U | 3 |
| 13. | Explain in short the principle of Rudder lock. |  | CO3 | U | 3 |
| 14. | Briefly describe the effects of propeller in static lateral stability. |  | CO4 | R | 3 |
| 15. | Compare and contrast stick fixed and stick free stability. |  | CO5 | U | 3 |
| 16. | Write the expression for Rouths discriminant. |  | CO6 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No. 17 to 23, Q.No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Articulate the expression for wing and horizontal tail towards static longitudinal stability. | CO1 | A | 8 |
|  | b. | Write short notes on elevator requirements. | CO1 | A | 4 |
| 18. | a. | Deduce the steps involved in estimation of hinge moment parameters. | CO2 | An | 7 |
|  | b. | Estimate the factors influencing the aft most position of Centre of Gravity for stable configuration. | CO2 | U | 5 |
| 19. | a. | Establish the expression for fuselage and propeller effects on static directional stability. | CO3 | A | 12 |
| 20. | a. | Associate yawing moment with its conventions. | CO4 | U | 6 |
|  | b. | Enumerate the aileron control force requirements for lateral control. | CO4 | R | 6 |
| 21. | a. | Interpret the equations of motion for stick fixed condition of dynamic longitudinal stability. | CO5 | U | 12 |
| 22. | a. | Devise the steps involved in determining the aileron control reversal speed. | CO5 | An | 9 |
|  | b. | Illustrate Dutch roll with a neat sketch. | CO5 | U | 3 |
| 23. | a. | Classify the effects of coupling of static directional and lateral motion in dynamic stability. | CO5 | U | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Write short notes on the following: (i) Spiral Divergence. (ii) Directional Divergence. (iii) Aileron control effectiveness. (iv) Autorotation. | CO6 | A | 12 |

CO – COURSE OUTCOME	BL – BLOOM’S LEVEL M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the static stability behavior of an aircraft |
| CO2 | Analyze the effects of Elevator on static longitudinal stability. |
| CO3 | Assess the motion of aircraft and related modes of directional stability. |
| CO4 | Estimate the static lateral stability of aircraft. |
| CO5 | Understand the dynamic longitudinal stability of aircraft. |
| CO6 | Perform the dynamic analysis to determine stability of aircraft. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | R | U | A | An | E | C | Total |
| CO1 | 4 | 1 | 12 |  |  |  | 17 |
| CO2 |  | 8 | 9 |  |  |  | 17 |
| CO3 |  | 5 | 12 |  |  |  | 17 |
| CO4 | 11 | 6 |  |  |  |  | 17 |
| CO5 |  | 21 | 9 |  |  |  | 40 |
| CO6 |  | 1 | 15 |  |  |  | 16 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 20AE2027 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # FINITE ELEMENT ANALYSIS | # Max. Marks | # 100 |

| Q. No. | Questions |  |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |  |
| 1. | Define weighting function. |  |  | CO1 | R | 1 |
| 2. | Locate nodes in a structure. |  |  | CO1 | R | 1 |
| 3. | Define the discretization of a structure. |  |  | CO2 | R | 1 |
| 4. | Define the local coordinate system. |  |  | CO2 | R | 1 |
| 5. | List the steps involved in preprocessing stage. |  |  | CO3 | R | 1 |
| 6. | Write the expression of l, m and le for a truss. |  |  | CO3 | A | 1 |
| 7. | Identify the degrees of freedom of the beam. |  |  | CO4 | R | 1 |
| 8. | Define LST element. |  |  | CO4 | R | 1 |
| 9. | Define plane strain. |  |  | CO5 | R | 1 |
| 10. | Write the expression of stiffness matrix when the element is subjected to heat conduction and convection on its right end. |  |  | CO6 | A | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |  |
| 11. | State the principle of Galerkin’s method. |  |  | CO1 | R | 3 |
| 12. | Explain the characteristics of shape function. |  |  | CO2 | U | 3 |
| 13. | List the assumptions made in finding the forces in a truss. |  |  | CO3 | R | 3 |
| 14. | Write the equation of the stiffness matrix for a 1D beam element. |  |  | CO4 | A | 3 |
| 15. | Explain the quadratic strain triangular (QST) element. |  |  | CO5 | U | 3 |
| 16. | Distinguish between streamline and path line. |  |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |  |
| 17. |  | The following differential equation is available for a physical phenomenon. d2yd2x+50=0, 0≤x≥10, Trial function, y= a1x(10-x) Boundary conditions are y (0) =0 and y (10) =0. Determine the value of the parameter a1 by the following methods: - Point collocation - Subdomain collocation - Least squares - Galerkin | CO1 |  | A | 3 3 3 3 |
| 18. |  | A two-noded truss element is considered and its nodal displacements are u1= 3 mm at x1=20 mm and u2= -5 mm at x2= 36 mm. Calculate the following: - Shape functions N1 and N2 - Displacement u at x=24 mm. | CO2 |  | A | 6 6 |
| 19. |  | For the two bar truss shown in fig below, determine the displacements of node 1 and the stress in element 1-3. Take Young’s modulus E= 70 GPa, Area A= 200 mm². | CO3 |  | A | 12 |
| 20. |  | A beam, fixed at one end and supported by a roller at the other end, has a 20000 N concentrated load applied at the center of the span. Calculate the deflection under the load and construct the shear force and bending moment diagrams for the beam. | CO4 |  | A | 12 |
| 21. |  | Determine the stiffness matrix for the constant strain triangular (CST) element shown in Fig. The coordinates are given in units of millimeters. Assume plane stress conditions. Take E= 210 GPa, ʋ=0.25 and t=10 mm. | CO5 |  | A | 12 |
| 22. |  | Produce the shape function equations for fluid mechanics in the two- dimensional element. | CO6 |  | A | 12 |
| 23. |  | The nodal coordinates for an axisymmetric triangular element are given in Fig. Determine the strain displacement matrix. | CO5 |  | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |  |
| 24. |  | An aluminum alloy fin of 7 mm thick and 50 mm long protrudes from a wall, which is maintained at 120ºC. The ambient air temperature is 22ºC. The heat transfer coefficient and thermal conductivity of the fin material are 140 W/m²K and 55 W/mK respectively. Determine the temperature distribution of the fin. | CO6 |  | A | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the approximate methods applied to structural problems. |
| CO2 | Understand the discretization of bar elements. |
| CO3 | Develop mathematical models for truss problems. |
| CO4 | Derive the finite element equations for beam elements. |
| CO5 | Assemble finite element equation for 2D plane elements. |
| CO6 | Solve filed problems for finding the unknowns in heat and fluid flow problems. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | R | U | A | An | E | C | Total |
| CO1 | 5 | - | 12 | - | - | - | 17 |
| CO2 | 2 | 3 | 12 | - | - | - | 17 |
| CO3 | 4 | - | 13 | - | - | - | 17 |
| CO4 | 2 | - | 15 | - | - | - | 17 |
| CO5 | 1 | 3 | 24 | - | - | - | 28 |
| CO6 | - | 1 | 27 | - | - | - | 28 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 20AE2030 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # TECHNICAL APTITUDE | # Max. Marks | # 100 |

- In Gas Turbines, Turbine is used to convert _________

a. Kinetic energy of hot gases to Rotational energy

b. Heat energy of the gases to Rotational energy

c. Potential energy of the gases to Rotational energy

d. All of the mentioned

- In a radial flow turbine, the centrifugal forces act in ________on the blades.

a. Tangential direction		b. Flow direction

c. Radial direction			d. None of the mentioned

- Based on type of flow arrangements gas turbines are classified into ____

a. Axial or radial flow gas turbine 	b. Closed cycle gas turbine

c. Open cycle gas turbine		d. Impulse-reaction gas turbine

- ______ completes a stage in turbine expansion.

a. Row of moving blades followed by a row of another moving blade

b. One cycle passing through the engine

c. One cycle passing through turbine

d. Stationary blades row followed by moving blades row

- The blades of the gas turbine rotor are made of ___________

a. High alloy steel	b. High nickel alloy	c. Carbon steel		d. Stainless steel

- In impulse stage the rotor blades cause energy transfer and there is_____

a. No energy transformation		b. Less energy transformation

c. No heat transfer 			d. More energy transformation

- Quality of air required for air cooling of turbine blade is ______

a. 2-5 %		b. 1-3 %		c. 5-7%			d. Above 7%

- A ___________ drawn from the head of turbine blade to tail is called as chord.

a. Straight line		b. Multiline		c. Linear axis		d. Curved axis

- The purpose of v-gutter in a afterburner is to

a. Increase the flame intensity			b. Hold the flame in the combustion chamber

c. Increase the temperature			d. Reduce the pressure

- The mixture ratio of a hypergolic engine is 2.8 for a total propellant flow of 17 kg/s find the fuel flow rate

a. 6.07 kg/s		b. 7.91 kg/s		c. 10 kg/s		d. 4.28 kg/s

- A rocket engine produces a total thrust of 25 kN with a chamber pressure of 6 MPa and thrust coefficient of 1.24 determine the nozzle throat area

a. 5.360 x10-3 m2	b. 4.360 x10-3 m2	c. 2.360 x10-3 m2	d. 3.360 x10-3 m2

- Determine the theoretical exhaust velocity if the specific impulse is 288 s

a. 2825.28 m/s		b. 3825.28 m/s		c. 1825.28 m/s		d. 4825.28 m/s

- 13. Determine the thrust produced if the mass flow rate is  32 kg/s and specific impulse is 288 s

a. 100408.96 N		b. 90408.96 N		c. 80408.96 N		d. 70408.96 N

- The following data are given for a certain rocket unit, thrust 7000 N, Propellant consumption 3.5 kg/s determine the specific impulse in s

a. 203.87 s		b. 203.87 s		c. 203.87 s		d. 203.87 s

- In PPT plasma is created by ablating _____from discharge of capacitor across electrodes

a. Plastic		b. Wax			c. Teflon		d. Aluminium

- PPTs use solid Teflon propellant to deliver specific impulses in the range of  ____

a. 900 - 1,200 s		b. 1900 - 2,200 s	c. 500 - 900 s		d. 5000 - 9000 s

- In hybrid rocket motor with a constant oxidizer flow-rate, the ratio of fuel production rate to oxidizer flow rate will changes due to___

a. Change in the mass flow rate			b. Change in temperature of the combustion products

c. Change due to the mission’s requirement	d. Change in fuel burning surface area

- Integral ram rocket find its applications in ____

a. Missile		b. Space launch		c. Satellite launch		d. All of the given

- Hydrazine decomposes into ammonia, nitrogen and hydrogen at about 800ºC, when it comes into contact with  ______

a. Platinum catalyst	b. Oxygen		c. Air		d. Aluminium catalyst

- Cryogenic propellants can be stored for long period of time only if the tanks are

a. Well insulated	b. Having baffles	c. Having vent valve		d. Made of steel

- A small percentage of the propellant, such as the hydrogen in a LO2/LH2 engine, is fed through passages in the thrust chamber wall for cooling and subsequently dumped overboard through openings at the rear end of the nozzle skirt, this is a type of

a. Regenerative cooling		b. Dump cooling	c. Film cooling		d. Radiative cooling

- The specific impulse of the gel propellant is ____ than cryogenic propellants

a. Equal		b. Greater 		c. Less		d. Greater or equal

- _______will reduce the burning rate of some composite propellants.

a. Kelzan		b. Iron oxide		c. Silica gel	d. Lithium fluoride

- _______ has been the favourite binder in recent years as it allows higher solid fraction and relatively good physical properties at a particular temperature limit.

a. HTPB		b. PVC		c. Iron oxide		d. Silica gel

- In an ideal rocket the gas velocity pressure, temperature and density are all _________ across any section normal to the axis.

a. Equal		b. Non-uniform		c. Uniform		d. None of the above

- For a linearly elastic, isotropic and homogeneous material, the number of elastic constants required to relate stress and strain is:

- Two 		b) Three 		c)  Four 		d) Six

- For an isotropic, homogeneous and linearly elastic material, which obeys Hooke's law, the number of independent elastic constant is:

a) 1 			b) 2 			c) 3 			d) 6

- In a simple tension test, Hooke's law is valid up to the [IAS-1998]

a) Elastic limit 				b) Limit of proportionality

c) Ultimate stress 				d) Breaking point

- The materials which show direction dependent properties are called

a)  Homogeneous materials 			b) Viscoelastic materials

c)  Isotropic materials 			d) Anisotropic materials

- In the case of an engineering material under unidirectional stress in the x-direction, the Poisson's ratio is equal to

- εyεx       		b) σyσx             	c)  εyσx       		d) σyεx

- Which one of the following is correct in respect of Poisson’s ratio (ν) limits for an isotropic elastic solid?

- - ≤ ν ≤         	b) 1/4 ≤ ν ≤1/3		c)  -1 ≤ ν  ≤1/2 	d)  -1/2  ≤ ν ≤1/2

- If a piece of material neither expands nor contracts in volume when subjected to stress, then the Poisson’s ratio must be

- Zero 		b)  0.25  		c)  0.33 		d)  0.5

- Young's modulus of elasticity and Poisson's ratio of a material are 1.25 × 105 MPa and 0.34 respectively. The modulus of rigidity of the material is:

a) 0.4025 ×105 MPa 				b) 0.4664 × 105 MPa

c) 0.8375 × 105 MPa 			d) 0.9469 × 105 MPa

- For a composite consisting of a bar enclosed inside a tube of another material when compressed under a load ‘P’ as a whole through rigid collars at the end of the bar. The equation of compatibility is given by (suffixes 1 and 2 refer to bar and tube respectively).

a) P1+P2=P 	b) P1A1E1= P2A2E2 	c) P1E1A1= P2E2A2 		d) P1+P2=Constant

- A 100 mm × 5 mm × 5 mm steel bar free to expand is heated from 15°C to 40°C. Then the bar is subjected to

a) Tensile stress 				b) Compressive stress

c) Tensile strain 				d) Compressive strain

- If a material expands freely due to heating, it will develop

a) Thermal stress				b) Tensile stress

c) Compressive stress 			d) No stress

- A copper rod 400 mm long is pulled in tension to a length of 401.2 mm by applying a tensile load of 330 MPa. If the deformation is entirely elastic, the Young’s modulus of copper is

a) 110 GPa 		b) 110 MPa 		c) 11 GPa 		d) 11 MPa

- The Young's modulus of elasticity of a material is 2.5 times its modulus of rigidity. The Poisson’s ratio for the material will be

a) 0.25 		b) 0.33			c) 0.50 		d) 0.75

- The moduli of elasticity and rigidity of a material are 200 GPa and 80 GPa, respectively. the value of the Poisson's ratio of the material is

a) 0·30 		b) 0·26 		c) 0·25 		d) 0·24

- The elastic constants Young’s modulus (E) and bulk modulus (K) are related as (ν is the Poisson’s ratio)

a) E = 2K (1 – 2 ν) 	b) E = 3K (1- 2 ν) 	c) E = 3K (1 + ν) 	d) E = 2K(1 + 2 ν)

- The ratio of Young's modulus of Elasticity E and Bulk modulus K for a material is given by

a) 2 (1 – 2 ν) 		b) 3 (1- 2 ν) 		c) 3 (1 - ν) 		d) 2(1 + 2 ν)

- The condition for Mohr’s circle to be tangent to the y-axis

a) σx ≠ 0, σy = 0, τxy = 0			b) σx = σy, τxy = 0

c) σx ≠ 0, σy ≠ 0, τxy ≠ 0			d) σx ≠ 0, σy ≠ 0, τxy = 0

- The condition for Mohr’s circle to be a circle having center at the intersection of the x and y axes

a) σx ≠ 0, σy = 0, τxy = 0			b) σx = σy, τxy = 0

c) σx ≠ 0, σy ≠ 0, τxy ≠ 0			d) σx = 0, σy = 0, τxy ≠ 0

- The condition for Mohr’s circle to degenerate into a point

a) σx ≠ 0, σy = 0, τxy = 0			b) σx = σy, τxy = 0

c) σx ≠ 0, σy ≠ 0, τxy ≠ 0			d) σx ≠ 0, σy ≠ 0, τxy = 0

- The maximum shear stress (τmax) acting on a plane of an element is obtained by (σ1 and σ2 are the principal stresses)

- (σ1 + σ2)/2				b)  (σ1 - σ2)/2

c)    2 (σ1 + σ2)				d)  2(σ1 - σ2)

- An element of a material subjected to a plane state of stress such that the maximum shear stress is equal to the maximum tensile stress, would correspond to

- b)

d)

- A solid circular shaft is subjected to a maximum shearing stress of 140 MPa. The magnitude of the maximum normal stress developed in the shaft is:

a) 140 MPa 		b) 80 MPa 		c) 70 MPa 		d) 60 MPa

- In a bi-axial stress state, the stresses in x and y directions are (σx = 200 MPa & σy =100 MPa. The maximum principal stress in MPa, is

a) 50 			b) 100 			c) 150 			d) 200

- The maximum principle stress for the stress state shown in the figure is

a) σ 			b) 2 σ 			c) 3 σ 			d) 1.5 σ

- In a Mohr's circle, the radius of the circle is taken as

a) σx-σy22-τxy2 				b) σx+σy22-τxy2

c) σx-σy22+τxy2 				d) σx+σy22+τxy2

- A two-dimensional fluid element rotates like a rigid body. At a point within the element, the pressure is 1 unit. Radius of the Mohr's circle, characterizing the state of stress at that point, is

a) 0.5 unit 		b) 0 unit 		c) 1 unit 		d) 2 units

- If two principal stresses at a point are 1000 MPa & -600 MPa, then the maximum shear stress is

a) 800 MPa		b) 500 MPa		c) 1600 MPa		d) 200 MPa

- Principal stresses at a point in plane stressed element are σx = σy = 500 MPa. Normal stress on the plane inclined at 45o to x-axis will be

a) 0 			b) 500 MPa		c) 707 MPa		 d) 1000 MPa

- A body is subjected to a pure tensile stress of 100 units. The maximum shear produced in the body at some oblique plane due to the above is

a) 100 units 		b) 75 units 		c) 50 units 		d) 0 unit

- In a strained material one of the principal stresses is twice the other. The maximum shear stress in the same case is τmax. Then, the value of the maximum principle stress is

a) τmax		b) 2τmax 	c) 4τmax 		d) 8τmax

- In a strained material, normal stresses on two mutually perpendicular planes are σx and σy (both alike) accompanied by a shear stress τxy. One of the principal stresses will be zero, only if

a) τxy= σx +σy2 	b) τxy= σx σy 	c) τxy= σx σy	d) τxy= σx2+σy2

- Normal stresses of equal magnitude p, but of opposite signs, act at a point of a strained material in perpendicular direction. The magnitude of the resultant normal stress on a plane inclined at 45° to the applied stresses is

a) 2p	 		b) p/2 			c) p/4 			d) Zero

- A plane element is subjected to the state of stress given by σx = τxy = 100 MPa and σy = 0. Maximum shear stress in the element is equal to

a) 503 MPa 	b) 100 MPa 		c) 505 MPa		d) 150MPa

- A structural member in which loads applied at right angles to the axis is called

a) Beam 		b) Bar		c) Column 		d) Shaft

- The ratio of the area under the bending moment diagram to the flexural rigidity between any two points along a beam gives the change in

a) Deflection 	b) Slope 	c) Shear force 		d) Bending moment

- Consider a simply supported two-dimensional beam. If the beam is converted into a fixed-fixed beam, then the degree of static indeterminacy will

a) Increase by 3  				b) Increase by 2

c) Increase by 1				d) Decreases by 2

- A cantilever 5 m long, carries a point load of 5 N at its free end and a uniformly distributed load of 2 N/meter run throughout its length, the maximum bending moment on the cantilever is

a) 100 N-meters 	b) 50 N-meters 	c) 25 N-meters 	d) 75 N-meters

- A cantilever 10 meters long carries a uniformly distributed load of 10 kN/meter run starting from free end up to the middle of its length. The BM at the fixed end of the cantilever is

a) 25 kNm 		b) 75 kNm		c) 50 kNm 		d) 100 kNm.

- The maximum shear force of a cantilever beam of length ‘L’ loaded with uniform distributed load ‘w’ throughout is

a) wL 		b) wL/2 		c) 2wL 		d) wL/4

- The maximum bending moment of a simply supported beam of length ‘L’ loaded with mid- point load ‘P’ is

a) PL 		b) PL/2 		c) PL/4 		d) PL/8

- At the point of contraflexure, the value of bending moment is

a) Zero 		b) Maximum 		c) Minimum 		d) Can’t be determined

- A spring mass damper system with a mass of 1 kg is found to have a damping ratio of 0.2 and a natural frequency of 5 rad/s. The damping of the system is given by

  - 2 Ns/m	 	b. 2 N/s		c. 0.2 kg/s 	 d. 0.2 N/s

- In a spring mass damper single degree of freedom system, the mass is 2 kg and the undamped natural frequency is 20 Hz. The critical damping constant of the system is

a. 160π Ns/m		b. 80π Ns/m 	   c. 1 Ns/m		d. 0 Ns/m

- A 1 kg mass attached to a spring elongates it by 16 mm. This mass is then pulled from its equilibrium position by 10 mm and released from rest. Assuming the acceleration due to gravity of 9.81 m/s2, the response of the mass in mm is given by

a. x=10 sin24.76t	b. x=10 cos24.76t	   c. x=sin16t	    d. x=10 cos16t

- For a critically damped single degree of freedom, spring mass damper with a damping constant of 4 Ns/m and spring constant of 16 N/m, the system mass m is

a. 0.5 kg		b. 0.25 kg			c. 2 kg		d. 4 kg

- During an under-damped oscillation of a single degree of freedom system, in the time-displacement plot the third peak is of magnitude 100 and the tenth peak is of magnitude 10. The damping ratio ζ is approximately

a. 0.052 		b. 0.023		c. 0.366		d. 0.159

- A single degree of freedom system is vibrating with initial (first cycle) amplitude of 5 cm. The viscous damping factor associated with the vibrating system is  2%. Vibration amplitude of the fifth cycle (in cm) is

a. 1.65		b. 4.41		c. 2.67		d. 3.02

- A 0.5 kg mass is suspended vertically from the point fixed on the earth by a spring having a stiffness of 5 N/mm. The static displacement (in mm) of the mass is ______.

a. 0.981		b. 3.02		c. 2.67		d. 4.41

- A linear mass-spring dashpot is over-damped. In free vibration, this system undergoes

a. non-oscillatory motion			b. random motion

c. oscillatory and periodic motion		d. oscillatory and non-periodic motion

- An aircraft landing gear can be idealized as a single degree of freedom spring-mass-damper system. The desirable damping characteristics of such a system is:

a. Under damped	b. Over damped	c. Critically damped		d. Undamped

- The angle between the wing's chord line and the oncoming free stream airflow is called as _____.

a. Angle of pitch	b. Angle of twist	 c. Angle of attack	d. Pitching moment

- Winglets reduce drag by _____.

a. Decreasing the weight of the aircraft		b. Decreasing the lift generated by the wings

c. Decreasing the vortices at the wingtips	d. Increasing the lift generated by the wings

- The full form of NACA is _____.

a. National Advisory Committee for Aeronautics	b. National Aeronautical Control Agency

c. National Association of Civil Aviation		d. National Aeronautics and Space Administration

- The lift coefficient is defined as _____.

a. The ratio of lift force to dynamic pressure		b. The ratio of lift force to wing area

c. The ratio of lift force to weight			d. The ratio of lift force to drag force

- The critical angle of attack is defined as_____.

a. The angle at which the aircraft takes off		b. The angle at which drag becomes zero

c. The angle at which lift becomes zero			d. The angle at which the aircraft stalls

- The type of flow which has a positive effect on lift is _____.

a. Detached flow					b. Attached flow

c. Both attached and detached flow			d. Neither attached nor detached flow

- When an aircraft with a symmetric aerofoil pitches up, the lift _____.

a. Increases	b. Remains the same	c. Becomes negative	d. Decreases

- Coefficient of moment is constant with angle of attack at _____ of the aerofoil.

a. Center of pressure           b. Quarter chord	 c. Aerodynamic center          d. Leading edge

- At aerodynamic center _____.

a. dcma.c.dα=0   	b. dcma.c.dα=Constant 	c. cma.c.=0 	d. cma.c.α=Constant

- If the planform area of the wing increases, the lift produced by the wing _____.

a. Remains constant		b. Increases quadratically

c. Increases parabolically	d. Increases linearly

- The primary cause of induced drag is _____.

a. Vortex formation	     b. Skin friction	c. Air resistance		d. Separated flow

- Induced drag _____ with the increasing angle of attack (within stall).

a. Remains constant	       b. Increases	c. Decreases	      d. Has no relation

- Form drag is caused by the _____.

a. Altitude of the aircraft	b. Weight of the aircraft

c. Shape of the aircraft		d. Skin friction

- Form drag _____ of the aircraft.

a. Remains constant with speed			b. Decreases with increasing speed

c. Is not affected by speed			d. Increases with increasing speed

- Induced drag _____ of the aircraft.

a. Remains constant with speed			b. Increases with increasing speed

c. Is not affected by speed			d. Decreases with increasing speed

- The value CD, using drag polar is given by _____.

a. CD=CD,0*KCL2   	b. CD=CD,0-KCL2	c. CD=CD,0+KCL2	d. CD=CD,0/KCL2

- The bypass ratio in a turbofan engine is _____.

a. The ratio of the mass of air bypassing the engine core to the mass of air passing through the engine core

b. The ratio of the mass of air passing through the engine core to the mass of air bypassing the engine core

c. The ratio of the thrust produced by the engine core to the thrust produced by the bypass air

d. The ratio of the thrust produced by the bypass air to the thrust produced by the engine core

- The efficiency of an aircraft engine _____ with increase in thrust.

a. Increases	    b. Decreases	           c. Remains constant    	     d. Has no relation

- _____ type of engine is more efficient in terms of fuel consumption.

a. Turboprop			b. Turbojet

c. Both turbojet and turboprop	d. Either turbojet or turboprop, depends on the specific requirements

- _____ type of engine is more suitable for high-altitude flights.

a. Turbojet			b. Both turbojet and turboprop

c. Turboprop			d. Either turbojet or turboprop, depends on the specific requirements

- The role of the turbine in a turbojet engine is to _____.

a. Ignite the fuel-air mixture		b. Compress the incoming air

c. Direct exhaust gases			d. Drive the compressor

- The component of a turbojet engine responsible for converting the energy from burning air-fuel mixture into thrust is _____.

a. Nozzle	b. Combustion chamber		c. Compressor		d. Turbine

- The part of the piston engine responsible for controlling the opening and closing of the valves is____.

a. Crankshaft	        b. Camshaft	   c. Connecting rod             d. Flywheel

- The function of the crankshaft in a piston engine is to _____.

a. Convert reciprocating motion into rotational motion	b. Control fuel intake

c. Compress air-fuel mixture					d. Control opening and closing of valves

- The primary function of a piston ring in a piston engine is to _____.

a. Seal the combustion chamber and prevent leakage of gases

b. Regulate the flow of air-fuel mixture

c. Reduce friction between the piston and the cylinder wall

d. Provide structural support to the piston

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 20AE2033 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # DESIGN AND ANALYSIS OF COMPOSITE STRUCTURES | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | Name the reinforcement phase in E-Glass Fiber/Epoxy composite. |  | CO1 | R | 1 |
| 2. | Define Poisson’s ratio. |  | CO1 | R | 1 |
| 3. | Define Plane strain condition. |  | CO2 | R | 1 |
| 4. | Define unidirectional lamina. |  | CO2 | R | 1 |
| 5. | Distinguish between local coordinate system and global coordinate system. |  | CO3 | U | 1 |
| 6. | Define angle lamina. |  | CO3 | R | 1 |
| 7. | Define voids in composites. |  | CO4 | R | 1 |
| 8. | Define mass fraction of composite materials. |  | CO4 | R | 1 |
| 9. | Define Laminate code. |  | CO5 | R | 1 |
| 10. | Name the different forms of fibers. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | Explain Polymer Matrix Composite. |  | CO1 | U | 3 |
| 12. | Explain plane stress condition and state the assumptions followed. |  | CO2 | A | 3 |
| 13. | Explain maximum stress failure theory for an angle lamina. |  | CO3 | U | 3 |
| 14. | Explain volume fraction of composite materials. |  | CO4 | U | 3 |
| 15. | Represent the composite layers for the laminate codes: i. [0/-45/90/60/0] ii. [0/-45/902/60/0] |  | CO5 | U | 3 |
| 16. | Explain plain weave fibers and chopped fibers. |  | CO6 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No. 17 to 23, Q.No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Explain the different types of composite materials and explain its applications in aircraft and aerospace industry. | CO1 | U | 6 |
|  | b. | Explain Hooke’s law for different types of materials. | CO1 | A | 6 |
| 18. |  | Determine the engineering elastic constants of lamina from the relationship between the compliance and stiffness matrices. | CO2 | A | 12 |
| 19. | . | For a graphite/epoxy unidirectional lamina, Determine the following - Compliance matrix - Minor Poisson’s ratio - Reduced stiffness matrix - Strains in 1-2 coordinate system if the applied stresses are σ1= 2MPa, σ2= -3MPa, τ12= 4MPa. The properties of unidirectional graphite/epoxy lamina are E1=181 GPa, E2= 10.3 GPa, ν12=0.28 and G12= 7.17 GPa. | CO2 | A | 12 |
| 20. |  | Develop Hooke's law for a two-dimensional unidirectional angle lamina and calculate the engineering elastic constants. | CO3 | A | 12 |
| 21. |  | Determine longitudinal Young’s modulus E1 and transverse Young’s modulus E2 and major Poisson’s ratio of a composite material using strength of materials approach. | CO4 | A | 12 |
| 22. |  | A glass/epoxy lamina consists of 70% fiber volume fraction. The density of glass fiber and epoxy are given as ρf = 2500kg/m3 and ρm = 1200kg/m3 Calculate: i. Density of lamina ii. Mass fraction of glass & epoxy iii. Volume of composite lamina if the mass of lamina is 4kg iv. Volume and mass of glass and epoxy when the mass of lamina is 4 kg. | CO4 | An | 12 |
| 23. |  | Determine the forces and moment resultants related to mid plane strains and curvatures of a laminate and derive ABD matrix by using Classical laminate theory. | CO5 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Explain in detail about the production process of different composite fibers used in aerospace industries. | CO6 | U | 12 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL              M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Describe the various types of composite materials. |
| CO2 | Understand the structural behavior of lamina. |
| CO3 | Compare the various failure theories of composite materials. |
| CO4 | Assess various properties of lamina. |
| CO5 | Analyze the stresses developed in a laminate. |
| CO6 | Describe the manufacturing techniques of fibers. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | 2 | 9 | 6 | - | - | - | 17 |
| CO2 | 2 | - | 27 | - | - | - | 29 |
| CO3 | 1 | 16 | - | - | - | - | 17 |
| CO4 | 2 | 3 | 12 | 12 | - | - | 29 |
| CO5 | 1 | 3 | 12 | - | - | - | 16 |
| CO6 | 1 | 12 | 3 | - | - | - | 16 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 20AE2035 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # STRUCTURAL VIBRATION | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO |  | BL | M |
| --- | --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |  |
| 1. | Write the relationship between - Angular frequency ‘ω’ and linear frequency ‘f’ b) Linear frequency ‘f’ and Time period ‘T’ |  | CO1 |  | A | 1 |
| 2. | A 0.5 kg mass is suspended vertically by a spring having a stiffness of 5 N/mm. Determine the static displacement (in mm) of the mass. |  | CO1 |  | A | 1 |
| 3. | In a simple harmonic oscillator, at the mean position state whether kinetic energy is maximum or minimum. |  | CO2 |  | R | 1 |
| 4. | Write the equation of motion for the simple pendulum with a spring attached as shown in Figure. |  | CO2 |  | A | 1 |
| 5. | Indicate the sum of natural frequencies of an elastic beam with cantilever boundary conditions. |  | CO3 |  | U | 1 |
| 6. | Write the orthogonality condition of a 2 DoFs and explain the terms. |  | CO4 |  | A | 1 |
| 7. | State the advantage of studying matrix iteration method in determining the mode shape and frequency of a given system. |  | CO4 |  | R | 1 |
| 8. | Define buffeting in aeroelasticity. |  | CO5 |  | R | 1 |
| 9. | State the function of a seismometer. |  | CO6 |  | R | 1 |
| 10. | State the condition on the ratio r=ω/ωn for measuring acceleration using an accelerometer. |  | CO6 |  | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |  |
| 11. | A spring mass damper system with a mass of 1 kg is found to have a damping ratio of 0.2 and a natural frequency of 5 rad/s. Determine the damping coefficient of the system. |  | CO1 |  | A | 3 |
| 12. | Consider the spring-mass system shown in the Figure below. This system has two degrees of freedom representing the motions of the two masses. Indicate the type of coordinate coupling of the system (static/dynamic). Justify the answer. |  | CO2 |  | U | 3 |
| 13. | State the assumption in vibration of a string in tension. |  | CO3 |  | R | 3 |
| 14. | Write the mass matrix [m] and flexibility matrix [δ] for the spring-mass system shown in the Figure. |  | CO4 |  | A | 3 |
| 15. | Explain aileron control reversal speed. |  | CO5 |  | U | 3 |
| 16. | The size of the accelerometer is small compared to that of the vibrometer. Justify the statement. |  | CO6 |  | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |  |
| 17. |  | Derive and explain the three cases of damped free vibration (critically damped, over damped and under damped) and state the reason for the gradual diminishing of the amplitude of under damped condition with the help of the derived equation. |  | CO1 | A | 12 |  |
| 18. |  | Determine the natural frequencies and the corresponding mode shape for the two degrees of freedom system shown in Figure. Derive the equation of motion from the condition of force equilibrium. Consider k1=k2=100 N/m, m1=4 kg and m2=2 kg. |  | CO2 | A | 12 |  |
| 19. |  | Derive the expression for the frequency of a simply supported string under vibration. |  | CO3 | A | 12 |  |
| 20. |  | Determine the fundamental natural frequency and the corresponding mode shapes for the system shown in Figure by matrix iteration method. Also, obtain higher mode shape and natural frequency by applying the orthogonality condition. Consider m = 2 kg and k = 100 N/m. |  | CO4 | A | 12 |  |
| 21. |  | Explain each aeroelastic phenomenon that occurs due to the combined effect of aerodynamic, elastic and inertia forces using the Collar’s triangle. |  | CO5 | U | 12 |  |
| 22. |  | Explain the working principle of a vibrometer, a vibration measuring instrument, by deriving the required equation and with the help of a neat sketch. |  | CO6 | U | 12 |  |
| 23. |  | Determine the natural frequencies and the corresponding mode shape for the two degrees of freedom system shown in Figure. Derive the equation of motion from the Lagrangian principle. Consider m = 5 kg and k = 200 N/m. |  | CO2 | A | 12 |  |
| COMPULSORY QUESTION |  |  |  |  |  |  |
| 24. | a. | Derive the natural frequency of a simple spring-mass system as shown in Figure. |  | CO1 | A | 6 |  |
|  | b. | Derive the natural frequency of a simple pendulum as shown in Figure. |  | CO1 | A | 6 |  |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Describe various types of vibration systems. |
| CO2 | Understand multi degree freedom systems. |
| CO3 | Calculate frequency of free vibration of simple structures. |
| CO4 | Compare the different methods of vibration analysis. |
| CO5 | Understand the vibration of various components in aircraft. |
| CO6 | Identify techniques used in vibration measurement. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | R | U | A | An | E | C | Total |
| CO1 | - | - | 29 | - | - | - | 29 |
| CO2 | 1 | 3 | 25 | - | - | - | 29 |
| CO3 | 3 | 1 | 12 | - | - | - | 16 |
| CO4 | 1 | 0 | 16 | - | - | - | 17 |
| CO5 | 1 | 15 | - | - | - | - | 16 |
| CO6 | 2 | 15 | - | - | - | - | 17 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 20AE2037 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # CRYOGENIC PROPULSION | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Name the best material for handling cryogenic fluids. |  | CO1 | R | 1 |
| 2. | Observe the properties of super fluid helium. |  | CO1 | U | 1 |
| 3. | Identify the boiling point of Liquid air in Kelvin. |  | CO2 | U | 1 |
| 4. | Define J-T inversion curve. |  | CO2 | R | 1 |
| 5. | Write the equation of energy added isothermally in an ideal refrigeration system. |  | CO3 | A | 1 |
| 6. | State the type of refrigeration used for temperatures below 4 K. |  | CO3 | R | 1 |
| 7. | State the purpose of outer vessel in the storage vessel of cryogens. |  | CO4 | R | 1 |
| 8. | Write the minimum thickness of the inner shell for a cylindrical vessel. |  | CO4 | A | 1 |
| 9. | List the basic insulation types. |  | CO5 | R | 1 |
| 10. | Explain the need of cryogenics in liquid rocket engine. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Explain the importance of ductility in material selection for cryogenic applications. |  | CO1 | U | 3 |
| 12. | Differentiate – J-T expansion and adiabatic expansion. |  | CO2 | U | 3 |
| 13. | List the advantages of cryogen-free dilution refrigerators. |  | CO3 | R | 3 |
| 14. | Explain the types of cryogenic valves with necessary points. |  | CO4 | U | 3 |
| 15. | State and explain apparent thermal conductivity in cryogenic insulation. |  | CO5 | R | 3 |
| 16. | Discuss the applications of liquid helium as a cryogen. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | State the significance of thermal expansion between different materials. | CO1 | R | 6 |
|  | b. | Discuss the historical background of cryogenics with a timeline. | CO1 | U | 6 |
| 18. |  | Explain thermodynamic ideal system with a neat sketch also derive the equation for work required per unit mass of gas liquefied | CO2 | U | 12 |
| 19. | a. | Explain Vuilleumier refrigerator with necessary equations and sketches. | CO3 | U | 8 |
|  | b. | Differentiate – Solvay refrigerator and Gifford – McMahon refrigerator. | CO3 | U | 4 |
| 20. |  | Discuss the cryogenic container piping category with necessary sketch and equations. | CO4 | U | 12 |
| 21. | a. | Explain the requirements of insulation for cryogenic propellant tank. | CO5 | A | 4 |
|  | b. | Explain in detail the gas filled powders and fibrous insulation method used for insulating cryogenic containers. | CO5 | U | 8 |
| 22. |  | Describe the Precooled Linde Hampson system with necessary sketch and equations. | CO2 | R | 12 |
| 23. |  | Explain magnetic refrigeration system with neat sketch. | CO3 | U | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Illustrate the working principle of liquid oxygen –liquid hydrogen rocket engine. | CO6 | U | 8 |
|  | b. | State the advantage of cryogenic engine over semi cryogenic rocket engine. | CO6 | R | 4 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Remember the thermal, physical and flow properties of cryogenic fluids. |
| CO2 | Understand the liquefaction systems to produce cryogenic fluids. |
| CO3 | Know various methods of cryogenic refrigeration systems. |
| CO4 | Know the various cryogenic fluid storage and transfer lines. |
| CO5 | Understand various insulations for cryogenic propellant tanks. |
| CO6 | Know the various applications of cryogenics in propulsion systems. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | R | U | A | An | E | C | Total |
| CO1 | 7 | 10 |  |  |  |  | 17 |
| CO2 | 13 | 16 |  |  |  |  | 29 |
| CO3 | 4 | 24 | 1 |  |  |  | 29 |
| CO4 | 1 | 15 | 1 |  |  |  | 17 |
| CO5 | 4 | 8 | 4 |  |  |  | 16 |
| CO6 | 4 | 12 |  |  |  |  | 16 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 20AE2038 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # ROCKETS AND MISSILES | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | Differentiate between rockets and missiles in one word. |  | CO1 | U | 1 |
| 2. | Extend the term “SLBM” in rocket classification. |  | CO1 | U | 1 |
| 3. | Classify the airframe components of rockets and missiles. |  | CO2 | U | 1 |
| 4. | Define – slender body in aerodynamics. |  | CO2 | R | 1 |
| 5. | Describe an induced drag in rocket. |  | CO3 | U | 1 |
| 6. | Predict the effects of increasing the aspect ratio of a rocket. |  | CO3 | U | 1 |
| 7. | Write the alternate name of ideal rocket equation |  | CO4 | A | 1 |
| 8. | Define – Newton's law of universal gravitation. |  | CO4 | R | 1 |
| 9. | Describe the necessary of rocket staging. |  | CO5 | U | 1 |
| 10. | Write the drawback of tailless rocket control. |  | CO6 | A | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | Describe an ASLV in rocket classification. |  | CO1 | U | 3 |
| 12. | List other forebody shapes of rockets with necessary points. |  | CO2 | R | 3 |
| 13. | Describe an upwash and downwash in rocket with a neat sketch. |  | CO3 | U | 3 |
| 14. | Describe inertial frame and vehicle reference frame of a rocket motion with graph. |  | CO4 | A | 3 |
| 15. | Define – Ullage rocket motor with its functionality. |  | CO5 | R | 3 |
| 16. | Describe the wing control of a rocket with neat sketch. |  | CO6 | R | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No. 17 to 23, Q.No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Explain in detail about the GSLV rocket. | CO1 | U | 6 |
|  | b. | Discuss the basic aerodynamic characteristics of missiles. | CO1 | U | 6 |
| 18. | a. | Infer the equations for forces acting on a missile while passing through the atmosphere. | CO2 | U | 8 |
|  | b. | Explain briefly the airframe components of rockets and missiles. | CO2 | U | 4 |
| 19. | a. | Deduce the equation for lateral aerodynamic damping moment of a rocket. | CO3 | An | 12 |
| 20. | a. | Deduce the equations for two dimensional rocket motion. | CO4 | An | 9 |
|  | b. | Discuss the rocket dispersion with a neat sketch. | CO4 | U | 3 |
| 21. | a. | A two-stage exploration vehicle is launched from a high-orbit satellite into a gravity-free vacuum trajectory. The following data are given: Flight velocity increment in gravity-free vacuum = 4500 m/sec Specific impulse (each stage) = 290 sec Initial takeoff launch vehicle mass = 4400 kg Propellant mass fraction, ζi (each stage) = 0.88 Assume that the same propellant is used for both stages and that there is no “stage separation delay.” Determine the payload for the following cases: 1. When the two propulsion system or stage masses are equal [mi1) = mi2]. 2. When the mass ratios of the two stages are equal [mfm01 = mfm02]. (Note: Refer figure 1.1) | CO5 | A | 12 |
| 22. | a. | Classify the types of missiles based on its launching platform with its characteristics. | CO2 | U | 12 |
| 23. | a. | Explain the influence of various parts of a rocket on moment with its necessary equations. | CO3 | A | 7 |
|  | b. | Discuss the influence of characteristic fins on ∂Cl∂α | CO3 | U | 5 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Explain the different types of rocket thrust vector control methods with a neat sketch. | CO6 | U | 8 |
|  | b. | Describe the thrust magnitude control of a rocket. | CO6 | U | 4 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Discuss types of rockets and missiles with respect to Indian & International scenario. |
| CO2 | Analyze the Aerodynamics of rockets & missiles. |
| CO3 | Understand the performance of rocket and missiles within the atmosphere. |
| CO4 | Estimate the rocket performance in free space and gravitational field. |
| CO5 | Design the basic staging of rockets and missiles. |
| CO6 | Identify the control methods of rockets and missiles. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | - | 17 | - | - | - | - | 17 |
| CO2 | 4 | 25 | - | - | - | - | 29 |
| CO3 | - | 10 | 7 | 12 | - | - | 29 |
| CO4 | 1 | 3 | 4 | 9 | - | - | 17 |
| CO5 | 3 | 1 | 12 | - | - | - | 16 |
| CO6 | 3 | 12 | 1 | - | - | - | 16 |
|  |  |  |  |  |  |  | 124 |

Figure 1.1

END SEMESTER EXAMINATION – APRIL / MAY 2024

| Course Code | 20AE2044 | Duration | 3hrs |
| --- | --- | --- | --- |
| Course Name | BOUNDARY LAYER THEORY | Max. Marks | 100 |

| Q. No. | Questions | CO | BL | M |
| --- | --- | --- | --- | --- |
|  | PART – A (10 X 1 = 10 MARKS) |  |  |  |
| 1. | Sketch the hydrodynamic and thermal boundary layers for steady incompressible laminar flow of a constant property fluid over a thin flat plate, showing the relative size for Pr < 1, Pr =1 and Pr > 1. | CO1 | R | 1 |
| 2. | Specify the type of flow that can be analyzed by the Euler momentum equations | CO1 | R | 1 |
| 3. | Sketch the laminar and turbulent hydrodynamic boundary layer for steady incompressible laminar flow of a constant property fluid over a thin flat plate, showing shape and relative sizes and comment on the velocity profiles in laminar and turbulent flow | CO4 | U | 1 |
| 4. | The main observation by Prandtl based on which the boundary layer equations are developed is _________________ | CO1 | R | 1 |
| 5. | Express in one sentence, the meaning of hydrodynamically fully developed flow | CO2 | U | 1 |
| 6. | Differentiate the Darcy friction factor and Fanning friction coefficient | CO2 | U | 1 |
| 7. | State the non-dimensional numbers that are analogous to each other in hydrodynamic, thermal and concentration boundary layers | CO2 | R | 1 |
| 8. | A liquid metal to water heat exchanger used in a nuclear reactor has water on the shell side and liquid sodium on the tube side. For analyzing the flow in the tubes, assess the treatment of the flow, with regard to the state of development of the thermal and hydrodynamic boundary layers. | CO2 | U | 1 |
| 9. | What are the relative sizes of the thermal and hydrodynamic boundary layer in natural convection | CO2 | U | 1 |
| 10. | Laminar to Turbulent Transition happens are Reynolds number of ______ for internal flow and _______for external flow | CO4 | R | 1 |

|  | PART – B (6 X 3 = 18 MARKS) |  |  |  |
| --- | --- | --- | --- | --- |
| 11. | Express the most general law of conservation of mass in differential form (a) If the flow is steady, what equation will result? Will the resulting equation represent a flow that is compressible or incompressible? (b) If the flow is incompressible, what equation will result? Can this flow be unsteady? If yes, which quantities would vary with time? | CO1 | A | 3 |
| ‘12. | A nozzle is designed to accelerate the fluid from a velocity V1 to V2 in a linear fashion. That is, V = ax + b, where a, b are constants. If the flow is steady with V1 = 5 m/s at x1 = 0 and V2 = 15 m/s at x2 = 1 m determine the local, convective and total acceleration of the field at points 1 and 2 | CO2 | A | 3 |
| 13 | Define the stress tensor and the definition of pressure and explain how the definition is appropriate | CO2 | U | 3 |
| 14. | Briefly explain the appropriateness of the diagonal components of the symmetric component of the gradient tensor. | CO2 | U | 3 |
| 15. | A Newtonian fluid having a specific gravity of 0.92 and a kinematic viscosity of 5 x 10-4m2/s flows past a flat plate. The velocity profile is given by uU=32yδ-12yδ3. Using the momentum integral method the local boundary layer thickness for this velocity profile is given by the equation δx=4.64Rex, where the local Reynolds number is Rex=ρUxμ and x is the distance along the plate. Express the skin friction coefficient as a function of Rex. Calculate the shear stress at the plate surface at a point where the boundary layer thickness,  is 1 mm and free stream velocity U=10 m/s.. | CO2 | An | 3 |
| 16. | List the advantages of the Karman-Pohlhausen momentum integral method in solving the Navier Stokes equations, compared to Blasius solution? | CO5 | R | 3 |

| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.no 17 to 23, Q.no 24 is Compulsory) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17. | A tank of 0.05 m3volume contains air at 800 kPA (absolute) and 15oC. At time t = 0, air escapes from the tank through a valve with a flow area of 65 mm2. The air passing through the valve at t = 0 has a velocity of 311 m/s and a density of 6.13 kg/m3. Properties of the rest of the tank may be assumed to be uniform. Determine the instantaneous rate of change of density in the tank at t = 0. | CO2 | An | 12 |
| 18. | With a neat sketch write a note on boundary layer separation and list few methods to control it. | CO3 | A | 12 |
| 19. | From the continuity and Navier-Stokes equations, show that the velocity profile for steady, laminar, fully developed flow of an incompressible fluid between two infinite parallel plates, one of which is stationary and the other moving with a constant velocity U (Couette Flow) is given by u=Uyb+12μ∂p∂xy2-by. | CO2 | E | 12 |
| 20. | Determine a cubic velocity profile for the hydrodynamic boundary layer with appropriate boundary conditions. | CO5 | An | 12 |
| 21. | Determine a cubic temperature profile for the thermal boundary layer with appropriate boundary conditions. | CO5 | An | 12 |
| 22. | Consider the steady, laminar flow of an incompressible fluid past a flat plate. The boundary layer velocity profile is approximated as uU=yδ(x) for 0≤y≤δ and u=U for y> δ. Determine the hydrodynamic boundary layer thickness and the shear stress and skin friction coefficient using the Integral Momentum Boundary Layer equation, in terms of the local Reynolds number. Start from the Karman Pohlhausen Integral Equation and definition of Momentum Thickness for the given conditions | CO5 | E | 12 |
| 23. | (a) Describe the boundary layer effects in hypersonic flows. (b) Explain the differences between the analysis of hypersonic boundary layers compared to icompressible boundary layers. (c) Explain the differences between two and three dimensional boundary layers. | CO6 | An | 12 |
|  | Compulsory: |  |  |  |
| 24. | Consider the steady, laminar flow of a liquid metal past a flat plate. Determine the thermal boundary layer thickness and the surface heat flux and the Nusselt number using the Integral Thermal Boundary Layer equation for a cubic temperature profile, in terms of the local Reynolds number and Prandtl No. Consider the value of the Prandtl number for the given fluid. Assume that the thermal boundary layer is so thick that the velocity inside the thermal boundary layer is the free stream velocity, U∞ throughout. Start from thermal integral equation for the given conditions. | CO5 | E | 12 |

# Some useful information required

Properties Required –Density of air = 1.16 kg/m3, Dynamic Viscosity of air = 184.6 x 10-7N.s/m2

δ1x=0δ(x)1-u(x,y)U∞(x) dy,        δ2x=0δ(x)u(x,y)U∞(x)1-u(x,y)U∞(x) dy

For a thin flat plate

U∞2dδ2dx=τsρ          U∞ddx0δt(x)ρCpu(x,y)U∞T-T∞ dy=qs

|  | COURSE OUTCOMES |
| --- | --- |
|  | The student will be able to |
| CO1 | Define the fundamentals of boundary layer theory. |
| CO2 | Solve the equations involved in boundary layer theory. |
| CO3 | Analyze the different kinds of boundary layer control. |
| CO4 | Differentiate turbulent and laminar boundary layers. |
| CO5 | Estimate the boundary layer thickness for flow over a different bodies. |
| CO6 | Attain knowledge of boundary layer effects in hypersonic flows. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 3 |  | 3 |  |  |  | 6 |
| CO2 | 1 | 10 | 15 | 15 |  |  | 41 |
| CO3 |  |  | 12 |  |  |  | 12 |
| CO4 | 1 | 1 |  |  |  |  | 2 |
| CO5 | 3 |  |  | 24 | 24 |  | 51 |
| CO6 |  |  |  |  | 12 |  | 12 |
|  | 8 | 11 | 30 | 39 | 36 | 0 | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 20AE2048 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # UNMANNED AIRCRAFT SYSTEMS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Define UAS. |  | CO1 | R | 1 |
| 2. | Name any two elements of an UAS. |  | CO1 | R | 1 |
| 3. | What are the three phases of the design of most aircraft-based systems? |  | CO2 | R | 1 |
| 4. | Differentiate MAV and NAV. |  | CO2 | U | 1 |
| 5. | What is the name of the regulatory authorities for civilian UAVs in the USA and Australia? |  | CO3 | R | 1 |
| 6. | Name any two reasons that a UAV system remains undetected in operation. |  | CO3 | U | 1 |
| 7. | Differentiate between non-dispensable and dispensable payloads. |  | CO4 | U | 1 |
| 8. | Name the three media for communicating between ground control station and UAS. |  | CO4 | R | 1 |
| 9. | Whar are the different types of UAV sub-assembly and sub-system testing? |  | CO5 | R | 1 |
| 10. | Illustrate the important roles of UAV in naval, army and air force sectors. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Draw the functional structure of UAV system. |  | CO1 | U | 3 |
| 12. | What are the different aircraft configurations? Discuss in detail. |  | CO2 | U | 3 |
| 13. | What are the aspects of airframe design? |  | CO3 | U | 3 |
| 14. | What are the driving factors in describing the various payloads? |  | CO4 | U | 3 |
| 15. | Write a short note on radio frequency spectra. |  | CO5 | U | 3 |
| 16. | Illustrate the phases of UAS development. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Discuss in detail about the categories of systems based upon air vehicle types. | CO1 | A | 12 |
| 18. | a. | Discuss the important parameters to be considered during the selection of the UAV system. | CO2 | A | 12 |
| 19. | a. | Write a short note on defence standard 00-970 – Part 9 of an UAV. | CO3 | A | 4 |
|  | b. | Explain the concept of flight envelope and build-standard document in detail with schematic. | CO3 | A | 8 |
| 20. | a. | What are the three systems in use to designate frequency bands? Discuss in detail. | CO4 | A | 6 |
|  | b. | Explain the concept of mid-air collision (MAC) avoidance. | CO4 | A | 6 |
| 21. | a. | Write a brief note on system ground testing. | CO5 | A | 6 |
|  | b. | Write a brief note on system in-flight testing. | CO5 | A | 6 |
| 22. | a. | Explain the principal means of detecting an air vehicle through its signatures. | CO3 | A | 12 |
| 23. | a. | Explain the types of antennas used in UAV with a schematic. | CO4 | An | 6 |
|  | b. | Explain the concept of catapult launch system recovery. | CO4 | An | 6 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Discuss in detail about the launch of HTOL & VTOL systems. | CO6 | An | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the basic terminologies and classification of UAS. |
| CO2 | Relate the design parameters of UAV systems. |
| CO3 | Obtain knowledge on the application of aerodynamic principles to design UAS. |
| CO4 | Obtain knowledge on payloads and launch systems for UAS. |
| CO5 | Understand the basic principles of UAV Testing. |
| CO6 | Apply the principles to design UAS for specific applications. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | R | U | A | An | E | C | Total |
| CO1 | 2 | 3 | 12 |  |  |  | 17 |
| CO2 | 1 | 4 | 12 |  |  |  | 17 |
| CO3 | 1 | 4 | 24 |  |  |  | 29 |
| CO4 | 1 | 4 | 12 | 12 |  |  | 29 |
| CO5 | 1 | 3 | 12 |  |  |  | 16 |
| CO6 | - | 4 | - | 12 |  |  | 16 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 20AE2052 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # WIND TUNNEL TECHNIQUES | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | Define Reynolds number and Mach number. |  | CO1 | R | 1 |
| 2. | Name the component that has converging passage at the upstream of the test section. |  | CO2 | R | 1 |
| 3. | Recite the terms “Lift to Drag Ratio”. |  | CO4 | R | 1 |
| 4. | Express, how the higher Mach numbers are achieved in the supersonic wind tunnels. |  | CO2 | U | 1 |
| 5. | Define different type of flows based on Mach number. |  | CO3 | R | 1 |
| 6. | Define the “Total Pressure” in pitot tube. |  | CO3 | R | 1 |
| 7. | Infer the loads and moments acting on a body immersed in a stream of fluid. |  | CO4 | U | 1 |
| 8. | State the use of LDA. |  | CO5 | R | 1 |
| 9. | Recite the use of manometers and draw a “U type manometer”. |  | CO4 | R | 1 |
| 10. | Identify the applications of “PIV” in wind tunnel measurement system. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | Write the Compressible flow theory. |  | CO4 | A | 3 |
| 12. | Distinguish between the “Hypersonic” and “Supersonic” condition in the wind tunnel. |  | CO2 | U | 3 |
| 13. | Differentiate between the “internal balance” and “outer balance “in wind tunnel measurement system. |  | CO4 | U | 3 |
| 14. | Brief on the procedure to measure moments using internal balances. |  | CO3 | U | 3 |
| 15. | Distinguish between optical and non-optical visualization techniques. |  | CO6 | U | 3 |
| 16. | Compare the use of “surface oil flow” and “tufts” in visualization techniques used in wind tunnel. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Explain the various parts of a “water tunnel” and write the arrangement, working and applications. | CO1 | A | 6 |
|  | b. | Explain the various parts of a subsonic wind tunnel with help of a diagram and explain its working principle. | CO1 | A | 6 |
| 18. | a. | Analyze the “Runtime mass flow rate” in the supersonic wind tunnel. | CO4 | An | 6 |
|  | b. | Compare the Continuous and intermittent type of wind tunnels. | CO2 | An | 6 |
| 19. | a. | Sketch the arrangement of shock tube, describe its construction and its notable performance with neat sketches. | CO2 | A | 6 |
|  | b. | Appraise about the “Starting and stopping Loads” in Supersonic wind tunnel. | CO2 | An | 6 |
| 20. |  | Establish the various calibration tests to be conducted on wind tunnels with suitable sketches. | CO2 | A | 12 |
| 21. | a. | Distinguish between the blow down and other types of wind tunnel. | CO2 | An | 6 |
|  | b. | Explain the working principle of Pitot tube and its applications in wind tunnel measurement. | CO4 | A | 6 |
| 22. | a. | Explain the manometers used in wind tunnel. | CO5 | A | 6 |
|  | b. | Articulate the various parts of an “Internal Strain gauge balance” with help of a diagram and explain its working principle. | CO5 | A | 6 |
| 23. |  | Illustrate the operation of PLIF in detail with help of a schematic diagram. | CO5 | An | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Explain how the “Shadowgraph” technique can be used in wind tunnel with help of a neat sketch. | CO6 | A | 6 |
|  | b. | Explain the “Schlieren flow visualization” technique used in wind tunnel measurement system with help of a neat sketch. | CO6 | A | 6 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the various types of wind tunnels and test techniques. |
| CO2 | Choose proper high speed wind tunnel for required test |
| CO3 | Choose correct model for wind tunnel testing. |
| CO4 | Estimate the forces and moments for given model. |
| CO5 | Estimate pressure, velocity and temperature using measurement techniques. |
| CO6 | Choose the proper flow visualization techniques |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO | R | U | A | An | E | C | Total |
| CO1 | 1 | - | 12 | - | - | - | 13 |
| CO2 | 1 | 4 | 18 | 18 | - | - | 41 |
| CO3 | 2 | 3 | - | - | - | - | 5 |
| CO4 | 2 | 4 | 9 | 6 | - | - | 21 |
| CO5 | 1 | - | 12 | 12 | - | - | 25 |
| CO6 | - | 7 | 12 | - | - | - | 19 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 20AE2056 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # BASICS OF AEROSPACE ENGINEERING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Major aircraft component which holds the passengers along with cargo is called as __________. |  | CO1 | R | 1 |
| 2. | Chanute designed hang gliders with _________method of structural rigging. |  | CO1 | R | 1 |
| 3. | The force caused by air resistance that slows down an airplane is known as _______________. |  | CO2 | R | 1 |
| 4. | High pressure air can escape around the tip and flow up around to the top surface leading to ______________. |  | CO2 | R | 1 |
| 5. | Name 4 common metals used in aircrafts. |  | CO3 | R | 1 |
| 6. | Classify aircrafts based on their usage. |  | CO3 | R | 1 |
| 7. | About ____________ % of thrust in a turboprop engine is generated in the Propellers. |  | CO4 | R | 1 |
| 8. | Write the name of the navigation system developed by India. |  | CO5 | R | 1 |
| 9. | What is meant by payload in a multi copter? |  | CO5 | R | 1 |
| 10. | Name the first successful mission landing on the moon by NASA. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Recall two advantages of monoplanes over Biplanes. |  | CO1 | R | 3 |
| 12. | Discuss the reason why the equal transit theory is not right in explaining the generation of lift. |  | CO2 | R | 3 |
| 13. | Draw the stress-strain curve of a typical composite material. |  | CO3 | R | 3 |
| 14. | Why is the efficiency of a turboprop engine more than a turbojet engine? |  | CO4 | R | 3 |
| 15. | Write a brief note on the medium earth orbit and its applications. |  | CO5 | R | 3 |
| 16. | Write a note on India’s chandrayaan-II mission. |  | CO6 | R | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No 17 to 23, Q.No 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Illustrate the vital contributions of Wright brothers that led to the first successful flight in 1903. | CO1 | R | 12 |
| 18. | a. | Draw the outline of a typical aircraft and detail its parts and functions. | CO1 | U | 12 |
| 19. | a. | Explain the principle and working of control surfaces in an aircraft wing with a neat sketch. | CO2 | U | 12 |
| 20. | a. | Explain the principal axes of an aircraft and its related maneuvers with a neat sketch. | CO2 | U | 12 |
| 21. | a. | Explain the working of a turboprop engine with a neat sketch. | CO4 | U | 12 |
| 22. | a. | Explain the working and advantages of a liquid propellant rocket engine? | CO4 | R | 12 |
| 23. | a. | Explain the basic components of a multi-copter drone with neat sketch. | CO6 | U | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Explain the uniqueness and characteristics of Mangalyaan mission. | CO5 | U | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the evolution of aircrafts and flying vehicles |
| CO2 | Understand the parts and functions of aircrafts |
| CO3 | Obtain knowledge on principles of flight. |
| CO4 | Understand the fundamentals of structures and materials used in Aerospace applications. |
| CO5 | Understand the principles of aircraft and rocket propulsion. |
| CO6 | Obtain knowledge on the components and function of Multi-copter drones. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 17 | 12 | - | - | - | - | 29 |
| CO2 | 5 | 24 | - | - | - | - | 29 |
| CO3 | 5 | - | - | - | - | - | 5 |
| CO4 | 4 | 24 | - | - | - | - | 28 |
| CO5 | 5 | 12 | - | - | - | - | 17 |
| CO6 | 4 | 12 | - | - | - | - | 16 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 20AE2062 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # HEAT AND MASS TRANSFER | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | State the Fourier’s law of heat transfer. |  | CO1 | R | 1 |
| 2. | Define thermal conductivity. |  | CO1 | R | 1 |
| 3. | Explain thermal resistance. |  | CO2 | U | 1 |
| 4. | Identify the unit of thermal diffusivity. |  | CO2 | R | 1 |
| 5. | State Newton’s law of cooling. |  | CO3 | R | 1 |
| 6. | Define the Prandtl number. |  | CO3 | R | 1 |
| 7. | Define the natural convection. |  | CO4 | R | 1 |
| 8. | State the significance of characteristic length. |  | CO4 | R | 1 |
| 9. | Define the intensity of radiation. |  | CO5 | R | 1 |
| 10. | Define the parallel flow heat exchanger. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | Give examples of heat convection. |  | CO1 | R | 3 |
| 12. | Explain lumped analysis. |  | CO2 | U | 3 |
| 13. | Explain the hydrodynamic entrance region. |  | CO3 | U | 3 |
| 14. | Give examples of free convection. |  | CO4 | U | 3 |
| 15. | Explain the emissivity of the radiation. |  | CO5 | U | 3 |
| 16. | Explain the features of the recuperator. |  | CO6 | U | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No. 17 to 23, Q.No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Explain the advantages of heat transfer over thermodynamics. | CO1 | U | 4 |
|  | b. | A furnace wall is made up of three layers, the inside layer with a thermal conductivity of 8.5 W/mK, the middle layer with a thermal conductivity of 0.25 W/mK and the outer layer with conductivity of 0.08 W/mK. The respective thickness of the inner, middle and outer layers are 25 cm, 5 cm and 3 cm respectively. The inside and outside wall temperatures are 600˚C and 50˚C respectively. Sketch the equivalent electrical circuit for the conduction of heat through the wall and calculate thermal resistance, heat flow/m2 and interface temperatures. | CO2 | A | 8 |
| 18. | a. | Produce the general three-dimensional heat conduction equation in a cylindrical coordinate system. | CO2 | A | 8 |
|  | b. | A sphere of 100 mm diameter has a thermal conductivity of 0.18 W/mK. The outer surface temperature is 8 ˚C and 250 W/m² of energy is released due to heat source. Calculate the following - (i) Heat generated (ii) Temperature at the center of the sphere | CO2 | A | 2 2 |
| 19. | a. | A wire of 7 mm diameter is covered with an insulating material (k= 1 W/mK). The wire temperature and ambient temperature are 80˚C and 15 ˚C. If the inside convective heat transfer coefficient is 8.2 W/m2K, calculate the minimum thickness of insulation and the percentage of increase in the heat dissipation. | CO2 | A | 6 |
|  | b. | Aluminum fins 1.5 cm wide and 10 mm thick are placed on a 2.5 cm diameter tube to dissipate the heat. The tube surface temperature is 170 ˚C and the ambient temperature is 20 ˚C. Calculate the heat loss per fin. Take h= 130 W/m2 ˚C and k= 200 W/m ˚C for aluminum. | CO2 | A | 6 |
| 20. | a. | Explain the development of a thermal boundary layer over a plate. | CO3 | U | 6 |
|  | b. | Air at 25 ˚C at the atmospheric pressure is flowing over a flat plate at 3 m/s. If the plate is 1 m wide and the temperature Tw = 75 ˚C. Calculate the following at a location of 1 m from the leading edge. - Hydrodynamic boundary layer thickness - Local friction coefficient - Thermal boundary layer thickness | CO3 | A | 2 2 2 |
| 21. | a. | A horizontal plate of 800 mm long and 70 mm wide is maintained at a temperature of 140 ˚C in a large tank full of water at 60 ˚C. Determine the total heat loss from the plate. | CO4 | A | 12 |
| 22. | a. | A black body of 1200 cm2 emits radiation at 1000 K. Calculate the following - Total rate of energy emission - Intensity of normal radiation - Wavelength of maximum monochromatic emissive power - Intensity of normal radiation along a direction at 60˚ to the normal | CO5 | A | 12 |
| 23. | a. | Calculate the net radiant heat exchange per m² area for two large parallel plates at temperatures of 427˚C and 27˚C respectively. The emissivities of hot and cold plates are 0.9 and 0.6 respectively. If a polished aluminum shield is placed between them, calculate the percentage of reduction in the heat transfer. Take the emissivity of the shield as 0.4. | CO5 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | A parallel flow heat exchanger is used to cool 4.2 kg/min of hot liquid of specific heat 3.5 kJ/kg K at 130˚C. A cooling water of specific heat 4.18 kJ/kg K is used for cooling purposes at a temperature of 15˚C. The mass flow rate of cooling water is 17 kg/min. Calculate the following - Outlet temperature of liquid - Outlet temperature of water - Effectiveness of heat exchanger Take the overall heat transfer coefficient is 1100 W/m²K and heat exchanger area is 0.3 m². | CO6 | A | 4 4 4 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand the various modes of heat transfer and the factors affecting it. |
| CO2 | Solve steady state and transient heat conduction problems. |
| CO3 | Understand the physical phenomena associated with convective transport processes. |
| CO4 | Understand the role of non-dimensional parameters and use them to solve practical convective heat transfer problems. |
| CO5 | Understand the physical mechanisms involved in radiation heat transfer. |
| CO6 | Select and design heat exchangers for a given application and heat load. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | R | U | A | An | E | C | Total |
| CO1 | 5 | 4 | - | - | - | - | 9 |
| CO2 | 1 | 4 | 32 | - | - | - | 37 |
| CO3 | 2 | 9 | 6 | - | - | - | 17 |
| CO4 | 2 | 3 | 12 | - | - | - | 17 |
| CO5 | 1 | 3 | 24 | - | - | - | 28 |
| CO6 | 1 | 3 | 12 | - | - | - | 16 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 21AE3002 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # ADVANCED STRUCTURAL ANALYSIS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |  |
| 1. |  | Draw the Mohr’s circle of the stress element shown below. Determine the principal stresses and the maximum shear stresses. Verify the answer with analytical solutions. | CO1 | A | 16 |
| 2. |  | Determine the stress-strain relation for an angle lamina using transformation matrix (T) and router (R). | CO2 | A | 16 |
| 3. |  | A cantilever beam of rectangular section is subjected to a load of 1000 N which is inclined at an angle of 30° to the vertical. Determine the normal stress due to bending at point D near the built-in-end. | CO3 | A | 16 |
| 4. |  | The cross section of a channel beam with double flanges and constant thickness throughout the section is shown in the Figure. Derive the following formula for the distance e from the centerline of the web to the shear center S: | CO4 | A | 16 |
| 5. |  | The cross section of a slit rectangular tube of constant thickness is shown in the Figures. Derive the following formula for the distance e from the centerline of the wall of the tube in the Figure part a to the shear center S: | CO4 | A | 12 |
| 6. |  | A cantilever beam as shown in Fig. carries concentrated loads as shown. Calculate the distribution of stiffener loads and the shear flow distribution in the web panels assuming that the latter are effective only in shear. | CO5 | A | 16 |
| 7. |  | Find the shear flow and twist per unit length of the two cell tube made of aluminium as shown Fig. and is subjected to a torque 90000 Ncm | CO6 | A | 16 |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |  |
| 8. |  | A cantilever of angle is 1 m long and is fixed at one end, while it is subjected to a load of 3 kN at the free end at 200 to the vertical. Determine the bending stress at A, B and C. | CO2 | A | 20 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CO1 | Understand stress and strain compatibility conditions. |  |  |  |  |  |  |  |  |
| CO2 | Derive Stress-strain relationship of a lamina. |  |  |  |  |  |  |  |  |
| CO3 | Differentiate the symmetrical and unsymmetrical bending. |  |  |  |  |  |  |  |  |
| CO4 | Determine the shear center in various open and closed section of aircraft structures. |  |  |  |  |  |  |  |  |
| CO5 | Analyze the buckling of plates to predict the critical stress. |  |  |  |  |  |  |  |  |
| CO6 | Design aircraft composite panel for aerospace applications. |  |  |  |  |  |  |  |  |
| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |  |
| CO / BL |  | R | U | A | An | E | C | Total |
| CO1 |  |  |  | 16 |  |  |  | 16 |
| CO2 |  |  |  | 36 |  |  |  | 36 |
| CO3 |  |  |  | 16 |  |  |  | 16 |
| CO4 |  |  |  | 32 |  |  |  | 32 |
| CO5 |  |  |  | 16 |  |  |  | 16 |
| CO6 |  |  |  | 16 |  |  |  | 16 |
|  |  |  |  |  |  |  |  | 132 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 21AE3005 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # ELEMENTS OF DATA ANALYTICS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |  |
| 1. | a. | Describe in detail about the types of variables and levels of measurement. | CO1 | U | 10 |
|  | b. | Give a brief introduction to R-programming. | CO1 | R | 6 |
| 2. | a. | Write short notes on measures of location of dispersion | CO2 | An | 6 |
|  | b. | (i) On the television game Let’s Make a Deal, a contestant is presented with a choice of three (closed) doors. Behind exactly one door is a terrific prize. The other doors conceal cheap items. First, the contestant is asked to choose a door. Then the host of the show (Monty Hall) shows the contestant one of the worthless prizes behind one of the other doors. At this point, there are two closed doors, and the contestant is given the opportunity to switch from his original choice to the other closed door. The question is, is it better for the contestant to stick to his original choice or to switch doors? (ii) Doctors apply a medical test for a certain rare disease that has the property that if the patient is affected by the disease, then the test is positive in 99% of the cases. However, it happens in 2% of the cases that a healthy patient tests positive. Statistical data shows that one person out of 1000 has the disease. What is the probability for a patient with a positive test to be affected by the disease? | CO2 | A | 10 |
| 3. | a. | Differentiate between binomial, Poisson and geometric distributions. | CO2 | U | 6 |
|  | b. | Explain in detail about the different types of statistical hypothesis tests | CO3 | U | 10 |
| 4. | a. | 25 patients with blisters Treatments: Treatment A, Treatment B, Placebo Measurement: Number of days until blisters heal A: 5,6,6,7,7,8,9,10 B: 7,7,8,9,9,10,10,11 P: 7,9,9,10,10,10,11,12,13 - Are these differences significant? - Compare largest and smallest standard deviations for all the 3 groups - Also determine F | CO3 | A | 9 |
|  | b. | Differentiate SST, MST and standard deviation and highlight their correlations. | CO3 | A | 7 |
| 5. | a. | (i) How many three-digit numbers can be formed from the digits 0, 1, 2, 3, 4, 5, and 6 if each digit can be used only once? (ii) How many of these are odd numbers? (iii) How many are greater than 330? | CO4 | A | 8 |
|  | b. | Define : i) Normality (ii) Additivity (iii) Independence and (iv) F-ratio | CO4 | R | 8 |
| 6. | a. | Distinguish between - Correlation and causation (ii) Variance and covariance - Linear and Multiple regression | CO5 | U | 8 |
|  | b. | Explain in detail about least squares regression. | CO5 | A | 8 |
| 7. |  | Write short notes on i) simple linear regression (ii) Ridge Regression, (iii) Lasso Regression, iv) K Nearest Neighbours Regression | CO6 | A | 16 |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |  |
| 8. |  | A car dealer wants to find the relationship between the odometer reading and the selling price of used cars. A random sample of 100 cars is selected, and the data recorded. Test to determine whether there is enough evidence to infer that there is a linear relationship between the car auction price and the odometer reading for all three-year-old Tauruses, Use a = 5%. Find the regression line, the standard error of estimate and the coefficient of determination using the table below. Car; Odometer; Price. 1; 37388; 14636. 2; 44758; 14122. 3; 45833; 14016. 4; 30862; 15590. 5; 31705; 15568. 6; 34010; 14718. | CO4 | 20 | An |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Find a meaningful pattern in data |
| CO2 | Graphically interpret data. |
| CO3 | Implement the analytic algorithms |
| CO4 | Handle large scale analytics projects from various domains |
| CO5 | Develop intelligent decision support systems. |
| CO6 | Understand the various techniques in handling the data |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | R | U | A | An | E | C | Total |
| CO1 | 6 | 10 |  |  |  |  | 16 |
| CO2 |  | 6 | 10 | 6 |  |  | 22 |
| CO3 |  | 10 | 16 |  |  |  | 26 |
| CO4 | 8 |  | 8 | 20 |  |  | 36 |
| CO5 |  | 8 | 8 |  |  |  | 16 |
| CO6 |  |  | 16 |  |  |  | 16 |
|  |  |  |  |  |  |  | 132 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 21AE3006 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # ADVANCED PROPULSION TECHNOLOGY | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |  |
| 1. | a. | Describe the methods of thrust augmentation with neat sketches. | CO1 | U | 8 |
|  | b. | Compare characteristics of turboprop, turbofan and turbojet engines. | CO1 | U | 8 |
| 2. | a. | Explain surge margin requirements and thrust engine start envelope. | CO2 | A | 8 |
|  | b. | Describe the matching of turbine and compressor with neat sketch. | CO2 | A | 8 |
| 3. | a. | Explain the operating principle of chemical rocket with a neat sketch. | CO3 | U | 8 |
|  | b. | What is meant by hybrid rocket propulsion? Briefly explain the relative advantages of liquid rockets over solid rockets. | CO3 | U | 8 |
| 4. | a. | Elaborate the need and importance of green propellants. | CO4 | An | 8 |
|  | b. | Discuss on Paraffin wax-Hydrogen Peroxide and Nitrous oxide as green candidates for hybrid rockets. | CO4 | An | 8 |
| 5. | a. | Explain the concept of electric rocket propulsion with neat sketch. | CO5 | An | 8 |
|  | b. | Describe the nuclear rocket engine design and performance with neat sketches. | CO5 | An | 8 |
| 6. | a. | Illustrate the working of gas turbine engine with a neat sketch. | CO1 | A | 8 |
|  | b. | Explain the operating principle and performance characteristics of ramjet engine with neat sketches. | CO1 | A | 8 |
| 7. | a. | How solar sail works? Explain the types of electric thruster. | CO5 | A | 8 |
|  | b. | Compare fission and fusion propulsion. Discuss the applications of nuclear propulsion systems. | CO5 | A | 8 |
| PART – B (1 X 20 = 20 MARKS) [Compulsory Question] |  |  |  |  |  |
| 8. | a. | Explain the salient features of scramjet engine and its applications for hypersonic vehicles. | CO6 | An | 10 |
|  | b. | Describe the different types of tests performed before rocket propulsion systems are put into operational use. | CO6 | An | 10 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL        M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Illustrate the performance of various cycles of turbine engine. |
| CO2 | Estimate the performance of aircraft engines. |
| CO3 | Design the subsystems for chemical rockets. |
| CO4 | Analyze and compare the performance of chemical rockets. |
| CO5 | Design the subsystems for green propulsion systems |
| CO6 | Evaluate the performance of space thrusters. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | R | U | A | An | E | C | Total |
| CO1 |  | 16 | 16 |  |  |  | 32 |
| CO2 |  |  | 16 |  |  |  | 16 |
| CO3 |  | 16 |  |  |  |  | 16 |
| CO4 |  |  |  | 16 |  |  | 16 |
| CO5 |  |  | 16 | 16 |  |  | 32 |
| CO6 |  |  |  | 20 |  |  | 20 |
|  |  |  |  |  |  |  | 132 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 21AE3007 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # MODELING AND SIMULATION OF AEROSPACE VEHICLES | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |  |
| 1. | a. | Describe the system modeling, highlighting the static and dynamic models with the aid of a schematic. | CO1 | U | 8 |
|  | b. | Discuss about the distributed lag models used in system simulation. | CO1 | U | 8 |
| 2. | a. | Develop an autopilot system for a vanguard missile based on the lead compensation technique. | CO2 | A | 8 |
|  | b. | Construct the tasks associate with simulation programming and its execution with the aid of a schematic. | CO2 | A | 8 |
| 3. | a. | Explain the forces and moments acting on an aircraft with a schematic. | CO3 | U | 16 |
| 4. | a. | Articulate on the various qualities considered for evaluating an architecture. With an example explain how the availability is considered for architecture evaluation. | CO4 | A | 16 |
| 5. | a. | Explain the feedback loops employed in flight simulation. | CO5 | U | 8 |
|  | b. | Show the stock and flow diagram and indicate its salient features. | CO5 | U | 8 |
| 6. | a. | Summarize the need for constitutive equations and explain the three principles that are satisfied by constitutive equations. | CO5 | U | 8 |
|  | b. | Identify the building blocks for modeling aerospace vehicle dynamics adopting a mathematical shorthand notation to describe them in space and time. | CO5 | U | 8 |
| 7. | a. | Enumerate on the four principles used in modeling. | CO1 | R | 8 |
|  | b. | Examine the simulation numerical technique based on a continuous model. Plot the graph representing the sales made. | CO1 | R | 8 |
| PART – B (1 X 20 = 20 MARKS) [Compulsory Question] |  |  |  |  |  |
| 8. | a. | Describe how a flight simulator is employed as a training device as well as research tool. | CO6 | R | 10 |
|  | b. | Comparex the advantages and disadvantages of flight simulators. | CO6 | R | 10 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL        M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Analyze the concepts of system models. |
| CO2 | Practice system simulation for cockpit systems. |
| CO3 | Model and design aircraft elements. |
| CO4 | Comprehend the principles behind system assessment, validation and certification. |
| CO5 | Relate system dynamics and mathematical models for flight simulation. |
| CO6 | Relate to the usage of flight simulator for various aircrafts. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | R | U | A | An | E | C | Total |
| CO1 | 16 | 16 | - | - | - | - | 32 |
| CO2 | - | - | 16 | - | - | - | 16 |
| CO3 | - | 16 | - | - | - | - | 16 |
| CO4 | - | - | 16 | - | - | - | 16 |
| CO5 | - | 32 | - | - | - | - | 32 |
| CO6 | 20 | - | - | - | - | - | 20 |
|  |  |  |  |  |  |  | 132 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 21AE3010 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # ADVANCED AIRCRAFT MATERIALS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (4 X 20 = 80 MARKS) (Answer all the Questions) |  |  |  |  |  |
| 1. |  | Explain the characteristics and applications of wood, including hardwoods and softwoods, in aerospace, focusing on engineering properties and usage in wing spars and stringers. | CO1 | U | 20 |
|  |  | (OR) |  |  |  |
| 2. |  | Explain the properties and applications of various metallic materials used in aircraft construction, including carbon steels, nickel steels, and corrosion-resistant alloys, emphasizing their roles in different components. | CO1 | An | 20 |
| 3. |  | Explain the characteristics and applications of titanium alloys and describe the heat treatment process of this alloy. | CO2 | A | 20 |
|  |  | (OR) |  |  |  |
| 4. | a. | Explain the roles and applications of fiber-reinforced aluminum laminates and titanium matrix composites in aircraft components. | CO3 | A | 10 |
|  | b. | Describe the uses of lightweight metals and metal-matrix composites in aircraft components, highlighting their specific advantages. | CO3 | U | 10 |
| 5. |  | Explain the fabrication methods and microstructural developments in non-oxide ceramics, particularly silicon nitride matrix composites. | CO4 | A | 20 |
|  |  | (OR) |  |  |  |
| 6. |  | Describe the use of high-temperature materials like non-oxide ceramics and carbon-based composites in aircraft components such as turbine blades. | CO4 | A | 20 |
| 7. |  | Explain the manufacturing processes involved in composite production, including prepreg manufacturing, resin infusion, and machining techniques for composites. | CO5 | U | 20 |
|  |  | (OR) |  |  |  |
| 8. |  | Explain the properties and applications of fire-resistant composites. | CO6 | U | 20 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 9. |  | Describe the properties, working principles and applications of various smart materials in aerospace industries. | CO6 | A | 20 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL        M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Explore the use of conventional materials for aircraft structures. |
| CO2 | Learn the properties and composition of alloys for aerospace application. |
| CO3 | Design and analyze light weight metals and composite structures. |
| CO4 | Understand the definition and classification of aerospace composites. |
| CO5 | Choose suitable manufacturing method for composite materials. |
| CO6 | Examine smart and intelligent material characteristics and engineering effect. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | R | U | A | An | E | C | Total |
| CO1 | - | 20 | - | 20 | - | - | 40 |
| CO2 | - | - | 20 | - | - | - | 20 |
| CO3 | - | 10 | 10 | - | - | - | 20 |
| CO4 | - | - | 40 | - | - | - | 40 |
| CO5 | - | 20 | - | - | - | - | 20 |
| CO6 | - | 20 | 20 | - | - | - | 40 |
|  |  |  |  |  |  |  | 180 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 21AE3011 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # SIMULATION AND MODEL BASED SYSTEMS ENGINEERING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | Marks |
| --- | --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |  |
| 1. | a. | Give an overview of system engineering and explain the various challenges of system engineering. | CO1 | R | 8 |
|  | b. | Explain the life cycle stages of system engineering. | CO1 | A | 8 |
| 2. | a. | Mention the importance of presentations and its requirements for a systems engineering process. | CO2 | U | 8 |
|  | b. | Explain architectural design and significance of verification and validation. | CO2 | A | 8 |
| 3. | a. | Briefly explain enabling patterns and its significance to system engineering. | CO3 | R | 8 |
|  | b. | Explain your understanding on test pattern with example. | CO3 | U | 8 |
| 4. | a. | Explain your understanding on life cycle pattern. | CO4 | U | 8 |
|  | b. | Explain with block diagram pattern-based systems engineering. | CO4 | A | 8 |
| 5. | a. | Explain the following of context patterns a.Aim, b. Concept, c.Viewpoint and d. Overview. | CO5 | A | 8 |
|  | b. | Evaluate model based system engineering for aerospace industries. | CO5 | E | 8 |
| 6. | a. | Explain the aim, concept, viewpoint and overview of analysis patterns. | CO6 | A | 8 |
|  | b. | Explain assessing framework ontologies using patterns for model assessment. | CO6 | A | 8 |
| 7. | a. | Explain multidisciplinary approach in a system engineering process with necessary block diagram. | CO1 | U | 16 |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |  |
| 8. | a. | XYZ is the customer for an ABC engine manufacturing company. The ABC company needs to supply the turbojet engine for the aircraft, based on the customer’s requirement. Suggest the roles and responsibility of the system engineers and the various modelling tools to be used for the optimum operation of the entire system. | CO6 | An | 20 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Understand System Engineering and its usage. |
| CO2 | To understand how Model Based Engineering used in development of Systems. |
| CO3 | To articulate the usage of Modelling Patterns. |
| CO4 | To illustrate the concepts of MBSE. |
| CO5 | To understand the concepts of Modelling Patterns. |
| CO6 | To examine applications and case studies of Modelling Patterns. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 8 | 16 | 8 |  |  |  | 32 |
| CO2 |  | 8 | 8 |  |  |  | 16 |
| CO3 | 8 | 8 |  |  |  |  | 16 |
| CO4 |  | 8 | 8 |  |  |  | 16 |
| CO5 |  |  | 8 |  | 8 |  | 16 |
| CO6 |  |  | 16 | 20 |  |  | 36 |
|  |  |  |  |  |  |  | 132 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 21AE3013 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # DATA ANALYTICS AND VISUALIZATION | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |  |
| 1. | a. | Explain the different steps in the data science process. Identify the step which takes the longest time to complete? Justify your answer. | CO1 | U | 10 |
|  | b. | Interpret the information found in the following visualizations. | CO1 | An | 6 |
| 2. | a. | Describe the detailed structure of the four basic dataset types with the help of a neat diagram and write the basics of Natural Language Processing. | CO2 | U | 16 |
| 3. | a. | Estimate the regression parameters with a few (x, y) observations as shown in the following table using the least square regression method. X; Y. 10; 23. 20; 43. 30; 63. 40; 83. 50; 103. | CO3 | A | 16 |
| 4. | a. | Describe in detail about the four levels of visualization design and explain different set of threats to validity associated with visual design. | CO4 | U | 16 |
| 5. | a. | Elucidate the five major building blocks of power BI namely the dashboards, reports, workbooks, datasets and workflows. Also explain how to create the dashboard using Power BI. | CO5 | R | 16 |
| 6. | a. | Distinguish between bar charts and histograms. Explain the different classification of histograms based on the frequency distribution of the data with the help of necessary diagrams. | CO5 | U | 10 |
|  | b. | Discuss the precautions to take into account while using treemaps with power BI. List the appropriate use cases for treemaps. | CO5 | U | 6 |
| 7. | a. | Discuss in detail about linear and logistic regression with suitable examples? | CO2 | U | 10 |
|  | b. | Explain how to create an excel dashboard and list the advantages of the dashboards. | CO6 | R | 6 |
| PART – B (1 X 20 = 20 MARKS) (Compulsory Question) |  |  |  |  |  |
| 8. | a. | The following table lists values of temperature and windspeed. Use the data to draw a scatter graph. Describe the type of correlation that you observe. Give some situations where the scatter plots are used. Temperature; 14; 12; 8; 7; 4; 6; 3; 2. Wind speed; 5; 7; 7; 10; 11; 13; 16; 20. | CO6 | An | 15 |
|  | b. | Construct a box plot and write the five number summary for the given data. 18, 27, 34, 52, 54, 59, 61, 68, 78, 82, 85, 87, 91, 93, 100 | CO6 | A | 5 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Examine the concepts of data and visualization. |
| CO2 | Perform data analysis and categorize data. |
| CO3 | Perform statistical analysis and abstraction of data. |
| CO4 | Evaluate various representation of spatial data. |
| CO5 | Represent data in various charts in Power BI. |
| CO6 | Plot and analyze data in various charts in excel. |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / P | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | - | 10 | - | 6 | - | - | 16 |
| CO2 | - | 26 | - | - | - | - | 26 |
| CO3 | - | - | 16 | - | - | - | 16 |
| CO4 | - | 16 | - | - | - | - | 16 |
| CO5 | 16 | 16 | - | - | - | - | 32 |
| CO6 | 6 | - | 5 | 15 | - | - | 26 |
|  |  |  |  |  |  |  | 132 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 21AE3014 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # ARTIFICIAL INTELLIGENCE AND MACHINE LEARNING IN AEROSPACE APPLICATIONS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (5 X 16 = 80 MARKS) (Answer any five from the following) |  |  |  |  |  |
| 1. | a. | Compare the four categories of AI systems. Also, mention their design approach. | CO1 | U | 6 |
|  | b. | With the help of an real-world example, explain the concept of “Agent and Environment” | CO1 | U | 10 |
| 2. | a. | Explain the properties of Search Algorithms. | CO2 | U | 6 |
|  | b. | For the search problem represented by the tree diagram, find the optimal value using minimax algorithm. Clearly indicate the steps followed. | CO2 | An | 10 |
| 3. |  | Discuss the design considerations of AI based expert system for avionic applications. | CO3 | A | 16 |
| 4. | a. | Differentiate between bias and variance in machine learning models. | CO4 | An | 6 |
|  | b. | How will you apply logistic regression to classify hard-to-position and loss-of-effectiveness control surface fault in a flight simulator test? Assume that the simulation data is acquired from four sensors. | CO4 | A | 10 |
| 5. | a. | With the help of pseudo code, explain the ANN training process. Apply the relevant mathematical concepts | CO4 | A | 6 |
|  | b. | For the given airfoil self-noise data, construct a linear regression model. With the help of suitable metrics, analyze the performance of the model. Input feature: f - Frequency in Hertz [Hz]. Target: SSPL - Scaled sound pressure level, in decibels [dB]. f; 800; 1000; 1250; 1600; 2000; 2500; 3150; 4000. SSPL; 126.201; 125.201; 125.951; 127.591; 127.461; 125.571; 125.201; 123.061. | CO4 / An | 10 |
| 6. | a. | A classifier has been trained to classify the images of two fighter aircrafts. The actual and predicted class are as follows. Actual= [‘Carrier’, ‘Escort’, ‘Carrier’, ‘Escort’, ‘Carrier’, ‘Carrier’, ‘Escort’, ‘Carrier’,‘Escort’,‘Carrier’] Predicted= [‘Carrier’, ‘Carrier’, ‘Carrier’, ‘Escort’, ‘Carrier’, ‘Carrier’, ‘Escort’, ‘Escort’, ‘Escort’, ‘Escort’]. Construct the confusion matrix and calculate all the performance metrics. | CO5 | An | 6 |
|  | b. | Outline the k-NN classification algorithm. Compare the advantages and disadvantages of the algorithm. | CO5 | U | 10 |
| 7. |  | Apply the decision tree concept for unmanned aircraft configuration selection. Clearly indicate the feature selection measures. | CO5 | A | 16 |
| PART – B (1 X 20 = 20 MARKS) [Compulsory Question] |  |  |  |  |  |
| 8. |  | The classification dataset of aircraft types based on remote sensing images has a total of 3,954 valid aircrafts, which is divided into 7 classes : 1. Swept-back wing aircraft, 2. Swept-back aircraft with leading edge, 3. Forward-swept wing aircraft with trailing edge, 4. Delta-wing aircraft, 5. Flat wing aircraft, 6. Propeller aircraft, 7. Helicopter. Design an ANN based classifier. Select the proper architecture. Each image is a grey image with size 256×256. | CO6 | C | 20 |

CO – COURSE OUTCOME	        BL – BLOOM’S LEVEL        M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Comprehend the concept of artificial intelligent systems.. |
| CO2 | Execute suitable strategy for solving real world problems. |
| CO3 | Design expert systems for specific applications. |
| CO4 | Select and evaluate linear algorithms. |
| CO5 | Compare and contrast nonlinear and ensemble algorithms. |
| CO6 | Implement machine learning techniques. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | R | U | A | An | E | C | Total |
| CO1 |  | 16 |  |  |  |  | 16 |
| CO2 |  | 6 |  | 10 |  |  | 16 |
| CO3 |  |  | 16 |  |  |  | 16 |
| CO4 |  |  | 16 | 16 |  |  | 32 |
| CO5 |  | 10 | 16 | 6 |  |  | 32 |
| CO6 |  |  |  |  |  | 20 | 20 |
|  |  |  |  |  |  |  | 132 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 23AE1001 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # PROFESSIONAL ETHICS IN ENGINEERING AND AVIATION INDUSTRY | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Mention the various leadership qualities which are expected from a good leader. |  | CO2 | U | 1 |
| 2. | State why the advanced education and training is required. |  | CO1 | R | 1 |
| 3. | Indicate the various leadership styles. |  | CO2 | U | 1 |
| 4. | List out the major three points where the “code of ethics” are exhibited. |  | CO3 | R | 1 |
| 5. | With few examples explain the “uncertainty” in engineering projects. |  | CO4 | U | 1 |
| 6. | Define “Occupational Crime”. |  | CO3 | R | 1 |
| 7. | Give an example for the “work safety”. |  | CO5 | U | 1 |
| 8. | Show why the statement “A Ship in harbor is safe” is not relevant to safety aspects. |  | CO4 | U | 1 |
| 9. | List out the various details available in a standard “pay roll slip”. |  | CO3 | R | 1 |
| 10. | Indicate the important security measures which are to be ensured in the airport. |  | CO6 | U | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Differentiate between Ethics and Morality. |  | CO1 | U | 3 |
| 12. | Express the “Great man theory”. |  | CO2 | U | 3 |
| 13. | Explain the limitations of “code of ethics”. |  | CO3 | U | 3 |
| 14. | Differentiate between the “Qualitative” and “Quantitative” risk analysis. |  | CO4 | A | 3 |
| 15. | Mention about the “white-collar occupational crime” with a suitable examples. |  | CO5 | A | 3 |
| 16. | Write the code of ethics expected in the airport & air-travel. |  | CO6 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No 17 to 23, Q.No 24 is Compulsory) |  |  |  |  |  |
| 17. |  | With help of a block diagram illustrate the “Gilligan’s theory” in detail. | CO2 | An | 12 |
| 18. |  | Outline the Kohlberg’s theory of moral development with suitable block diagram. | CO2 | A | 12 |
| 19. |  | Explain the “code of ethics” with help of a suitable example. | CO3 | A | 12 |
| 20. |  | Appraise the Institutional and Expert authorities in detail. | CO4 | An | 12 |
| 21. |  | Establish various Risk reducing techniques in detail. | CO5 | A | 12 |
| 22. |  | Discuss the importance of “Respect for Authority” concept with suitable example. | CO4 | A | 12 |
| 23. |  | Illustrate the various conflict resolution techniques in detail. | CO5 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | Elaborate the various fraud awareness training aspects for employees. | CO6 | An | 12 |

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Identify the potential value of the approaches in the organization (strategic, managerial and operational/tactical) relating to Ethical values. |
| CO2 | Appraise the role of leaders in ethically managing society |
| CO3 | Apply the professional ethics with a demonstrated commitment to leadership practice and interpersonal skill |
| CO4 | Assess the effectiveness of own capability and performance in meeting organizational values and goals. |
| CO5 | Evaluate the impact of failure due to non-compliance of ethical consideration in organizations |
| CO6 | Examine the Ethics followed in the aviation industry. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | Remember | Understand | Apply | Analyze | Evaluate | Create | Total |
| CO1 | 1 | 3 | - | - | - | - | 4 |
| CO2 | - | 8 | 12 | 12 | - | - | 32 |
| CO3 | 3 | - | 12 | - | - | - | 15 |
| CO4 | 1 | 1 | 15 | 12 | - | - | 29 |
| CO5 | - | 1 | 27 | - | - | - | 28 |
| CO6 | - | 1 | 3 | 12 | - | - | 16 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 23AE2001 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # INTRODUCTION TO AEROSPACE ENGINEERING | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) (Answer all the questions) |  |  |  |  |  |
| 1. | Write the full form of “NAL”. |  | CO1 | A | 1 |
| 2. | Explain the function of spoilers. |  | CO1 | U | 1 |
| 3. | Describe chord line of an airfoil. |  | CO2 | R | 1 |
| 4. | List the types of drag under parasite drag. |  | CO2 | R | 1 |
| 5. | List the primary structural components of an airplane wing. |  | CO3 | R | 1 |
| 6. | Name the materials which are commonly used in the construction of aircraft wings. |  | CO3 | R | 1 |
| 7. | List the types of jet engines used in airplanes. |  | CO4 | R | 1 |
| 8. | List the disadvantages of a reciprocating engine. |  | CO4 | R | 1 |
| 9. | State a disadvantage of solid rocket propulsion system. |  | CO5 | R | 1 |
| 10. | State Kepler’s first law of planetary motion. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) (Answer all the questions) |  |  |  |  |  |
| 11. | Calculate the atmospheric density at 20 km altitude by considering the sea level properties as follows. Pressure po = 101325 N/m2, Density ρo = 1.225 kg/m3 and Temperature To = 288.15 °K |  | CO1 | A | 3 |
| 12. | An Aero-plane travels in air of pressure of 1 bar at 10 Degree Celsius at a speed of 1800 km/hr. Determine the Mach number. Take γ=1.4 and R=287 J/Kg K. |  | CO2 | A | 3 |
| 13. | Distinguish between monocoque and semi-monocoque structure. |  | CO3 | U | 3 |
| 14. | Explain the principle of power generation in a four stroke reciprocating engine used in airplane. |  | CO4 | U | 3 |
| 15. | List the classifications of chemical rocket propulsion. |  | CO5 | R | 3 |
| 16. | Define Newton’s three laws. |  | CO6 | R | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q.No. 17 to 23, Q.No. 24 is Compulsory) |  |  |  |  |  |
| 17. | a. | Define standard atmosphere and explain the variation of temperature in earth’s atmosphere with a sketch. | CO1 | R | 8 |
|  | b. | Determine the atmospheric temperature at 10 km altitude. | CO1 | A | 4 |
| 18. |  | Explain the functions of primary and secondary flight control surfaces. | CO2 | U | 12 |
| 19. |  | Explain the major aircraft components and their functions with a neat sketch. | CO3 | U | 12 |
| 20. |  | Explain the working principle of turbofan engine with a neat sketch and clearly indicate all the structural components used in turbofan engine. | CO4 | U | 12 |
| 21. | a. | Discuss the working principle of rocket with a neat sketch. | CO5 | R | 9 |
|  | b. | Define specific impulse and write its unit. | CO5 | R. | 3 |
| 22. |  | Explain the origin and characteristics of space debris and discuss the environmental and economic aspects of it. | CO6 | U | 12 |
| 23. |  | Explain the classifications of aircraft according to the wing and engine with neat sketches. | CO3 | U | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. | a. | Explain the theory of lift generation with the help of laws of conservation. | CO2 | A | 6 |
|  | b. | Write short notes on formation of wing tip vortices and induced drag with a neat sketch. | CO2 | A | 6 |

CO – COURSE OUTCOME					BL – BLOOM’S LEVEL

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Relate the fundamentals of aerospace technologies to Aircrafts & Spacecrafts. |
| CO2 | Demonstrate proficiency in basic principles of aerodynamics. |
| CO3 | Identify and describe different types of fuselage and wing construction |
| CO4 | Compare and contrast the different types of propulsion systems. |
| CO5 | Interpret the concepts of rocket and missile dynamics |
| CO6 | Summarize the laws of interplanetary physics |

| Assessment Pattern as per Bloom’s Taxonomy |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | R | U | A | An | E | C | Total |
| CO1 | 8 | 1 | 8 |  |  |  | 17 |
| CO2 | 2 | 12 | 15 |  |  |  | 29 |
| CO3 | 2 | 27 |  |  |  |  | 29 |
| CO4 | 2 | 15 |  |  |  |  | 17 |
| CO5 | 16 |  |  |  |  |  | 16 |
| CO6 | 4 | 12 |  |  |  |  | 16 |
|  |  |  |  |  |  |  | 124 |

END SEMESTER EXAMINATION – APRIL / MAY 2024

| # Course Code | # 23AE2002 | # Duration | # 3hrs |
| --- | --- | --- | --- |
| # Course Name | # ENGINEERING MECHANICS | # Max. Marks | # 100 |

| Q. No. | Questions |  | CO | BL | M |
| --- | --- | --- | --- | --- | --- |
| PART – A (10 X 1 = 10 MARKS) |  |  |  |  |  |
| 1. | Splitting up a force into components along the fixed reference axes is called as __________ of a force. |  | CO1 | R | 1 |
| 2. | If Point A is in equilibrium under the action of the applied forces, the tension in the string TAC is ______________ . |  | CO1 | A | 1 |
| 3. | If the sum of horizontal forces and vertical forces are 95.01 N and 178.2 N respectively, the resultant of the forces is ________________. |  | CO2 | U | 1 |
| 4. | According to Lami’s Theorem, when three forces acting at a point are in equilibrium, then each force is proportional to the ______________ of the angle between the other two forces. |  | CO2 | R | 1 |
| 5. | A particle projected in space at an angle to the horizontal plane is called as ____________ . |  | CO4 | R | 1 |
| 6. | A train covers 60 miles between 2 p.m. and 4 p.m. How fast was it going at 3 p.m.? |  | CO4 | U | 1 |
| 7. | Define elastic collision. |  | CO4 | A | 1 |
| 8. | Define velocity of approach. |  | CO5 | R | 1 |
| 9. | State the work-energy principle. |  | CO5 | R | 1 |
| 10. | The coefficient of friction ranges from ____ to ______. |  | CO6 | R | 1 |
| PART – B (6 X 3 = 18 MARKS) |  |  |  |  |  |
| 11. | Two concurrent forces of 12 N and 18 N are acting at an angle of 60º. Find the resultant force. |  | CO1 | A | 3 |
| 12. | State and prove Varignon’s theorem. |  | CO2 | R | 3 |
| 13. | State and illustrate perpendicular axis theorem with a diagram. |  | CO3 | R | 3 |
| 14. | Define centroid and write the formula for the Centroid of a rectangle of height “h” and breadth “b”. |  | CO3 | U | 3 |
| 15. | A stone dropped from the top of a tower reaches the ground in 8 seconds. Find the height of the tower. |  | CO4 | A | 3 |
| 16. | A body of weight 100 N is placed on a rough horizontal plane and pushed by a force of 45N as shown in Figure to just cause sliding over the horizontal plane. Determine the coefficient of friction. |  | CO5 | A | 3 |
| PART – C (6 X 12 = 72 MARKS) (Answer any five Questions from Q. No. 17 to 23, Q. No. 24 is Compulsory) |  |  |  |  |  |
| 17. |  | Five forces are acting on a particle whose magnitude are 300 N, 600 N, 700 N, 900 N and P. Their respective angles with the horizontal are 0º, 60º, 135º, 210º and 270º. If the vertical component of all forces is -1000 N, find the value of P. Also determine the magnitude and direction of the resultant. | CO1 | A | 12 |
| 18. |  | Determine the support reactions of the beam shown in figure. | CO2 | A | 12 |
| 19. |  | Find the Moment of Inertia of the channel section shown in Figure. | CO3 | A | 12 |
| 20. |  | A train is travelling from A to D along a track. Its initial velocity is zero. The train takes 5 min to cover the distance AB, 2250 m length and 2.5 min to cover the distance BC, 3000 m and on reaching station C, the brakes are applied and the train stops 2250 m beyond, at D. Find a) retardation in CD b) time the train takes to get from A to D c) average speed of the train for the whole distance. | CO4 | A | 12 |
| 21. |  | A body of mass 15 kg is initially at rest on a 10º inclined plane. Then it slides down. Calculate the distance moved by the body, on the inclined plane, when the velocity reaches 6 m/s using D’Alembert’s principle. The coefficient of friction between the body and the plane is 0.1. | CO5 | A | 12 |
| 22. |  | A body of weight 200 N kept on a horizontal plane is pulled by a force of 150 N applied at an angle of 60º to the horizontal. The block moves with an instant velocity of 2 m/s while the coefficient of friction between the body and the surface is 0.25. Find the velocity of the body after 15 sec using impulse-momentum method. | CO5 | A | 12 |
| 23. |  | Two blocks A and B of weight 200 N and 300 N respectively are initially at rest on a 20º inclined plane as shown in Figure. The distance between the blocks is 7 m. The coefficient of friction between the block A and the plane is 0.2 and that between block B and the plane is 0.1. If they are released at the same time, in what time the upper block B reach the lower block A. | CO5 | A | 12 |
| COMPULSORY QUESTION |  |  |  |  |  |
| 24. |  | A uniform ladder of weight 1000 N and of length 4 m rests on a horizontal ground and leans against a smooth vertical wall. The ladder makes an angle of 60º with horizontal. When a man of weight 750 N stands on the ladder at a distance of 3 m from the top of the ladder, the ladder is at the point of sliding. Determine the coefficient of friction between the ladder and the floor. | CO6 | A | 12 |

CO – COURSE OUTCOME	       BL – BLOOM’S LEVEL       M – MARKS ALLOTTED

|  | COURSE OUTCOMES |
| --- | --- |
| CO1 | Determine the resultant force and moment for a given system of forces. |
| CO2 | Apply physical concepts to verify the equilibrium condition of rigid bodies. |
| CO3 | Estimate the second moment of Inertia for simple solids. |
| CO4 | Derive the relationship between displacement, velocity and acceleration of a body in motion. |
| CO5 | Identify the cause of motion imparted on a particle. |
| CO6 | Distinguish between static and kinetic Friction. |

| Assessment Pattern as per Bloom’s Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO / BL | R | U | A | An | E | C | Total |
| CO1 | 1 |  | 16 |  |  |  | 17 |
| CO2 | 4 | 1 | 12 |  |  |  | 17 |
| CO3 | 3 | 3 | 12 |  |  |  | 18 |
| CO4 | 1 | 1 | 16 |  |  |  | 18 |
| CO5 | 2 |  | 39 |  |  |  | 41 |
| CO6 | 1 |  | 12 |  |  |  | 13 |
|  |  |  |  |  |  |  | 124 |